3GPP TSG-RAN WG2 #121	R2-2300918

Athens, Greece, 27th Feb – 3rd May 2023

Source:                    	DENSO CORPORATION

Title:  	Discussion on XR-specific capacity improvements

Document for:        	Discussion and decision

Agenda Item:         	8.5.4		XR-specific capacity improvements

- Introduction

In RAN#98-e, new WID on XR enhancements was approved [1], and the following objectives were specified regarding XR-specific capacity improvements.

Specify the enhancements related to capacity: - Multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration (RAN1, RAN2); - Dynamic indication of unused CG PUSCH occasion(s) based on UCI by the UE (RAN1); - BSR enhancements including at least new BS Table(s); (RAN2); - Delay reporting of buffered data in uplink; (RAN2); - Provision of XR traffic assistance information for DL and UL (e.g. periodicity); (RAN2); - Discard operation of PDU Sets (RAN2);

And the following agreements related to feedback enhancements were reached in RAN2#120 meeting.

RAN2 thinks we need one or more additional BSR table(s) for XR. FFS whether these are static (=specified) or dynamic (e.g. generated, differs according to some RRC parameter), can be discussed in WI phase. RAN2 will introduce data volume information associated with delay information (e.g. remaining time) in a MAC CE. FFS if this is extension of BSR or new format. FFS how to do that (e.g. what exactly is reported) and how to ensure this information is up-to-date e.g. considering UL scheduling delay. RAN2 needs to discuss additional BSR triggering conditions to allow timely availability of buffer status information at gNB. This can be discussed in WI phase.

In this contribution, we provide the discussion on UE feedback enhancements for XR capacity.

- Discussion

Through buffer status reporting (BSR) procedure, the UE provides the gNB with the information regarding UL data volume. This helps the gNB allocate as many resources as the UE needs. However, there is a quantisation error between the buffer size reported by BSR and the actual one at the UE because BSR reports the indexes corresponding to buffer size using the BS tables described in TS38.321 [2]. As a result, the gNB may overestimate the amount of UL data and allocate more resources than necessary. This leads to waste of radio resources and capacity loss. Furthermore, in the current specification, an exponential function is used to determine the BS values in the tables. This means that the quantization error increases sharply as the buffer size increases. The legacy BS tables are not well suited for XR services characterized by high bit rates. Therefore, it is beneficial to introduce additional BS table(s) to reduce quantisation errors.

Observation 1:	It is beneficial to introduce additional BS table(s) to reduce quantisation errors for XR services characterized by high bit rates.

The simplest way is to increase the number of bits used to indicate the buffer size. Additional BS table(s) could be specified based on the increased number of bits (e.g., more than 8 bits). Larger bit fields can provide finer granularity, but the concern is the increased signalling overhead due to BSR. Another way to avoid increased overhead is to narrow down the range of buffer sizes that can be reported while using fields of up to 8 bits. For example, additional BS table(s) could have different maximum and/or minimum buffer size values from the existing ones to match the possible packet size distribution. In this case, it would be even more useful to allow the NW to dynamically configure these values (e.g., max/min buffer sizes) based on different traffic characteristics.

Proposal 1:	The following enhancements to the BS table can be considered to reduce the quantization errors.

- Increase the number of bits in the buffer size field.

- Narrow down the range of buffer sizes that can be reported.

Proposal 2:	Whether the additional BS table(s) should be static or dynamic depends on which option is introduced to reduce the quantization errors.

In addition, it is assumed that XR services have strict latency requirements. Since any data which has exceeded the requested packet delay budget (PDB) may not be useful, the uplink resources need to be allocated before then. If the gNB can know that the UE has UL data that urgently needs to be transmitted due to short remaining time, it could improve the capacity by prioritizing the scheduling of the corresponding data. Therefore, the gNB can benefit from the dynamic delay information provided by UE rather than PDB or PSDB. It is also important that delay information is associated with buffer size information so that the gNB can allocate as many resources as needed for urgent data. From this aspect, delay information can be reported by extending the legacy BSR formats.

The legacy BSR formats can have buffer size fields which indicates only the total amount of data available per each LCG. One possible enhancement is to introduce new buffer size fields that can report the amount of UL data with short remaining time before exceeding the delay budget. This allows the gNB to determine how many UL resources urgently need to be allocated to the UE so that data with short remaining time is not dropped.

Proposal 3:	Delay information can be reported by extending the legacy BSR formats.

Proposal 4:	RAN2 could introduce new buffer size fields that can report the amount of UL data with short remaining time before exceeding the delay budget.

- Summary and proposal

In this contribution, we provided the following observation and proposals.

Observation 1:	It is beneficial to introduce additional BS table(s) to reduce quantisation errors for XR services characterized by high bit rates.

Proposal 1:	The following enhancements to the BS table can be considered to reduce the quantization errors.

- Increase the number of bits in the buffer size field.

- Narrow down the range of buffer sizes that can be reported.

Proposal 2:	Whether the additional BS table(s) should be static or dynamic depends on which option is introduced to reduce the quantization errors.

Proposal 3:	Delay information can be reported by extending the legacy BSR formats.

Proposal 4:	RAN2 could introduce new buffer size fields that can report the amount of UL data with short remaining time before exceeding the delay budget.

- References

[1]	RP-223502, New WID on XR Enhancements for NR, Nokia, Qualcomm

[2]	3GPP TS 38.321 V17.3.0 (2022-12)