3GPP TSG-RAN WG2 Meeting #121 	R2-2300919

Athens, Greece, 27th Feb. – 3rd Mar., 2023

Agenda item:	8.19.3

Source:	Huawei, HiSilicon

Title:	Early identification and access restriction for eRedCap UEs

Document for:	Discussion and Decision

- Introduction

In RAN#98-e, the WID on Enhanced support of reduced capability NR devices [1] was agreed. A new agreement is included, i.e. Support additional separate early indication(s). In RAN1#111, several agreements upon transmission restriction are achieved. In this paper, we will discuss the related issues on early indication and access control.

RAN#98: ### 4.1 Objective of Core part WI … Complexity/cost reduction - Further reduced UE complexity in FR1 [RAN1, RAN2, RAN4] - UE BB bandwidth reduction - 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH, with 20 MHz RF bandwidth for UL and DL - The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth. - Support additional separate early indication(s) [RAN1, RAN2] - UE peak data rate reduction - Relaxation of the constraint (vLayers·Qm·f ≥ 4) for peak data rate reduction - The relaxed constraint is, e.g., 1 (instead of 4). - The parameters (vLayers, Qm, f) can be as in Rel-17 RedCap. - Both 15 kHz SCS and 30 kHz SCS are supported. - Aim to define at most one Rel-18 RedCap UE type for further UE complexity reduction. - The existing UE capability framework is used, and changes to capability signalling are specified only if necessary. By default, all UE capabilities applicable to a Rel-17 RedCap UE are applicable unless otherwise specified.

RAN1#111: - For UE BB complexity reduction, a UE is not expected to receive an UL grant in a RAR or in a DCI scrambled with TC-RNTI with a Msg3 PUSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable. - For UE BB bandwidth reduction, for RAR (PDSCH) to Rel-18 RedCap UEs, the scheduling of RAR PDSCH is allowed to be larger than the maximum number of unicast PRBs that the UE can process per slot. - When the scheduling of RAR PDSCH is within the maximum number of unicast PRBs that the UE can process per slot, the legacy time between RAR reception and Msg3 transmission (not smaller than NT,1 + NT,2 + 0.5 ms) is applied. - When the scheduling of RAR PDSCH is larger than the maximum number of unicast PRBs that the UE can process per slot, - The UE receives the RAR and correspondingly transmits Msg3 if the TDRA for Msg3 in UL grant in RAR indicates that the time between RAR reception and Msg3 transmission is NOT smaller than NT,1 + NT,2 + 0.5 + X ms. - FFS: value(s) of X - Otherwise, the UE behavior is up to the UE implementation. - For UE BB complexity reduction, a UE is able to receive a DL assignment in a DCI with a unicast PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot. - The number of PRB scheduled in DCI is not larger than the maximum number of PRB agreed in previous agreement from 110b-e

- Discussion

  - Early identification of eRedCap UE

As agreed in WID, early identification is supported for R18 eRedCap UE, then we need to discuss which message is used to indicate NW, i.e. Msg1, Msg3 or both Msg1 and Msg3. According to RAN1 agreements, for UL transmission, Msg3 PUSCH resource indicated by RAR is no more than ~5 MHz per slot or per hop, and the minimum time between RAR reception and Msg3 transmission is longer than legacy mechanism in some case. So it is beneficial to introduce Msg1 identification for NW. Without Msg1 identification, NW can also ensure the successful transmission of Msg3 by implementation and UE send early indication in Msg3 so that the subsequent unicast transmissions can work well. Besides, this NW implementation also has impact on legacy UEs because NW need to schedule all UEs to meet the restriction of eRedCap UE. So R17 RedCap early identification mechanism can be reused. Both Msg1 based and Msg3 based early identification should be supported, and whether to configure Msg1 based identification is up to NW implementation. The eRedCap UE should support them mandatorily, as in R17 RedCap.

Proposal 1a: Both Msg1 based and Msg3 based early identification should be supported by eRedCap UE.

Proposal 1b: Similar to R17, the Msg1 based early identification mechanism for eRedCap UE is optionally configurable.

Like R17 RedCap UE, two new LCIDs can be introduced for Msg 3 based identification for eRedCap UE. When Msg1 identification resource is not configured, eRedCap specific LCID should be mandatory used. Considering that using new LCIDs will not introduce extra signaling overhead, eRedCap specific LCID can be always used in Msg3 similar to R17 RedCap UE.

Proposal 1c: Similar to R17, two new LCID are used for the Msg3 based early identification for eRedCap UE.

It is a reasonable method to support a fallback operation, which allows a R18 eRedCap UE to use Msg1 resource of R17 RedCap UE, if Msg1 based early identification is not configured for R18 eRedCap UE and Msg1 based early identification is configured for R17 RedCap UE. It is agreed that R18 eRedCap UE can share the R17 RedCap specific initial UL/DL BWP (assuming that there is no R18 eRedCap specific initial UL/DL BWP). So it is natural to reuse the Msg1 resource of R17 RedCap UE and receive the RAR on R17 RedCap specific initial DL BWP. we propose R18 eRedCap UE support the fallback operation when there is R17 RedCap UE specific Msg1 resource but no specific Msg1 resource for eRedCap UE.

Observation 1: An eRedCap UE using the R17 RedCap Msg1 identification can help the NW to understand that this UE are using the RedCap specific initial DL BWP, regardless of eRedCap or RedCap UE.

Proposal 2: eRedCap UE should use R17 Msg1 early identification for RedCap if configured, in case the R18 Msg1 early identification for eRedCap is not configured.

  - Access control of eRedCap UE

R17 RedCap UE is not allowed to access legacy NR cell because of the reduced capability, such as the maximum bandwidth, PDCP/RLC SN length and the number of DRBs. Similarly, considering the BB bandwidth of R18 eRedCap UE has further reduced, the eRedCap UE should not access to the legacy NR cell and R17 RedCap cell, either.

Proposal 3a: The eRedCap UE is not allowed to access to the R15/R16/R17 legacy cell (including the legacy cell either supporting or not supporting RedCap) or the R18 cell not supporting eRedCap.

If the eRedCap UE is supported in a cell, NW should make special scheduling strategy, such as 5 MHz BB bandwidth for PDSCH and PUSCH (for unicast). So whether to support eRedCap is a cell level capability and a new support indicator for eRedCap UE should be introduced in SIB1. Similar to R17 RedCap UE, a new IFRI for eRedCap can be supported. If the new IFRI is not present, then the eRedCap UE considers itself as barred. In addition, the new IFRI indication can also indicate whether intra frequency cell selection/reselection for eRedCap UE is allowed.

Proposal 3b: SIB1 should be able to indicate whether the cell supports eRedCap UE.

Proposal 3c: Introduce eRedCap UE specific IFRI in SIB1.

As discussed above, the transmission resource of eRedCap unicast transmission is restrict within 5WHz. In case of handover, the NW should ensure to handover the eRedCap UEs to the target cell which supports eRedCap UE. It seems the similar R17 RedCap Xn capability exchanging method can be reused with a new indication. So we think RAN2 should send LS to RAN3 for supporting such case.

Proposal 4a: Network should ensure to handover eRedCap UE to a gNB, which supports/allows eRedcap UE.

Proposal 4b: RAN2 send LS to RAN3 to support the Xn signalling (similar to the R17 RedCap Broadcast Information IE in 38.423).

Similar to R17 RedCap UE, an eRedCap UE is equipped with 1Rx branch or 2Rx branch. Then the R18 eRedCap UE may be also barred according to different RX braches. Considering that NW may have different access control strategies for R17 RedCap UE and R18 eRedCap UE, it is beneficial to introduce separate barring indication for R18 eRedCap UE with 1Rx branch and 2 Rx branch.

Proposal 5a: Introduce eRedCap UE specific cell barring indications in SIB1, i.e. cellBarredRedCapEnhanced1Rx-r18, cellBarredRedCapEnhanced2Rx-r18.

To save signaling overhead in SIB1, one optimization can be considered. When the NW intends to use yhr same access control of 1Rx/2Rx for both R17 RedCap UE and R18 eRedCap UE, the separate cellBarredRedCapEnhanced1Rx-r18, cellBarredRedCapEnhanced2Rx-r18 can be absent. And then R18 eRedCap UE can follow the 1Rx barring and 2Rx barring indication for R17 RedCap UE.

Proposal 5b: As to the cell barring, RAN2 to discuss if the eRedCap UE can follow the R17 RedCap cell barring indication, in case the eRedCap cell barring indication is absent.

To accelerate R18 eRedCap UE cell selection/reselection procedure, a new indication can be added to indicate the frequency supporting R18 eRedCap UE (like R17 RedCap). Then a R18 eRedCap UE will not measure the frequency that not support eRedCap UE to avoid camping on a cell not supporting eRedCap UE.

Proposal 6: Similar to R17, introduce eRedcapAccessAllowed-r18 in interFreqCarrierFreqList in SIB4, about the frequency of neighbour cell supporting eRedCap.

The study of R18 eRedCap is based on the capability framework of R17 RedCap but with further reduced BB bandwidth and peak data rate. R18 eRedCap UE has lower transmission efficiency. It means not a practical implementation for NW to only support R18 eRedCap but not support R17 RedCap. So RAN2 can confirm that it is invalid case where NW only support eRedCap UE but not support RedCap UE.

Observation 2: It as an invalid case that the NW only supports eRedCap UE but not supports RedCap UE.

As specified in WID, all UE capabilities of a R17 RedCap UE are assumed applicable to R18 eRedCap UE unless otherwise specified. So the R18 eRedCap UE also has the similar capabilities, e.g. HD-FDD, 1Rx and 2Rx.

Based on the above proposal, a cell which supports R18 eRedCap UE also supports R17 RedCap UE. So some NW indications applicable to a R17 RedCap UE are applicable to a R18 eRedCap UE unless NW want to have flexible/separate control. The halfDuplexRedCapAllowed indicates whether the cell supports half-duplex FDD only RedCap UEs, which is a kind of a fixed capability of NW. So the indication can also be reused by R18 eRedCap UE.

Proposal 7: There is no need to introduce a new “HD-FDD only” broadcasting indication specific for eRedCap UEs (i.e. just to reuse the legacy halfDuplexRedCapAllowed-r17).

- Conclusion

The corresponding proposals are listed as below:

Observation 1: An eRedCap UE using the R17 RedCap Msg1 identification can help the NW to understand that this UE are using the RedCap specific initial DL BWP, regardless of eRedCap or RedCap UE.

Observation 2: It as an invalid case that the NW only supports eRedCap UE but not supports RedCap UE.

Early identification

Proposal 1a: Both Msg1 based and Msg3 based early identification should be supported by eRedCap UE.

Proposal 1b: Similar to R17, the Msg1 based early identification mechanism for eRedCap UE is optionally configurable.

Proposal 1c: Similar to R17, two new LCID are used for the Msg3 based early identification for eRedCap UE.

Proposal 2: eRedCap UE should use R17 Msg1 early identification for RedCap if configured, in case the R18 Msg1 early identification for eRedCap is not configured.

Access control

Proposal 3a: The eRedCap UE is not allowed to access to the R15/R16/R17 legacy cell (including the legacy cell either supporting or not supporting RedCap) or the R18 cell not supporting eRedCap.

Proposal 3b: SIB1 should be able to indicate whether the cell supports eRedCap UE.

Proposal 3c: Introduce eRedCap UE specific IFRI in SIB1.

Proposal 4a: Network should ensure to handover eRedCap UE to a gNB, which supports/allows eRedcap UE.

Proposal 4b: RAN2 send LS to RAN3 to support the Xn signalling (similar to the R17 RedCap Broadcast Information IE in 38.423).

Proposal 5a: Introduce eRedCap UE specific cell barring indications in SIB1, i.e. cellBarredRedCapEnhanced1Rx-r18, cellBarredRedCapEnhanced2Rx-r18.

Proposal 5b: As to the cell barring, RAN2 to discuss if the eRedCap UE can follow the R17 RedCap cell barring indication, in case the eRedCap cell barring indication is absent.

Proposal 6: Similar to R17, introduce eRedcapAccessAllowed-r18 in interFreqCarrierFreqList in SIB4, about the frequency of neighbour cell supporting eRedCap.

Proposal 7: There is no need to introduce a new “HD-FDD only” broadcasting indication specific for eRedCap UEs (i.e. just to reuse the legacy halfDuplexRedCapAllowed-r17).

- Reference

[1] RP-223544, Revised WID on Enhanced support of reduced capability NR devices, Ericsson.

[2] RAN1#111 agreements on Further NR RedCap UE complexity reduction