3GPP TSG-RAN WG2 Meeting #121	R2-2300921

27th February – 3rd March 2023

Athens, GR

Source:                    	DENSO CORPORATION

Title:  	Discussion on NR-DC with selective activation of the cell groups.

Document for:        	Discussion and decision

Agenda Item:         	8.4.3	NR-DC with selective activation cell of groups

- Introduction

At RAN2 #120[1], the following was agreed.

Delta configuration

-A UE stores the reference configuration as a separate configuration.

-The reference configuration is managed separately

Furthermore, the following was also discussed at RAN2 #120.

Security Issue

RAN2 expects e.g. the following scenarios to be supported:

- the UE changes to PSCell #1a controlled by SN #1, then to PSCell #2a controlled by SN #2, then to PSCell #1a controlled by SN #1;

- the UE changes to PSCell #1a controlled by SN #1, then to PSCell #2a controlled by SN #2, then to PSCell #1b controlled by SN #1.

With existing procedures:

- the UE derives the S-KgNB from the sk-counter in the executed conditional reconfiguration, if included.

- in the above scenarios, the UE will use the same S-KgNB every time it is connected to PSCell #1a, i.e. before and after moving to SN #2.

- if the same value of sk-counter is included in the conditional reconfiguration for PSCell #1a and in the conditional reconfiguration for PSCell #1b, the UE will use the same S-KgNB before and after moving to SN #2.

Therefore, we discussed about security issues on NR-DC with selective activation of the cell groups.

In addition, it was also FFS if UE keeps all configurations or if those are indicated by the network in the selective activation of cell groups since RAN2 #119bis-e. This paper also focuses on these FFS points.

- Discussion

2.1 Security Issue

In RAN2 #119bis meeting [2], RAN2 agreed to support selective activation of cell groups without RRC reconfiguration with respect to security, as follows.

RAN2 aim to support selective activation of cell groups without RRC reconfiguration with respect to security (FFS, need to consult with SA3 at some point in time).

In Rel-17, UE update the Security Key for SN by using sk-Counter included in RRCReconfiguration received from NW. However, selective activation of PSCell has security issue that UE uses same sk-Counter under the following scenario since sk-Counters of cells which belonging same SN are set to same value and do not update by using RRC Reconfiguration.

- the UE changes to PSCell #1a controlled by SN #1, then to PSCell #2a controlled by SN #2, then to PSCell #1a controlled by SN #1;

- the UE changes to PSCell #1a controlled by SN #1, then to PSCell #2a controlled by SN #2, then to PSCell #1b controlled by SN #1.

In last meeting, RAN2 agreed to send the LS to SA3 about that weather SA3 considers the existing handling of sk-counter/ S-KgNB in the above scenarios acceptable or not.

We think RAN2 should wait for the reply of SA3, however, we also think that it is worth to discuss about solution for security key update. In this case, since the UE has no way to receive the new sk-Counter from NW in selective activation of PSCell, the UE need to update the sk-Counter on its own.

One of solution for security issue in selective activation of PSCell is that UE updates own security key after every PSCell changes. In this way, UE can update the security key without indication of RRC Reconfiguration by incrementing the own sk-Counter with common steps upon the PSCell change.

Observation 1：In selective activation of PSCell, the UE need to update the sk-Counter on its own.

Proposal 1：UE updates the security key by increasing the value of sk-Counter with common steps after each PSCell changes.

In this scenario, for ensuring the security communication between the UE and the SN, the SN needs to set the same security key with UE’s one. Therefore, security key update mechanism is needed for the SN after every PSCell Changes same as the UE.

However, in our understanding, the security key for the SN is indicated by SN Addition Request in Rel-17 inter-SN CPC. Then, if the UE updates own security key upon PSCell changes, the SN cannot detect the security key change and may set the different key from UE’s one. This may cause communication error between the UE and the SN. Therefore, we think RAN2 need to discuss the sharing mechanism of security key between the UE and the SN in selective activation of PSCell.

Observation 2：SN needs to set the same security key with UE when the UE updates the own security key.

Proposal 2：RAN2 to discuss the sharing mechanism of security key between the UE and SN in selective activation of PSCell.

2.2 Management of candidate PSCell configurations

In RAN2 #119bis meeting, RAN2 agreed that UE doesn’t release conditional reconfiguration after finishing the PSCell addition or change, and continues evaluating the execution conditions, as the baseline procedure to support subsequent secondary cell group change, as follows.

-	Baseline procedure to support subsequent secondary cell group change (FFS if UE keeps all configurations or if those are indicated by the network, FFS support of nested configs):

Observation 3：RAN2 support subsequent cell group change by maintaining the conditional reconfigurations after finishing the PSCell addition or change.

Then, it was FFS if UE keeps all configurations or if those are indicated by the network in the selective activation of cell groups. However, we think that both patterns may be necessary for various scenario of UE mobility pattern. For example, if the UE cyclically moves in the small area (e.g. event venue) as shown in Figure.1-(a), the UE need to keep the all configuration since the UE may change the PSCell to a cell that has already connected it in the past. And if the UE moves linearly in large area as shown in Figure.1-(b), the configuration for cell which already connected in past as in cell belonging SN1 in Fig.1-(b) is not necessary for UE since the UE rarely connect to such cells again. Thus, the configurations which the UE should keep are change depending on the scenario of UE mobility. Therefore, we think that RAN2 should more discussion about that whether UE keeps all configurations or some configurations indicated by the network by considering the various use cases.

Observation 4：The configurations which the UE should keep are change depending on the scenario of UE mobility.

Proposal 3：RAN2 should more discussion about that whether UE keeps all configurations or some configurations indicated by the network by considering the various use cases.

(a) the UE moves cyclically in the small area                          (b) the UE moves linearly in large area

Figure.1 The scenario of UE mobility

In RAN2 #119bis meeting, it was agreed that the UE keeps the conditional reconfiguration of the candidate PSCells, however, it was not discussed how to release the configuration. According to the current procedure, the NW can indicate removal or modification of the configuration to the UE. Thus, the additional RRC signaling would be necessary for release of the configuration and it may lead signaling overhead. Therefore, it is worth to discuss releasing method of the configuration which indicated to keep by the NW. The one way is the UE trigger-based method in which if some conditions (e.g. timer, measurement result) are fulfilled at the UE side, the UE assumes that the configuration is invalid. We propose RAN2 to discuss the releasing mechanism of the conditional reconfiguration of the candidate PSCell which indicated to keep by the NW.

Proposal 4: RAN2 to discuss the releasing mechanism of the conditional reconfiguration of the candidate PSCell which indicated to keep by the NW.

Furthermore, if the UE fail the random access to target-SN, the UE sends SCG Failure Information to NW for indication of random access problems. However, in selective activation of PSCell scenario, it may occur some problems since the UE maintains conditional reconfiguration for subsequent PSCell change. Therefore, we proposed RAN2 to discuss the procedure for Random Access Failure.

Proposal 5: RAN2 to discuss the procedure for Random Access Failure.

- Summary and proposal

This paper discussed the procedure of the subsequent conditional PSCell change after a cell group change. In summary, the followings were proposed:

Observation 1：In selective activation of PSCell, the UE need to update the sk-Counter on its own.

Observation 2：SN needs to set the same security key with UE when the UE updates the own security key.

Observation 3：RAN2 support subsequent cell group change by maintaining the conditional reconfigurations after finishing the PSCell addition or change.

Observation 4：The configurations which the UE should keep are change depending on the scenario of UE mobility.

Proposal 1：UE updates the security key by increasing the value of sk-Counter with common steps after each PSCell changes.

Proposal 2：RAN2 to discuss the sharing mechanism of security key between the UE and NW.

Proposal 3：RAN2 should more discussion about that whether UE keeps all configurations or some configurations indicated by the network by considering the various use cases.

Proposal 4: RAN2 to discuss the releasing mechanism of the conditional reconfiguration of the candidate PSCell which indicated to keep by the NW.

Proposal 5: RAN2 to discuss the procedure for Random Access Failure.

- References

[1] R2-2300002, “RAN2#120 Meeting Report,” ETSI MCC.

[2] R2-2209301, “RAN2#119-e Meeting Report,” ETSI MCC.

[3] RP-213565	“New WID on Further NR mobility enhancements,” MediaTek.