3GPP TSG-RAN WG2 Meeting #121	R2-2300950

Athens, 27th Feb – 3rd Mar 2023

Agenda item:	   8.16.2

Source:		Lenovo

Title:		General issues on AI for air interface

Document for:		Discussion and Decision

# Introduction

In the past RAN1 meetings, the following agreements have been made with respect to the general AI/ML for air interface framework. In this paper, we discuss the possible RAN2 impacts.

RAN1#109 Agreements: As indicated in SID, although specific AI/ML algorithms and models may be studied for evaluation purposes, AI/ML algorithms and models are implementation specific and are not expected to be specified. - RAN1 discussion should focus on network-UE interaction. - AI/ML functionality mapping within the network (such as gNB, LMF, or OAM) is up to RAN2/3 discussion. Take the following network-UE collaboration levels as one aspect for defining collaboration levels - Level x: No collaboration - Level y: Signaling-based collaboration without model transfer - Level z: Signaling-based collaboration with model transfer Note: Other aspect(s), for defining collaboration levels is not precluded and will be discussed in later meetings, e.g., with/without model updating, to support training/inference, for defining collaboration levels will be discussed in later meetings FFS: Clarification is needed for Level x-y boundary RAN1#110 Agreements: Study the following aspects, including the definition of components (if needed) and necessity, in Life Cycle Management - Data collection - Note: This also includes associated assistance information, if applicable. - Model training - [Model registration] - Model deployment - Note: Terminology is to be defined. - [Model configuration] - Model inference operation - Model selection, activation, deactivation, switching, and fallback operation - Model monitoring - Model update - Note: Terminology is to be defined. This includes model finetuning, retraining, and re-development via online/offline training. - Model transfer - UE capability Note: Some aspects in the list may not have specification impact. Note: Aspects with square brackets are tentative. Note: More aspects may be added as study progresses. RAN1#110bis Agreements: Working Assumption - Define Level y-z boundary based on whether model delivery is transparent to 3gpp signalling over the air interface or not. - Note: other procedures than model transfer/delivery are decoupled with collaboration level y-z - Clarifying note: Level y includes cases without model delivery. Agreement Clarify Level x/y boundary as: - Level x is implementation-based AI/ML operation without any dedicated AI/ML-specific enhancement (e.g., LCM related signalling, RS) collaboration between network and UE. (Note: The AI/ML operation may rely on future specification not related to AI/ML collaboration. The AI/ML approaches can be used as baseline for performance evaluation for future releases.) Agreement Study LCM procedure on the basis that an AI/ML model has a model ID with associated information and/or model functionality at least for some AI/ML operations when network needs to be aware of UE AI/ML models FFS: Detailed discussion of model ID with associated information and/or model functionality. FFS: usage of model ID with associated information and/or model functionality based LCM procedure FFS: whether support of model ID FFS: the detailed applicable AI/ML operations Agreement For model selection, activation, deactivation, switching, and fallback at least for UE sided models and two-sided models, study the following mechanisms: - Decision by the network - Network-initiated - UE-initiated, requested to the network - Decision by the UE - Event-triggered as configured by the network, UE’s decision is reported to network - UE-autonomous, UE’s decision is reported to the network - UE-autonomous, UE’s decision is not reported to the network FFS: for network sided models FFS: other mechanisms Conclusion Data collection may be performed for different purposes in LCM, e.g., model training, model inference, model monitoring, model selection, model update, etc. each may be done with different requirements and potential specification impact. FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined) Agreement Study potential specification impact needed to enable the development of a set of specific models, e.g., scenario-/configuration-specific and site-specific models, as compared to unified models. Note: User data privacy needs to be preserved. The provision of assistance information may need to consider feasibility of disclosing proprietary information to the other side. Agreement Study the specification impact to support multiple AI models for the same functionality, at least including the following aspects: - Procedure and assistance signaling for the AI model switching and/or selection FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined) Agreement Study AI/ML model monitoring for at least the following purposes: model activation, deactivation, selection, switching, fallback, and update (including re-training). FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined) RAN1#111 Agreements: Agreement For UE-part/UE-side models, study the following mechanisms for LCM procedures: - For functionality-based LCM procedure: indication of activation/deactivation/switching/fallback based on individual AI/ML functionality - Note: UE may have one AI/ML model for the functionality, or UE may have multiple AI/ML models for the functionality. - FFS: Whether or how to indicate Funtionality - For model-ID-based LCM procedure, indication of model selection/activation/deactivation/switching/fallback based on individual model IDs

# Discussion

## 2.1	AI/ML related discussion in RAN3 and SA5

First of all, it’s worth noticing that there are AI/ML related discussions on-going in other working groups in parallel.

In particular, RAN3 has completed a study item in Rel17 regarding using AI/ML to optimize network energy saving, load balancing, and mobility optimization. RAN3 is now continuing the discussion and will specify the related network procedures in the Rel18 work item (RP-220635). RAN3 also discussed whether the AI training/inference functionality is located and currently, and the following two ways of deployment is supported:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

- AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

RP-220635 Normative work is based on the conclusions captured in TR37.817. The detailed objectives of the WI are listed as follows: - Specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based Network Energy Saving, Load Balancing and Mobility Optimization. (RAN3)

AI/ML training/inference capabilities will be supported by gNB in Rel18 for network energy saving, load balancing and mobility optimization.

It is upon RAN3 discussion whether the AI/ML training/inference capabilities is located in CU or DU in a CU-DU split gNB architecture.

Besides, SA5 also started a study item in Rel18 to address the AI/ML management capabilities and management services issues, which includes the collaboration between OAM and NG-RAN (SP-211443).

SP-211443 To study the AI/ML management capabilities and management services to support/coordinate AI/ML in 5GS (3GPP management system, 5GC and NG-RAN) without disclosing or restricting the proprietary algorithm of AI/ML model, including - the use cases, potential requirements, and possible solutions for management of AI/ML capabilities for the AI/ML-enabled functions (e.g., MDA, RAN intelligence, NWDAF, etc.) in 5GS, including but not limited to the following capabilities: - Validation of AI/ML model and AI/ML-enabled function - Testing of AI/ML model and AI/ML-enabled function (before deployment) - Deployment of AI/ML model (new or updated model) and AI/ML-enabled function - Configuration of AI/ML-enabled function - Performance evaluation of AI/ML-enabled function Note: as a priority, the study will first focus on the objective 1), specifically addressing management capabilities for AI/ML model validation, testing and deployment to support the AI/ML in NG-RAN when AI/ML model training is in OAM and inference is in NG-RAN.

AI/ML management capabilities will probably be supported in Rel18/19 for OAM to deploy a trained AI/ML model to NG-RAN.

## 2.2	Network-UE collaboration for AI for air interface

RAN1 has discussed the possible collaboration between network and UE to facilitate the AI for air interface. Three levels of collaboration have been considered so far:

- Level x: No collaboration

- Level y: Signaling-based collaboration without model transfer

- Level z: Signaling-based collaboration with model transfer

For level x collaboration, in our understanding, that means both AI training and inference are operating either in network or in UE in an isolated way. One side (UE or network) does not even have to know an AI/ML approach is used by the other side (network or UE). The AI/ML approach can be regarded as a pure implementation option and has no impact on legacy Uu procedure/signalling. In that sense, level x collaboration has no RAN2 impact. Such understanding is also confirmed by the following RAN1 110bis agreement

RAN1#110bis Agreements: Agreement Clarify Level x/y boundary as: - Level x is implementation-based AI/ML operation without any dedicated AI/ML-specific enhancement (e.g., LCM related signalling, RS) collaboration between network and UE. (Note: The AI/ML operation may rely on future specification not related to AI/ML collaboration. The AI/ML approaches can be used as baseline for performance evaluation for future releases.)

Figure 1 Illustration of Level x, no AI/ML related signalling-based collaboration

For Network-UE level x collaboration (i.e., no collaboration), there is no RAN2 impact.

Comparatively, level y and z collaboration would require enhancements to the legacy Uu procedures/signalling. The difference between level y and z is whether model delivery is transparent to 3gpp signalling over the air interface or not. In other word, if the impact on 3gpp signalling is enhanced signalling for AI/ML inference/management without model transfer, it is level y. If the AI/ML is transferred or known by the 3gpp signalling, it is level z.

Working Assumption - Define Level y-z boundary based on whether model delivery is transparent to 3gpp signalling over the air interface or not.

On the other hand, for better assessment and design, RAN2 needs to understand what exact information shall be transferred between gNB and UE and what size is it, which are dependent on RAN1 progress. For example, as illustrated in our RAN1 contribution (R1-2209119), the transferred information can be for model management or inference operation.

Figure 2 Illustration of AI/ML related signalling-based collaboration

RAN2 studies the possible enhancements to support Network-UE level y collaboration (i.e., signalling-based without model transfer) and level z collaboration (i.e., signalling-based with model transfer) taking into account RAN1 progress.

## 2.3	LCM related issues

### 2.3.1	Data collection

Issues related to data collection have been discussed in the email discussion [Post120][054][AIML18] Data Collection. It seems common understanding that the requirements of data collection depend on the purpose of data collection. The four categories of data collection requirements seem agreeable to companies are:

a) the content of the data,

b) the data size,

c) latency and periodicity,

d) signalling and configuration aspects.

In our understanding, a) the content of data will eventually depend on RAN1. From RAN2 perspective, RAN2 can start by assuming all legacy available measurements in RRC Connected state can be collected for AI based CSI feedback enhancement or BM. We don’t think measurements in RRC Inactive/Idle states will be much useful for CSI/BM use cases at this stage.

For AI based positioning, it is a bit different since positioning in RRC Inactive state is supported in Rel17 and positioning in RRC Idle state is currently under Rel18 discussion. Thus, RAN2 can at least assume all legacy available measurements in RRC Connected and Inactive states can be collected for AI based positioning. As for data collection for positioning in RRC Idle state, it depends on the conclusion from Rel18 positioning.

For the content of data to be collected, RAN2 assumes

  - All legacy measurements in RRC Connected state can be collected for AI based CSI feedback enhancement and BM use cases.

  - All legacy measurements in RRC Connected and Inactive state can be collected for AI based positioning.

With respect to b) data size, the exact data size depends on the actual AI algorithm/implementation and RAN1 also has a say on this.  Qualitatively, in our understanding the data size needed for AI training (especially offline training) is usually larger than the data size needed for AI update/inference/monitoring/selection/etc. In particular, AI update can be seen as a smaller scale AI training. Besides, for per time of AI inference there could be a plenty of inputs needed, while for per time AI monitoring/selection/etc. it could be as simple as comparing a predicted result with a ground truth value. In this sense, the data size needed for AI inference could be considered larger than AI monitoring/selection/etc.

In addition, in order to better support data collection for AI model training (especially offline training), RAN2 is suggested to study approaches that can transmit measurements related to AI model training in one package, instead of per measurement report manner as in legacy.

| Data size (per time of AI operation) | High | Medium High | Medium Low | Low |
| --- | --- | --- | --- | --- |
| AI operation | AI model training | AI model update | AI model inference | AI model monitoring/selection/ activation/deactivation/switch/fallback |

RAN2 considers the data size requirements for (per time of) AI operations as following: AI model training > update > inference > monitoring/selection /activation/deactivation/switch/fallback

For AI training data collection, RAN2 studies approaches to support sending training data (e.g., collection of measurements) in one package from UE to NW.

In terms of c) latency and periodicity, qualitatively speaking, AI inference usually happens most frequently compared to other AI operation. Because AI update/monitoring/selection/etc. is performed based on the AI model performance in a period of time and AI training (especially offline training) is performed based on a series of AI monitoring (potentially from multiple UEs). Thus, similarly, we can qualitatively categorize the latency and periodicity requirements as in the table below.

| Latency and periodicity | High | Medium | Low |
| --- | --- | --- | --- |
| AI operation | AI model training | AI model update/monitoring/ selection/activation/deactivation/switch/fallback | AI model inference |

RAN2 considers the latency/periodicity requirements for AI operations as following: AI model training > update/monitoring/selection /activation/deactivation/switch/fallback > inference.

### 2.3.2	AI Model Transfer

Issues related to AI Model Transfer have been discussed in [Post120][053][AIML18] model transfer delivery. Roughly, there are 7 options on the table now when it comes to AI model transfer, namely:

-	Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling.

-	Solution 2a: CN (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling.

-	Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling.

-	Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data.

-	Solution 2b: CN (except LMF) can transfer/deliver AI/ML model(s) to UE via UP data.

-	Solution 3b: LMF can transfer/deliver AI/ML model(s) to UE via UP data.

-	Solution 4: Server can transfer/delivery AI/ML model(s) to UE (transparent to 3GPP).

In the meanwhile, it can be noticed from the RAN1 agreement that no matter it is NW or UE decision, NW can be aware of the model selection, activation, deactivation, switching, and fallback decision. Either decided by NW or informed by UE. In addition, for AI based CSI feedback enhancement and beam management, we believe it is beneficial for gNB get involved to monitor the AI model performance and trigger necessary model selection etc. Similarly, LMF should be involved for AI based positioning.

This is straightforward if the AI/ML model is transferred from gNB to UE (Solution 1a, 1b) or transferred from LMF to UE (Solution 3a, 3b). However, if it is a CN node (Solution 2a, 2b) or application server (Solution 4) that manages the AI/ML model, it is unclear how can gNB/LMF to be aware of the used AI/ML model since the AI model transfer is transparent to them.

RAN1 Agreement For model selection, activation, deactivation, switching, and fallback at least for UE sided models and two-sided models, study the following mechanisms: - Decision by the network - Network-initiated - UE-initiated, requested to the network - Decision by the UE - Event-triggered as configured by the network, UE’s decision is reported to network - UE-autonomous, UE’s decision is reported to the network - UE-autonomous, UE’s decision is not reported to the network

NW can be aware of the model selection, activation, deactivation, switching, and fallback based on its own decision or based on UE report according to RAN1 agreement.

If the AI/ML model is transferred to UE from CN node (Solution 2a, 2b) or application server (Solution 4), RAN2 studies how can gNB/LMF be aware of the AI/ML model usage for the sake of model monitoring and possible selection/activation/deactivation/switch/fallback.

Besides, UE-sided model and two-sided model, gNB may also use a network-sided model for CSI feedback enhancement and beam management. In this case, in our understanding, it is also possible that the network-sided model is trained by OAM or CN node and deployed at the gNB. Although there is no RAN2 impact, it is also suggested for RAN2 to confirm such understanding and involve RAN3 in WI if needed. Similarly, LMF may receive the LMF-sided model for positioning from OAM or CN, upon SA2 decision.

RAN2 assumes gNB may receive network-sided model from OAM or CN node. Upon RAN3 decision.

RAN2 assumes LMF may receive LMF-sided model from OAM or CN node. Upon SA2 decision.

### 2.3.3	AI Model Monitoring/Selection/…

On the other hand, no matter which option for AI model transfer is supported eventually, the AI/ML model monitoring and possible model selection, activation, deactivation, switching, and fallback operation should be performed by gNB/UE based on available measurements. Because using AI/ML based CSI feedback compression/prediction and beam prediction will directly impact the Uu interface performance (e.g., UL/DL throughput), and gNB/UE has the best knowledge about it based on its own measurement and UE report (e.g., measurement report, RLF report, BFD report etc.). This is also aligned with the following RAN1 agreement in their previous meeting.

For CSI feedback and beam management, the AI/ML model monitoring and possible selection/activation/deactivation/switch/fallback operation are performed by gNB/UE based on available measurements.

Similarly, for AI based positioning, the AI model monitoring and possible selection, activation, deactivation, switching, and fallback operation should be performed by LMF/gNB/UE based on available measurements.

For AI based positioning, the AI/ML model monitoring and possible selection/activation/deactivation/switch/fallback operation are performed by LMF/gNB/UE based on available measurements.

### 2.3.4	AI Model/Functionality Identification

R2 assumes that model ID can be used to identify which AI/ML model is being used in LCM including model delivery. R2 assumes that model ID can be used to identify a model (or models) during model selection/activation/deactivation/switching (can later align with R1 if needed).

In the last RAN2 meeting, RAN2 assumes that a model ID can be used to identify an AI/ML model in LCM. In fact, in our understanding, as long as an AI/ML model can be identified without ambiguity, it doesn’t really matter whether it is a global ID or local ID or even a local index. In one example, after gNB transfers a bunch of AI/ML models to UE, gNB can use a local index referring to the AI/ML model that has been transferred to UE before.

An AI/ML model can be identified during LCM by either a global ID or local ID or local index as long as there is no ambiguity.

In the meanwhile, RAN1 assumed in their last meeting that besides model itself, the AI/ML functionality can also be identified. This gives another possibility to perform LCM in the granularity of functionality. For instance, in case the UE sided model is provided by APP server, UE may only inform gNB about the functionality that it supports (e.g., AI for BM). In this case, when gNB performs LCM in terms of AI model monitoring and trigger possible activation/deactivation/fallback, it is done at the granularity of functionality.

Working Assumption Terminology; Description. Model identification; A process/method of identifying an AI/ML model for the common understanding between the NW and the UE Note: The process/method of model identification may or may not be applicable. Note: Information regarding the AI/ML model may be shared during model identification.. Terminology; Description. Functionality identification; A process/method of identifying an AI/ML functionality for the common understanding between the NW and the UE Note: Information regarding the AI/ML functionality may be shared during functionality identification. FFS: granularity of functionality. Note: whether and how to indicate Functionality will be discussed separately.

It is also possible that LCM is performed at the granularity of functionality, e.g., when UE receives UE-sided model from APP server and only inform gNB about the supported functionality (e.g., AI for BM).

RAN2 assumes LCM can be performed at the granularity of functionality in some cases, e.g., when UE receives UE-sided model from APP server and only inform gNB about the supported functionality (e.g., AI for BM).

# 3	Conclusion

Based on the discussion above, we observe:

Observation 1	AI/ML training/inference capabilities will be supported by gNB in Rel18 for network energy saving, load balancing and mobility optimization.

Observation 2	It is upon RAN3 discussion whether the AI/ML training/inference capabilities is located in CU or DU in a CU-DU split gNB architecture.

Observation 3	AI/ML management capabilities will probably be supported in Rel18/19 for OAM to deploy a trained AI/ML model to NG-RAN.

Observation 4	NW can be aware of the model selection, activation, deactivation, switching, and fallback based on its own decision or based on UE report according to RAN1 agreement.

Observation 5	It is also possible that LCM is performed at the granularity of functionality, e.g., when UE receives UE-sided model from APP server and only inform gNB about the supported functionality (e.g., AI for BM).

Based on the discussion above, we propose:

Proposal 1	For Network-UE level x collaboration (i.e., no collaboration), there is no RAN2 impact.

Proposal 2	RAN2 studies the possible enhancements to support Network-UE level y collaboration (i.e., signalling-based without model transfer) and level z collaboration (i.e., signalling-based with model transfer) taking into account RAN1 progress.

Proposal 3	For the content of data to be collected, RAN2 assumes

a.	All legacy measurements in RRC Connected state can be collected for AI based CSI feedback enhancement and BM use cases.

b.	All legacy measurements in RRC Connected and Inactive state can be collected for AI based positioning.

Proposal 4	RAN2 considers the data size requirements for (per time of) AI operations as following: AI model training > update > inference > monitoring/selection /activation/deactivation/switch/fallback

Proposal 5	For AI training data collection, RAN2 studies approaches to support sending training data (e.g., collection of measurements) in one package from UE to NW.

Proposal 6	RAN2 considers the latency/periodicity requirements for AI operations as following: AI model training > update/monitoring/selection /activation/deactivation/switch/fallback > inference.

Proposal 7	If the AI/ML model is transferred to UE from CN node (Solution 2a, 2b) or application server (Solution 4), RAN2 studies how can gNB/LMF be aware of the AI/ML model usage for the sake of model monitoring and possible selection/activation/deactivation/switch/fallback.

Proposal 8	RAN2 assumes gNB may receive network-sided model from OAM or CN node. Upon RAN3 decision.

Proposal 9	RAN2 assumes LMF may receive LMF-sided model from OAM or CN node. Upon SA2 decision.

Proposal 10	For CSI feedback and beam management, the AI/ML model monitoring and possible selection/activation/deactivation/switch/fallback operation are performed by gNB/UE based on available measurements.

Proposal 11	For AI based positioning, the AI/ML model monitoring and possible selection/activation/deactivation/switch/fallback operation are performed by LMF/gNB/UE based on available measurements.

Proposal 12	An AI/ML model can be identified during LCM by either a global ID or local ID or local index as long as there is no ambiguity.

Proposal 13	RAN2 assumes LCM can be performed at the granularity of functionality in some cases, e.g., when UE receives UE-sided model from APP server and only inform gNB about the supported functionality (e.g., AI for BM).

# 4	Reference