3GPP TSG-RAN WG2 Meeting #121	R2-2300955

Athens, Greece, 27 February-3 March, 2023

Agenda item:	 8.13.4

Source:		Lenovo

Title:		SON enhancements for SPR

Document for:		Discussion and Decision

1	Introduction

In RAN2#119bis-e meeting [1], agreements on SON enhancements for successful PSCell change report were achieved:

Agreements

1	RAN2 confirms the scenarios for SPR for NR-DC, including:

•	SN- and MN-initiated classic PSCell change / CPC

•	Intra-SN classic PSCell change / CPC

•	Classic Addition / CPA

1a	RAN2 will discuss HO with SN change later, after the basic solution for SPR is known

2	Given that PSCell addition is proposed by all companies, SPR is used as the abbreviations to use for the feature.

3	RAN2 confirm to prioritize NR-DC scenario for SPR.

4	SHR solution is taken as baseline for the SPR in terms of configuration and reporting at high level. Details of the configuration and report need to be tailored/customized/new message per use case.

5	Network configures SPR configuration IE for the UE, with at least the following triggering conditions:

•	T310 triggering condition

•	T312 triggering condition

•	T304 triggering condition

5a: Other triggering conditions are FFS

5b: Values of the triggering conditions are FFS

5c: Which node configures the triggering condition is FFS.

6	RAN2 agree to the following:

A.	SPR configuration is configured by network through otherConfig

B.	SPR is fetched via UE Information Request/Response procedure

7	UE logs at least the following information and measurements in the SPR IE (other information and measurements are FFS).

a)	Source PSCell info (cell ID, measurement result)

b)	Target PScell info (cell ID, measurement result)

c)	Neighbour Cells info (cell ID, measurement result, CPAC Candidate cells flag)

d)	Success PSCell change/addition cause value (e.g., t304, t310, t312 cause, etc.)

f)	The time elapsed between the CPAC execution towards the target cell and the corresponding latest CPAC configuration received for the selected target cell

7a: FFS on whether to reuse CHO candidate cell flag for the CPAC candidate cells or define a new flag to indicate CPAC candidate cell.

7b: FFS on whether to include or on conditional inclusion of random access related information.

7c:	FFS on Location Information

In RAN2#120 meeting [2], further agreements were achieved:

Agreements:

3	Only MN can retrieve the SPR from the UE.

4	For Q8, RAN2 agree following options: depends on which of nodes initiates SPR, i.e.:

For the MN-initiated PSCell Change/Addition, MN sends the SPR config to the UE

For the SN-initiated PSCell Change, the source-SN sends the Successful PSCell Change configuration within the container through MN.

T304 trigger needs to be configured by the target SN node.

Agreements:

1	UE stores both SPCR and SHR configuration (one for each type at most) if received from NW.

2	UE can send the (stored) SPR to gNB. FFS how long UE keeping SPR is FFS.

3	Only the latest successful PSCell change/addition is reported by the UE.

4	Random access related information is included in SPR at least when the SPR is triggered due to T304 exceeds the configured threshold. Other conditions are FFS.

5	UE records/reports PCell SHR and PSCell SPR separately

In this paper, details of SON enhancements for SPR are further discussed.

2	Discussion

Similar as T310/T312/T304 related trigger condition for SHR, for reporting successful PSCell addition/change, RAN2#119bis-e meeting agreed that configuration for generating SPR e.g. T310/T312/T304 trigger threshold associated with SCG can be configured to the UE. The values of the triggering conditions are FFS, similar as T310/T312/T304 trigger condition for SHR, a percentage value rather than an absolute value can be configured as the trigger condition of the SPR.

Proposal 1: A percentage value of T310/T312/T304 can be configured as the trigger condition of the SPR.

RAN2#120 meeting agreed that “For the SN-initiated PSCell Change, the source-SN sends the Successful PSCell Change configuration within the container through MN”, it implies that source SN generates/decides T310/T312 trigger threshold for SPR in SN-initiated PSCell Change. Similarly, for SN-initiated CPC, source SN generates/decides T310/T312 trigger threshold for SPR, and the source SN sends the Successful PSCell Change configuration within the container through MN to the UE.

Proposal 2: For the SN-initiated CPC, the source SN sends the Successful PSCell Change configuration within the container through MN to the UE.

For MN initiated PSCell change or CPC, it is suitable for MN to perform root cause analysis, thus MN can be the node to generate/decide T310/T312 trigger threshold for SPR, on the other hand, since T310/T312 trigger threshold of SPR may be a percentage value rather than an absolute value, it is not needed for the MN to achieve the configured timer value of T310/T312 from source SN when the MN generates/decides T310/T312 trigger threshold.

Proposal 3: For the MN-initiated PSCell Change/CPC, MN decides T310/T312 trigger threshold for SPR.

RAN2#120 meeting agreed that “For the MN-initiated PSCell Change/Addition, MN sends the SPR config to the UE”, similarly, for MN-initiated CPC/CPA, MN sends the SPR configuration to the UE.

Proposal 4: For the MN-initiated CPC/CPA, MN sends the SPR configuration to the UE.

When at least one configured trigger condition for SPR is fulfilled, SPR is stored by the UE including some necessary information for analysis of the near-failure PSCell addition/change, e.g. source PSCell information in case of PSCell change/CPC, target PSCell information, SPR cause and time elapsed between the CPAC execution and reception of CPAC configuration in case of CPAC. RAN2#120 meeting agreed that UE records/reports PCell SHR and PSCell SPR separately, it is straightforward to define a new IE and a new variable for logging and storing SPR in the UE to distinguish and decouple it from SHR, especially for the case of successful PCell change with successful PSCell change, i.e when the UE needs to store both SHR and SPR.

Proposal 5: SPR is logged in a new IE and stored in a new UE variable.

Besides source PSCell information, target PSCell information, neighbour cell information, SPR cause, time elapsed between the CPAC execution and reception of CPAC configuration, and random access related information at least when the SPR is triggered due to T304 exceeds the configured threshold, location information can also be reported. For the cell which is a CPAC candidate PSCell, it is better to define a new flag to indicate CPAC candidate PSCell which is clearer compared with reusing CHO candidate cell flag.

Proposal 6: The following information can be included in the SPR:

- location information

- a new flag to indicate CPAC candidate PSCell

After the PSCell addition or CPA as well as PSCell change or CPC procedure is successfully completed, simailar as SHR, it is not urgent to sent SPR immediately, i.e. the stored SPR can be sent to the network later. RAN2#119bis-e meeting agreed that SPR is fetched via UE Information Request/Response procedure, furthermore, RAN2#120 meeting agreed that UE can send the stored SPR to gNB, which means that an indication for availability of SPR may be transmitted to MN by the UE, based on the received indication, MN may request the SPR from the UE via UE Information Request message, and then the UE may transmit the stored SPR to MN via UE Information Response message.

Proposal 7: The UE sends an indication for availability of SPR to MN, MN may request and receive the SPR from the UE via the UE Information Request/Response procedure.

In RAN2#119bis-e meeting, it is FFS how long UE keeping SPR. Since SPR is not urgent to be sent to the network immediately, the UE may keep the stored SPR until at least one of the following cases happens:

-	a new SPR is initiated;

-	retrieval of SPR is successfully completed;

-	specific time (e.g. 48 hours) has passed since SPR is stored/logged;

-	upon power off or detach is initiated.

Proposal 8: The UE keeps the stored SPR until at least one of the following cases happens:

-	a new SPR is initiated;

-	retrieval of SPR is successfully completed;

-	specific time (e.g. 48 hours) has passed since SPR is stored/logged;

-	upon power off or detach is initiated.

3	Conclusion

In this contribution, the details of SON enhancements for SPR are discussed. We have the following proposals:

Proposal 1: A percentage value of T310/T312/T304 can be configured as the trigger condition of the SPR.

Proposal 2: For the SN-initiated CPC, the source SN sends the Successful PSCell Change configuration within the container through MN to the UE.

Proposal 3: For the MN-initiated PSCell Change/CPC, MN decides T310/T312 trigger threshold for SPR.

Proposal 4: For the MN-initiated CPC/CPA, MN sends the SPR configuration to the UE.

Proposal 5: SPR is logged in a new IE and stored in a new UE variable.

Proposal 6: The following information can be included in the SPR:

-	location information

-	a new flag to indicate CPAC candidate PSCell

Proposal 7: The UE sends an indication for availability of SPR to MN, MN may request and receive the SPR from the UE via the UE Information Request/Response procedure.

Proposal 8: The UE keeps the stored SPR until at least one of the following cases happens:

-	a new SPR is initiated;

-	retrieval of SPR is successfully completed;

-	specific time (e.g. 48 hours) has passed since SPR is stored/logged;

-	upon power off or detach is initiated.

Reference

[1] Chairman notes for RAN2#119bis-e meeting

[2] Chairman notes for RAN2#120 meeting