3GPP TSG-RAN WG2 Meeting #121	R2-2300963

Athens, Greece, 27th Feb.-3rd Mar., 2023

Agenda item:		8.4.2.2

Source:		Lenovo

Title:		Compliance check for LTM configuration

Document for:		Discussion and Decision

- Introduction

The following agreements related to LTM are obtained [1] [2].

- FFS whether ASN.1 decoding and validity/compliance check of candidate cell configuration are performed upon reception of the candidate cells configuration. FFS if this need to be specified.

In this contribution, we discuss the compliance issues for LTM.

- Discussion

Figure: LTM procedure

According to Figure, LTM is a procedure in which a gNB provides the candidate cell configurations to the UE through RRC message based on the received measurement reports from UEs. The gNB can configure UE to perform early TA procedure. Furthermore, the gNB changes UEs’ serving cell(s) through MAC CE. Regarding legacy CHO and CPAC, it is up to UE implementation to perform compliance check upon the reception of the message or upon CHO, CPA and CPC execution. Comparing to the CHO/CPAC, UE also can perform compliance check upon early TA acquisition execution. Therefore, UE can perform compliance check for LTM configuration upon reception of candidate cell configuration, performing early TA acquisition or executing cell switching.

Some companies argue that latency for compliance check can be reduced if compliance check can be performed before cell switching. Considering the diversity in email and online discussion, we can have a compromised solution. Namely, it is UE implementation to perform compliance check for LTM configuration upon reception of candidate cell configuration, performing early TA acquisition or executing cell switching.

Proposal 1: It is UE implementation to perform compliance check for LTM configuration upon reception of candidate cell configuration, performing early TA acquisition or executing cell switching.

It was agreed that candidate cell configuration can be provided as delta configurations on top of a reference configuration. The reference configuration is managed separately, and a UE stores the reference configuration as a separate configuration. Therefore, compliance check should also cover the reference configuration.

Observation 1: A UE stores the reference configuration as a separate configuration.

Proposal 2: The compliance check covers the reference configuration.

- Conclusion

In this contribution, we have the following proposals:

Proposal 1: It is UE implementation to perform compliance check for LTM configuration upon reception of candidate cell configuration, performing early TA acquisition or executing cell switching.

Observation 1: A UE stores the reference configuration as a separate configuration.

Proposal 2: The compliance check covers the reference configuration.

References

RAN2 chairman notes for RAN2#119bise.

RAN2 chairman notes for RAN2#120.