3GPP TSG-RAN WG2 Meeting #121	R2-2300965

Athens, Greece, 27th Feb.-3rd Mar.,2023                      Revision of R2-2212025

Agenda item:		8.9.2

Source:		Lenovo

Title:		Discussion on L2 U2U relay

Document for:		Discussion and Decision

- Introduction

This document discusses aspects related to relay selection, relay reselection, fallback from relay operation to direct PC5 link and bearer multiplexing in L2 U2U relay case.

- Discussion

Relay Selection:

It was agreed in RAN2#120 meeting that that UE-to-UE relay selection can be triggered based on the PC5 RSRP (FFS SL-RSRP or SD-RSRP) of the direct link falling below a threshold. Once PC5 measurement towards target remote UE is less than the (pre)configured threshold, the remote UE will be triggered to perform relay selection. If UE releases PC5 link immediately while PC5 measurement towards target remote UE is less than the (pre)configured threshold, it will result in the interruption. Therefore, an exchange between two remote UEs could be needed before releasing direct PC5 link. For example, when the transmitting remote UE is triggered to perform the relay selection based on the (pre)configured threshold, the transmitting remote UE transmits a message including the indication of relay selection to the peer remote UE. Furthermore, the candidate relay UE(s) can be included in the message. The peer remote UE can respond back to the transmitting remote UE indicating which Relay UE is the best/ suitable among the Relays included by the transmitting remote UE. If this mechanism is not used, RAN2 at least need to ensure that the selected relay must be the one which has suitable radio connection to both transmitter and receiver remote UEs.

Observation 1: A remote UE performs relay selection when PC5 measurement towards the peer remote UE is less than a (pre)configured threshold.

Proposal 1: When transmitting remote UE performs the relay selection based on the (pre)configured threshold, a PC5 RRC message including the indication of relay selection is sent to receiving (peer) remote UE.

Proposal 2a: When transmitting remote UE performs the relay selection based on the (pre)configured threshold, the transmitting remote UE sends a potential list of candidate Relay UEs to the receiving (peer) remote UE. The list is transmitted directly on PC5 RRC.

Proposal 2b: If P2a is not acceptable, RAN2 at least needs to ensure that the selected relay must be the one which has suitable radio connection to both transmitting remote UE and the receiving (peer) remote UEs.

As usual, the UE may detect the sidelink RLF of the PC5 direct link between two remote UEs. Once the RLF on the direct PC5 link is detected, the remote UE capable of U2U relay operation should perform relay selection.

Proposal 3: RLF on the direct PC5 link between two remote UEs can also be used to trigger relay selection.

Relay reselection:

Fig.1 U2U relay operation

It was agreed by RAN2 that UE-to-UE relay reselection can be triggered based on the PC5 RSRP (FFS SL-RSRP or SD-RSRP) between a remote UE and the relay UE falling below a threshold. In this way, one threshold will be configured to the remote UE e.g UE2 in Fig.1. UE2 determines the PC5 link between the relay UE and UE2 is less than the threshold associated with relay reselection, the remote UE2 indicates to the relay UE. Then, the relay UE transmits the indication such as PC5 unicast link release to the remote UE1. The remote UE1 will perform relay reselection as well.

If the threshold associated with relay reselection is also (pre)configured to the relay UE, the relay UE can transmit the indication of relay reselection to the first remote UE once the relay UE detects the PC5 link between the relay UE and the second remote UE is less than the threshold associated with relay reselection.

Proposal 4: Once the second (receiving) remote UE detects the PC5 link between the relay UE and the second(receiving) remote UE is less than the threshold associated with relay reselection, the second(receiving) remote UE indicates to the relay UE. Then, the relay UE transmits the indication to the first (transmitting) remote UE.

Proposal 5: If the threshold associated with relay reselection is also (pre)configured to the relay UE, the relay UE can transmit the indication of relay reselection to the first (transmitting) remote UE once the relay UE detects the PC5 link between the relay UE and the second (receiving) remote UE is less than the threshold associated with relay reselection.

Furthermore, ‘PC5 link release message from relay to remote’ was agreed as a condition to trigger relay reselection of a remote UE. RAN2 needs to discuss what condition will trigger the relay UE to transmit the PC5 unicast link release message. When the relay UE detects the PC5 RLF of the link between the relay UE and the second remote UE, the relay UE needs to indicate to the first remote UE to perform relay reselection e.g via PC5 unicast link release message.

Proposal 6: A U2U relay UE is triggered to transmit PC5 unicast link release message to the first remote UE if the relay UE detects the PC5 RLF on the link with the second remote UE.

Fallback to direct PC5 link

When a remote UE communicates another remote UE via a U2U relay UE, the direct PC5 link may become better. In this case, the remote UE may fall back to the direct PC5 link between two remote UEs. Once the quality of the PC5 direct link is better than the configured threshold, the remote UE can fall back to the direct PC5 link. Namely, the remote UE establishes the PC5 link to the peer remote UE and release the link between the remote UE and the relay UE.

Proposal 7: Remote UE can switch back from the U2U relay operation to direct PC5 link when Remote UE does not need U2U relay anymore.

Proposal 8: Remote UE can switch back from the U2U relay operation to direct PC5 link if PC5 signal strength condition of direct PC5 link between two remote UEs is better than a threshold.

Bearer Multiplexing

Figure 2 Control plane protocol stack for L2 UE-to-UE Relay

It was agreed in SI stage that the adaptation layer is located in the first hop and second hop between the UE1 and UE2. We don’t need to discuss again whether to add adaption layer into each hop.

Proposal 9: SRAP layer is needed for each hop in U2U relay case.

For the first hop of L2 UE-to-UE Relay, the N:1 mapping between remote UE SL Radio Bearers and first hop PC5 RLC channels should be supported in the first hop PC5 adaptation layer. Furthermore, the adaptation layer over first PC5 hop between Source Remote UE and Relay UE supports to identify traffic destined to different destination remote UEs. It is from resource efficency point of view beneficial to support that packets associated with the different destination UEs served by the same relay UE can be multiplexed in one TB for the first hop.

Proposal 10: In U2U relaying, multiplexing of sidelink data by the transmitter remote UE towards more than one Rx remote UE served by the same relay node into a TB is supported.

Figure 3

It was agreed in SI stage that PC5 adaptation layer of the second hop supports the N:1 bearer mapping between multiple ingress PC5 RLC channels over first PC5 hop and one egress PC5 RLC channel over second PC5 hop and supports the remote UE identification function. If the data from the different transmitting remote UEs towards the same destination UE are received by the relay UE, it is benefitial to allow the data from the different transmitting remote UEs to be multiplexed at the relay UE.

Proposal 11: In U2U relaying, the multiplexing data from the different transmitting remote UEs towards the same destination UE at the relay UE is supported.

- Conclusion

In this contribution, we have the following proposals:

Observation 1: A remote UE performs relay selection when PC5 measurement towards the peer remote UE is less than a (pre)configured threshold.

Proposal 1: When transmitting remote UE performs the relay selection based on the (pre)configured threshold, a PC5 RRC message including the indication of relay selection is sent to receiving (peer) remote UE.

Proposal 2a: When transmitting remote UE performs the relay selection based on the (pre)configured threshold, the transmitting remote UE sends a potential list of candidate Relay UEs to the receiving (peer) remote UE. The list is transmitted directly on PC5 RRC.

Proposal 2b: If P2a is not acceptable, RAN2 at least needs to ensure that the selected relay must be the one which has suitable radio connection to both transmitting remote UE and the receiving (peer) remote UEs.

Proposal 3: RLF on the direct PC5 link between two remote UEs can also be used to trigger relay selection.

Proposal 4: Once the second (receiving) remote UE detects the PC5 link between the relay UE and the second(receiving) remote UE is less than the threshold associated with relay reselection, the second(receiving) remote UE indicates to the relay UE. Then, the relay UE transmits the indication to the first (transmitting) remote UE.

Proposal 5: If the threshold associated with relay reselection is also (pre)configured to the relay UE, the relay UE can transmit the indication of relay reselection to the first (transmitting) remote UE once the relay UE detects the PC5 link between the relay UE and the second (receiving) remote UE is less than the threshold associated with relay reselection.

Proposal 6: A U2U relay UE is triggered to transmit PC5 unicast link release message to the first remote UE if the relay UE detects the PC5 RLF on the link with the second remote UE.

Proposal 7: Remote UE can switch back from the U2U relay operation to direct PC5 link when Remote UE does not need U2U relay anymore.

Proposal 8: Remote UE can switch back from the U2U relay operation to direct PC5 link if PC5 signal strength condition of direct PC5 link between two remote UEs is better than a threshold.

Proposal 9: SRAP layer is needed for each hop in U2U relay case.

Proposal 10: In U2U relaying, multiplexing of sidelink data by the transmitter remote UE towards more than one Rx remote UE served by the same relay node into a TB maybe supported.

Proposal 11: In U2U relaying, the multiplexing data from the different transmitting remote UEs towards the same destination UE at the relay UE is supported.

References

Chairman notes for RAN2#120 meeting.