3GPP TSG-RAN WG2 Meeting #121	R2- 2300978

Athens, 27th February – 3rd March, 2023

Agenda item:		8.3.5

Source:		Lenovo

Title:		NES impact to UE mobility

Document for:		Discussion and Decision

# Introduction

The following progresses for NES techniques are achieved in RAN2 #120 [1] and the related content was captured in TR 38.864after the issue on connected mode mobility was discussed.

Agreements

- Capture the solution on enhancing the CHO framework (for faster offloading/onloading during cell deactivation/activation) enabling a evaluation of CHO conditions depending on the NES state of the source/target cell. How to indicate to UE the triggering of the CHO evaluation is up to normative phase. Whenever mobility from source cell is triggered, one could also consider how UE would not select NES cell if any other cell is available when selecting the new cell. Corresponding TP for this is provided in the Annex

- RAN2 does not consider at this point group HO (optimizing R15 HO procedure).

- RAN2 does not consider at this point BWP adaptation with group signaling (no supporting papers in RAN2)

- From RAN2 perspective, CHO enhancements are feasible

In this contribution, we further discuss the related issues.

# Discussion

There could be the following cases from a network deployment perspective:

Case a) a cell implementing Rel-18 network energy savings techniques (NES cell) neighbors to the legacy cells;

Case b) a NES cell neighbors to the NES cells;

Case c) a NES cell neighbors to the legacy cells and NES cells as shown in Figure 1;

Considering the mobility of the UE that supporting the NES technique (NES UE), the UE could handover from the NES cell to the NES cell or legacy cell. Correspondingly, it could be possible that a NES UE connected to a legacy cell, and the UE handovers from the legacy cell to a NES cell. The above cases are all within the NES scope of considerations.

Observation 1: A NES UE can handover between an NES cell and a non-NES cell (legacy or otherwise).

Figure 1 NES cell neighboring to legacy cell and NES cell

For case c), a NES UE is at the edge of NES cell #1, for example the NES cell #1 is a single carrier or the anchor carrier of the NES UE. Both the NES cell #2 and the legacy cell #3 could be the target cell of the UE according to the measurement report. it is better for the source cell to prioritize the NES cell when selecting of the target cell for the NES UE from the perspective of power saving. But to some extent, whether the UE can prioritize a NES cell as a target cell could be related to the UE itself, for example, the traffic, the battery, etc. Therefore, in order to (de-)prioritize the NES cell as a target cell, the NES-capable UE can provide its preference for the state of the targe cell to the source cell.

Proposal 1: In order to (de-)prioritize the NES cell as a target cell, the NES-capable UE can provide its preference for the state of the targe cell to the source cell.

Correspondingly, the source cell will determine a targe cell for the UE based on the preference of the UE and the NES state of the target cell. Then, once the candidate targe cell is configured to the UE, the UE considers the NES state of the target cell is acceptable.

Proposal 2: Once the candidate targe cell is configured to the UE, the UE considers the NES state of the target cell is acceptable.

On the other side, if a legacy UE is at the edge of a cell. Both the NES cell and the legacy cell could be the target cell of the UE according to the measurement report. It is possible for the NES cell to apply the energy saving technologies, for example, Longer SSB/SIB1 periodicity, state transition to sleep mode, etc. The cell could change to an unsuitable cell for a legacy UE after the legacy UE handovers to NES cell and the NES cell starting to use the energy saving technologies. Although the network will hand over the legacy UE to another cell, it introduces signaling overhead. Shall the NES cell in the normal state be de-prioritized for the legacy UE among the candidate target cells because of the potential starting to use energy saving technologies?

Proposal 3: RAN2 to discuss whether the cell that supports NES technique(s) but is in the normal state shall be de-prioritized for the legacy UE among the target cells.

The corresponding aspects can be considered in the case of CHO, for example, the cell supports NES can be configured to the NES UE as one of the conditions in CHO case. Also, it was summarized as the Solution 2: Enhanced CHO by prioritizing NES cells in CHO condition evaluation [2] after the RAN2#119bis email discussion. However, it has not determined how to delay the UE to trigger the CHO evaluation. Referring to the discussion in NTN, we think event T1 can be reused to delay the evaluation of CHO.

Proposal 4: Reuse event T1 to delay the evaluation of CHO.

# Conclusion

In this contribution, we discuss the handover aspects in Rel-18 NES and provide the proposals:

Observation 1: A NES UE can handover between an NES cell and a non-NES cell (legacy or otherwise).

Proposal 1: In order to (de-)prioritize the NES cell as a target cell, the NES-capable UE can provide its preference for the state of the targe cell to the source cell.

Proposal 2: Once the candidate targe cell is configured to the UE, the UE considers the NES state of the target cell is acceptable.

Proposal 3: RAN2 to discuss whether the cell that supports NES technique(s) but is in the normal state shall be de-prioritized for the legacy UE among the target cells.

Proposal 4: Reuse event T1 to delay the evaluation of CHO.

# References

RAN2#120 chairman notes.

R2-2209758, Discussion on Network energy saving for CONNECTED UE - group CHO and BWP adaptation,	Apple