3GPP TSG-RAN WG2 Meeting #121	R2-2300980

Athens, Greece, February-March 2023                       Revision of R2-2212045

Agenda item:		8.6.3.1

Source:		Lenovo

Title:		CONNECTED neighbour cell measurement for NB-IoT in NTN

Document for:		Discussion

- Introduction

The latest RAN2#120 meeting made the following agreements regarding CONNECTED neighbour cell measurement for NB-IoT in NTN:

Agreements: - For NB-IoT we support a trigger for neighbour cell measurements based on t-Service (in the quasi-Earth fixed case) (this does not preclude anything for eMTC discussion) - At least for NB-IoT NTN, for quasi-earth fixed cells, UE shall start intra/inter frequency measurement in connected mode before the t-Service if present. The exact time to start measurements in connected mode before t-Service can be left to UE implementation” (can revisit if we agree other proposal based on neighbour cell coverage) - RAN2 will not specify the condition of stopping UE measurement before t-Service - For earth-moving cell, the UE derives when loss of coverage of current cell happens (how to derive this information is FFS) - For earth-moving cell, UE shall start intra/inter frequency measurements in RRC connected mode before losing coverage. The exact time to start measurements can be left to UE implementation

In this contribution we further discuss NB-IoT CONNECTED mode neighbour cell measurement and provide some options to solve the remaining issues.

- Discussion

In the latest RAN2#120 meeting, it was agreed to work on a trigger based on t-Service for quasi-Earth-fixed cells, wherein the implementation could be much easier as the t-Service is cell-specific, and thus eNB can indicate UE to initiate measurement before t-Service. For Earth-moving cells the actual stop serving time is location-specific and thus UE-specific, making it a bit more complicated for triggering the measurement. To help the UE derive when loss of coverage of current cell happens, eNB may need to indicate assistance information to help UE determine the stop serving time.

Observation 1: At least for Earth-moving cells, time-based trigger may not be simply implemented by eNB indication and further assistance information could be necessary.

And therefore:

Proposal 1: For time-based trigger in Earth-moving cells, RAN2 to discuss what assistance information shall eNB provide to help UE determine the stop serving time.

Meanwhile, the distance-based trigger was also FFS in the latest RAN2#119 meeting. Our understanding is that, for both quasi-Earth-fixed and Earth-moving cells, if eNB is aware of UE location information, it may directly indicate UE to initiate the measurement. Otherwise distance-based trigger could be considered and somehow could reuse what has been agreed for NR NTN neighbour cell measurement triggering in IDLE/INACTIVE. Meanwhile, distance-based trigger could be a bit more complicated for Earth-moving cells, as the cell reference location moves alongside with the satellite.

Observation 2: For Earth-moving cells, distance-based trigger may not be simply implemented due to cell movement.

For the indication of reference location in Earth-moving cells, the following options can be considered:

- Option 1: Multiple reference locations. Each reference location could be associated to a time duration, an area or elevation angle. And UE could apply one reference location based on the current time, its area or elevation angle.

- Option 2: Multiple offsets to referenceLocation. Each offset could be associated to a time duration, an area or elevation angle. And UE could apply one offset based on the current time, its area or elevation angle.

- Option 3: UE estimates real-time reference location based on ephemeris. Network could indicate UE that the reference location is varying and thus needs to be estimated.

- Option 4: Additional change information, e.g., drift or velocity vector of referenceLocation. UE estimates real-time reference location based on the change information.

- Option 5: Set the referenceLocation to the position of the satellite, so that UE-satellite distance is used for evaluation instead, which can be derived directly from UE-specific TA.

Proposal 2: If distance-based trigger is agreed, RAN2 to discuss how to efficiently indicate the cell reference location of an Earth-moving cell to the UE.

Additionally, in case of discontinuous coverage where UE can remain long periods of time without being able to detect a satellite cell, there could be no neighbour cell available for a UE to be measured but the measurement could still be triggered.

Observation 3: In discontinuous coverage scenario wherein no neighbour cell is available to be measured, unnecessary CONNECTED mode neighbour cell measurement could be triggered.

For NTN deployment with discontinuous coverage, additional rules can be considered to disable NB-IoT CONNECTED mode neighbour cell measurement when it is unnecessary to further reduce its power consumption.

Proposal 3: For discontinuous coverage scenario, the NB-IoT CONNECTED mode neighbour cell measurement can be disabled when there is no neighbour cell coverage.

- Conclusion

In this contribution, we discuss time/distance-based trigger for NB-IoT CONNECTED mode neighbour cell measurement. The following observation are given:

Observation 1: At least for Earth-moving cells, time-based trigger may not be simply implemented by eNB indication and further assistance information could be necessary.

Observation 2: For Earth-moving cells, distance-based trigger may not be simply implemented due to cell movement.

Observation 3: In discontinuous coverage scenario wherein no neighbour cell is available to be measured, unnecessary CONNECTED mode neighbour cell measurement could be triggered.

And we propose:

Proposal 1: For time-based trigger in Earth-moving cells, RAN2 to discuss what assistance information shall eNB provide to help UE determine the stop serving time.

Proposal 2: If distance-based trigger is agreed, RAN2 to discuss how to efficiently indicate the cell reference location of an Earth-moving cell to the UE.

Proposal 3: For discontinuous coverage scenario, the NB-IoT CONNECTED mode neighbour cell measurement can be disabled when there is no neighbour cell coverage.