3GPP TSG-RAN WG2 Meeting #121	R2- 2300993

Athens, Greece, 27 February – 18 March, 2023

Agenda item:	8.8.5

Source: 	Huawei, HiSilicon

Title: 	Consideration on UAV remote identification broadcast

Document for:	Discussion and Decision

- Introduction

In RAN meeting #94e, a new work item was approved for NR UAV [1] and one of the objectives is about the UAV identification broadcast as follows:

3. Study and specify, if needed, enhancements for UAV identification broadcast [RAN2, SA2].

Applicable to both LTE and NR

Note: This study should consider existing techniques based on Uu or non 3GPP technologies, or unlicensed band as the baseline.

Note: This description is a placeholder for a more detailed objective to be drafted once SA2 will have concluded their study on the architectural aspects.

In this contribution, we would like to clarify the RAN2 work on the UAV identification broadcast and some related proposals will be provided.

- Discussion

According to the NR UAV WID, RAN2’s work on UAV ID broadcast needs to wait for the conclusion of SA2. There were some agreements reached for BRID in the last SA2 meeting.

SA2 has agreed that UAV ID broadcasts can use both the MBS-based and PC5-based solutions. The USS/UTM (server) first requests the 5G system to establish the broadcast session for the MBS-based solution procedure [2], after which the UAV registers to the network and reports its ID to the server. If the broadcast session is established, the USS/UTM server starts to broadcast the remote ID of the UAV. We can see that there is no impact on RAN2 for this procedure.

Observation 1: There is no impact on RAN2 for the MBS-based UAV ID broadcast.

It is possible to use PC5-based UAV ID broadcast for both UAV ID broadcast and the detect and avoid mechanism (DAA) [2]. For the PC5-based UAV ID broadcast and direct DAA procedure, the application layer of UAV1 receives the broadcast message from UAV2 and detects a conflict. Then, the UAV1 selects a communication mode (broadcast or unicast) for DAA deconfliction and broadcasts a message (e.g., PC5-U message), e.g., deconfliction request message. Finally, UAV2 broadcasts a message (e.g., the PC5-U message) to provide the agreed DAA deconflicting policy. As a result, there is no impact on RAN2 due to the PC5-U message being used.

Observation 2: The PC5-U message is used to broadcast PC5-based UAV IDs.

One potential issue with PC5-based UAV ID broadcast is that a large amount of data may be required to be transferred due to a large number of UAV-UE in a specific area. This issue was also clarified in the last SA2 meeting, as follows [3].

Solution #5 may have the risk of a high potential amount of data that may need to be transferred, since in case of large number of UAVs in an area the solution needs to broadcast of a big amount of data. This issue may also happen to transport Broadcast Remote ID for Key Issue#2 as described in clause 5.2. There may be the scenario that a large number of Vehicle UEs in an area broadcast V2X messages over PC5, e.g. 10 messages per second from each UE. Such scenario has been already investigated for V2X sidelink communication by RAN WGs as in TR 36.885 [aa] and can be managed by the AS layer, e.g. Sidelink congestion control performed by RAN node for scheduled resource allocation mode and performed by UE for UE autonomous resource selection mode as defined in TS 36.300 [xx], TS 38.300 [yy] and TR 37.985 [zz]. Therefore, it is considered that there is no issue on the potential large amount of data transfer related to PC5 based DAA mechanism and PC5 based BRID mechanism.

As we can see, sidelink congestion control can handle this issue. Thus, there is also no impact on RAN2.

Observation 3: There is no impact on RAN2 from the large amount of potential data transfer related to PC5-based UAV ID broadcasts.

For the PC5-based "Detect and Avoid" mechanism. We can see that in clause 8.3 [2], there is the following note:

NOTE 3: Whether the UAV UEs served by different PLMNs and with subscriptions to different PLMNs is supported depends on the Rel-18 NR sidelink RAN study conclusion.

It mentioned that whether UAV supports inter-PLMN and inter-frequency sidelink communication depends on the RAN study's conclusion. In order to handle this issue, SA2 also sent a LS to the RAN2 [4]. The main issue is as follows:

The question was raised in SA2 whether and how PC5 based DAA mechanism can work between UAV UE#1 served by PLMN#1 operating Frequency Band#1 and UAV UE#2 served by PLMN#2 operating Frequency Band#2 in an area.

SA2 also indicates that inter-PLMN, inter-frequency sidelink communication has been supported in LTE. Based on the SA2 LS, we checked the LTE V2X discussion and found the following agreements for V2X sidelink communication reached in the RAN2 #97 meeting.

- Serving PLMN can provide V2x sidelink rx configuration of other PLMN(s) to UE for inter-PLMN V2x communication.

- The serving PLMN indicates to the UE the frequency carrier on which the UE may acquire the inter-PLMN sidelink resource configuration.

We can see that inter-PLMN, inter-frequency sidelink communication is indeed supported in LTE.

Observation 4: Inter-PLMN, inter-frequency sidelink communication is supported in LTE.

Rel-16 assumes that sidelink communication can only support a single carrier, and Rel-17 adopt this assumption as well. Therefore, inter-frequency sidelink communication is not supported now.

Observation 5: Inter-frequency sidelink communication is not supported in NR.

To handle the question mentioned in SA2 LS, it is better to introduce inter-PLMN inter-frequency sidelink communication in NR. Because the sidelink UE supported by different operators is a common case, i.e., the UAV served by different operators needs to exchange information frequently. Although sidelink CA will be studied in Rel-18, they are different. In this case, we can follow the LTE mechanism, i.e., allow the UAV-UE to receive the different frequency’s message. Thus, RAN2 should discuss whether to support inter-PLMN, inter-frequency sidelink communication for NR.

Proposal 1: RAN2 should discuss whether to support inter-PLMN, inter-frequency sidelink communication for NR.

- Conclusion

Observation 1: There is no impact on RAN2 for the MBS-based UAV ID broadcast.

Observation 2: The PC5-U message is used to broadcast PC5-based UAV IDs.

Observation 3: There is no impact on RAN2 from the large amount of potential data transfer related to PC5-based UAV ID broadcasts.

Observation 4: Inter-PLMN, inter-frequency sidelink communication is supported in LTE.

Observation 5: Inter-frequency sidelink communication is not supported in NR.

Proposal 1: RAN2 should discuss whether to support inter-PLMN, inter-frequency sidelink communication for NR.

- Reference

- RP-213600 “WI on NR Support for UAV”.

- 23.700-58 V18.0.0 “ Study of further architecture enhancements for uncrewed aerial systems and urban air mobility”

- S2-2301770_was00404r01_23700-58_DAA

- S2-2301854 “ LS on PC5 based Detect and Avoid mechanism”