3GPP TSG-RAN WG2 Meeting #121	R2-2300995

Athens, Greece, 27th Feb. – 03rd Mar., 2023

Agenda Item:	8.7.4.1.2

Source: 	ITL

Title: 	Discussion on NTN-NTN Cell re-selection

Document for:	Discussion and decision

Introduction

In the past RAN2 meetings [1-2], the agreements related to this contribution are as follows:

Agreements: 1. RAN2 to work on a solution so that measurements for TN’s coverage are performed only when relevant (FFS what relevant means). 2. RAN2 to work on assistance information that can be provided to NTN UEs for the above. 3. Cell reselection enhancements (for both NTN-NTN and NTN-TN mobility) are considered for both Earth-moving and (quasi-)Earth-fixed scenarios, at least via the use of system information for broadcasting necessary parameters (dedicated signalling is not precluded). FFS whether the same or different solutions are used for Earth-moving and (quasi-)Earth-fixed scenarios Agreements: 1. For NTN-NTN cell reselection with earth moving cell, RAN2 will consider providing parameters of serving cell to UE, for UE to estimate when the serving cell stops providing coverage at the present UE location (FFS whether this will be an optional UE feature) (this does not exclude any time-based or location-based approach) (other solutions can also be considered) 2. To enhance NTN-TN cell reselection, means are defined for a UE to differentiate when camping in an area only covered by NTN network (earth-moving or earth-fixed) vs an area where TN network(s) is/are also available. Agreements: 1. System information is the basic means for providing necessary parameters to assist UE to estimate when the serving cell stops providing coverage at the present UE location. 2. UE is not required to perform neighbour cell measurements for TN neighbour cells in an area where there is no TN network coverage. 3. The method of detecting the transmission energy or SIB presence to determine the NTN coverage when a UE currently camps on a TN cell is not pursued. 4. In Earth-moving cell, the reference location and distance threshold of serving cell are provided by network for UE to estimate when the serving cell stops providing coverage at the present UE location. FFS how the reference location and/or distance threshold are provided to the UE

In this contribution, we focus on cell reselection enhancements for earth moving cell.

Discussion

NTN-TN and NTN-NTN mobility and service continuity enhancements have been discussed in Rel-17 NTN. During the discussion of Rel-17 NTN, timing and location based cell re-selection for earth-fixed cell was mainly discussed because the fluctuation of received signal strength in NTN is smaller than in TN.

- Timing based cell re-selection: Network provides the time information (t-Service) that when a cell is going to stop serving the area to UE. Thus, UE measures neighbouring cells for cell re-selection before t-Service.

- Location based cell re-selection: UE measures the neighbour cells for cell re-selection when the distance between UE and reference location is greater than a distance threshold.

NTN-NTN cell reselection for earth moving cell

Figure 1. Reference location update

As discussed in Rel-17 NTN and for WID of Rel-18, timing and location based cell reselection for earth fixed cell can be considered as starting point. In Rel-17 NTN, timing and location based cell reselection for earth moving cell was not agreed due to satellite moving. In other words, in Figure 1, if the reference location is fixed in time [T1, T2] like Location based cell re-selection for earth fixed cell, UE 2 may choose not to perform the measurement of NR inter-frequency because the distance between UE 2 and the reference location is smaller than the distance threshold. For this reason, we think that the reference location needs to be updated for the earth moving cell. i.e., UE applies different reference locations at different times.

Proposal 1: Introduce reference location update scheme for cell reselection enhancements for earth moving cell

Figure 2. UlSyncValdityDuration

We think that timer based reference location update could be applied easily. In Figure 2, the parameter (UlSyncValidityDuration) is introduced to update parameters related to the UL Synchronization (TA Common, Ephemeris) in Rel-17 NTN. UE starts the UlSyncValidityDuration from the Epochtime (SFN & Subframe) that is the uplink time synchronization reference point. UE should attempt to re-acquire SIB19 (NTN specific SIB) before the end of the UlSyncValidityDuration. Like this, the UlSyncValidityDuration can be re-used to update the reference location which is broadcasted by SIB19. Also, a new timer (e.g., Reference location update timer) can be considered to update the reference location.

We also support to provide reference location list based scheme. Earth moving cell provides the multiple reference locations and its time information (Epoch time) to the UE. In this way, the UE can clearly know when to update the reference location and no additional signalling is required. Table 1 shows an example of a reference location list. The epoch time can be expressed SFN & Subframe number as in Rel-17 NTN.

Table 1. Example of reference location list

| Epoch time (SFN & Subframe number) | Reference location (Ellipsoid-point) |
| --- | --- |
| 2022-06-29 02:00:00.000 | Reference location 1 |
| 2022-06-29 02:05:00.000 | Reference location 2 |
| 2022-06-29 02:10:00.000 | Reference location 3 |
| 2022-06-29 02:15:00.000 | Reference location 4 |
| 2022-06-29 02:20:00.000 | Reference location 5 |
| 2022-06-29 02:25:00.000 | Reference location 6 |
| 2022-06-29 02:30:00.000 | Reference location 7 |

Proposal 2: Introduce reference location update scheme one of the following options:

- Option 1: Timer based reference location update, reuse the UlSyncValidityDuration or define a new timer (e.g., Reference location update timer)

- Option 2: Multiple reference locations and its time information of Earth-moving cell

Conclusion

In this contribution, we discussed cell re-selection enhancement. According to discussion in section 2, we have the following proposals:

Proposal 1: Introduce reference location update scheme for cell reselection enhancements for earth moving cell

Proposal 2: Introduce reference location update scheme one of the following options:

- Option 1: Timer based reference location update: reuse the UlSyncValidityDuration or define a new timer (e.g., Reference location update timer)

- Option 2: Multiple reference locations and its time information of Earth-moving cell

Reference

RAN2#119e Meeting report

RAN2#119bis-e Meeting report