3GPP TSG-RAN WG2 Meeting #121                                                   R2-2301020

Athens, February 27 – March 3, 2023

Source:	Qualcomm Incorporated

Title:	Further discussion on multi-path relay for Scenario 1 and Scenario 2

Agenda Item:	8.9 Enhanced NR Sidelink Relay

Document for:	Discussion and Decision

# Introduction & Background

According to WID [1] and RAN2 agenda for REl-18 Relay enhancement, and this contribution discusses the open issues for scenario 1 and scenario 2.

- Study the benefit and potential solutions for multi-path support to enhance reliability and throughput (e.g., by switching among or utilizing the multiple paths simultaneously) in the following scenarios [RAN2, RAN3]:

  - A UE is connected to the same gNB using one direct path and one indirect path via 1) Layer-2 UE-to-Network relay, or 2) via another UE (where the UE-UE inter-connection is assumed to be ideal), where the solutions for 1) are to be reused for 2) without precluding the possibility of excluding a part of the solutions which is unnecessary for the operation for 2).

Note 3A: Study on the benefit and potential solutions are to be completed in RAN#98 which will decide whether/how to start the normative work.

Note 3B: UE-to-Network relay in scenario 1 reuses the Rel-17 solution as the baseline.

Note 3C: Support of Layer-3 UE-to-Network relay in multi-path scenario is assumed to have no RAN impact and the work and solutions are subject to SA2 to progress.

# Discussion

## 2.1 Basic control plane issues for scenario 1

##### 2.1.2 Basic control plane procedures

This section discusses basic control plane procedures as shown in the following Table-1.

Table-1: Control plane procedures

| Procedures | Current mechanism | Clarification or Enhancement |
| --- | --- | --- |
| RRC inactive state | For camped cell in Inactive state - PCell or indicated cell in RRCRelease message in response to an RRCResumeRequest or an RRCResumeRequest1 | Following current mechanism, no further clarification or enhancement. |
| SI acquisition in CONNECTED state | For PCell SI, - The SI can be transmitted to the UE using RRCReconfiguration or UE acquires the SIs which depends on whether an active BWP with common search space configured to the UE. For SCell or PSCell SI, - In DCCA, the SIs of SCells and PSCell are transmitted to the UE using RRCReconfiguration message. - In Rel-17, The Remote UE can but is not required to obtain SI over Uu interface. | For the cells on non-anchor path, since the Remote UE is not required to obtain SI over Uu interface, then it should reuse DCCA-like mechanism, i.e. the SIs of the cells on non-anchor path are transmitted to the UE using RRCReconfiguration message in case of SI(s) update or the SI(s) are not valid in the UE. |
| RRC re-establishment | UE performs RRC re-establishment on PCell according to clause 5.3.7 in TS 38.331 | Following current specification that UE perform RRC re-eatablishment procedure on PCell according to clause 5.3.7 in TS 38.331 with MP relay context released. |
| SRB configuration | SRB1 and SRB2 or the primary RLC entity for split bearers are configured on MCG link which PCell is located on. | Following existing configuration that SRB1 and SRB2 are configured on the anchor path (if introduced) i.e., the path PCell is located on. |

Based on the analyses in the Table-1, it is proposed to clarify the control plane procedures.

Proposal 1: For MP relay, the control plane procedures are clarified as following.

- The SIs of the cells other than PCell are transmitted to the UE using RRCReconfiguration message.

- UE performs RRC re-establishment procedure on PCell according to clause 5.3.7 in TS 38.331 with MP relay context released.

- Inactive state, Remote UE camps on PCell or indicated cell in RRCRelease message in response to an RRCResumeRequest or an RRCResumeRequest1.

Proposal 2: SRB1 and SRB2 are configured on direct path.

Similar with current DC, SRB1 can be configured as duplication and non-duplication case. The non-duplication case can be used for UE to send MCG failure indication over SCG bearer. For SRB2, compared to existing scenario, there is no motivation to be used for non-duplication case.

Proposal 3: Split SRB1 can be configured as duplication and non-duplication case as defined in existing NR-DC; split SRB2 can only be used for duplication.

2.2 User plane issues for scenario 1

##### 2.2.1 MAC entity

For user plane, there is one open issue is how to configure MAC entity for MP relay. There are two potential options to be considered.

Option 1: Two separate MAC entities configured for direct path and indirect path.

This option borrows existing DC framework that there separate MAC entities configured for MCG and SCG. For MP relay, one Uu MAC entity is configured for direct path, and one SL MAC entity is configured for indirect path.

Option 2: One MAC entity is configured for both direct path and indirect path

This option is similar with existing Rel-16 mechanism that the SL MAC functions and Uu MAC functions share the same MAC entity.

Considering to reuse current mechanism defined in MAC, one MAC entity is applied for both direct path and indirect path.

Proposal 4: One MAC entity is applied for both direct path and indirect path.

2.3 Mobility-related procedure for scenario 1

##### 2.3.1 MP path addition

In the last RAN2 meeting, it was agreed to support both of following A and B.

A.	The remote UE operating only on the direct path adds the indirect path under the same gNB;

B.	The remote UE operating only on the indirect path adds the direct path under the same gNB;

Therefore, to support the agreed scenarios, some issues need to be discussed for MP path addition, e.g,

- How to ensure the direct path and the indirect path on the same gNB.

- Based on what information the gNB determines to configure MP relay.

- How to trigger RRC_IDLE/RRC_INACTIVE target relay UE to setup or resume RRC connection.

Figure 2: Indirect MP path addition-Relay UE in Connected state                         Figure 3: Direct MP path addition

As show in the call flow, the current Event Y2 can be reused to trigger indirect path addition. When the Remote UE discovers a Relay UE and the radio quality on sidelink satisfies Event Y2, the Remote UE can send measurement to the gNB, including the candidate Relays and serving cells. The gNB will select the target Relay which serving cell is in the same gNB as Remote UE Uu cell. The gNB will configure the indirect path as secondary path using existing relay configuration mechanism.

Similarly, the existing Event A4 can be reused for direct path addition and reuse existing Uu reconfiguration procedure; and it should be gNB to select the target cell which is in the same gNB as the Relay UE’s serving cell.

Proposal 5: gNB can add the indirect/direct path based on the measurement report from the Remote UE triggered by existing event, e.g. Event Y2 for indirect path addition or Event A4 for direct path addition.

Proposal 6: It is up to gNB to select the target Relay UE whose serving cell is in the same gNB in case of indirect path addition; gNB selects the target cell which is in the same gNB as the Relay UE’s serving cell.

On the issue that how to trigger the Relay UE to enter CONNECTED state, RAN2 will down-select the options.

Agreement:

RAN2 will downselect the solution for triggering IDLE/INACTIVE relay UE to enter CONNECTED state from:

-Option 1 (SL-RLC or UP-based approach (excluding SL-RLC1)),

-Option 3 (PC5-RRC approach)

-Option 4( RRCReconfigurationComplete-based approach),

Discovery/PC5-S-based solution can be further discussed if initiated from SA2.

For Option 1, it is unclear how it could work without PC5-RRC connection, and which SL-RLC should be reserved, and may change user plane format.

For Option 4, it requires split SRB1 always configured or separate SRB to be configured for RRCReconfigurationComplete transmitted on indirect path.

The Option 3 is easy way to add and indication in PC5-RRC message e.g. RemoteUEInformationSidelink message.

Proposal 7: Add an indication in RemoteUEInformationSidelink message to trigger the IDLE/Inactive target Relay UE to setup/resume RRC connection.

##### 2.3.2 MP path removal

Figure 4: Indirect path removal

Similar with MP path addition, the path removal can be based on the measurement report from the Remote UE and the Relay UE as shown in Figure 4. The existing Event X2 (for indirect path removal) and Event A2 (for direct path removal) evaluated by Remote UE and be reused to indicate the current sidelink on the indirect path or the Uu link on the direct is getting worse, and the gNB can determine to remove the indirect path or the direct path. Similarly, the indirect path can be removed based on the measurement report from Relay UE triggered by existing events, e.g. Event A2 in case that all of the serving cells are getting worse.

Proposal 8: gNB can remove the indirect or direct path based on the measurement report from the Remote UE triggered by existing events, e.g. Event X2 for indirect path removal or Event A2 for direct path removal.

##### 2.3.3 MP path change

Case 1: Indirect path change

Indirect path change is targeting to the scenario that the serving Relay UE changes but the Remote UE’s PCell on direct path is not changed. RAN2 discussed this scenario in last RAN2 meeting and achieve the following agreement, but it still FFS if it can be supported via separate release+add or a single path switch procedure.

G.	The remote UE operating in multi-path changes to a new relay UE for the indirect path while keeping the direct path under the same gNB.  FFS if this case would be supported via separate release-and-add (A+C in separate reconfigurations) or a single switch procedure (e.g. similar to i2i service continuity).

The indirect-to-indirect path service continuity is being discussed, and the indirect path change procedure for MP realy should be similar with indirect-to-indirect path switching. The differenct issue between the two scenarios is how to trigger the target IDLE/Inactive Relay UE to setup/resume RRC connection, which is same as the issue in indirect path addition procedure.

Proposal 9: Service continuity on indirect path to indirect path for intra-gNB can be used as baseline for indirect path changes during MP relay operation with the same issue addressed in indirect path addition on how to trigger IDLE/Inactive Relay UE connection setup or resume.

Case 2: Direct path change

Indirect path change is targeting to the scenario that Remote UE’s Special Cell changes on direct path but the Remote UE’s serving Relay UE and corresponding PCell is not changed, and RAN2 agreed to support this scenario. Still it is FFS if a single procedure would be supported.

E.	The remote UE operating in multi-path changes the direct path to a different cell of the same gNB while using the serving relay UE for the indirect path under the same gNB.

FFS if a single procedure for this case would be supported.

The existing PCell or PSCell change within the same gNB can be supported using a single procedure, i.e. intra-gNB handover with existing SCell removal and new SCell addition in one RRCReconfiguration message.

For the direct path change procedure in MP relay operation, it should be similar with the PCell or PSCell change procedure and can be achieved by intra-gNB handover with SCell removal and addition in one RRCReonfiguration message.

Proposal 10: Exiting PCell change procedure on direct path for intra-gNB can be reused as baseline for direct path change in MP relay operation, i.e. achieved by intra-gNB handover with Scell removal and addition in one RRCReconfiguration message.

2.4 Scenario 2 specific

One issue need to be discussed for scenario 2 is which UE should be used to identify the Remote UE. In scenario 1, Layer-2 ID is used to identify Remote UE, and in order to keep commonality with Scenario 1, Layer-2 can also be used to identify Remote UE in Scenario 2. And it is left to Remote UE and Relay UE implementation to generate and exchange Layer-2 ID

Proposal 11: Layer-2 ID is used to identify the Remote UE and Relay UE for scenario 2 and it is Remote UE and Relay UE implementation to generate and exchange Layer-2 ID.

About how to configure 1:1 mapping between the E2E bearer and RLC Channel in the Relay UE, the existing sl-L2RelayUE-Config-r17 can be reused as following. The gNB only needs to configure the yellow highlight IEs; for the unnecessary mandatory IE in green highlight, the Relay UE can ignore it.

–	SL-L2RelayUE-Config

The IE SL-L2RelayUE-Config is used to configure L2 U2N relay operation related configurations used by L2 U2N Relay UE, e.g. SRAP-Config.

SL-L2RelayUE-Config information element

-- ASN1START

-- TAG-SL-L2RELAYUE-CONFIG-START

SL-L2RelayUE-Config-r17 ::=        SEQUENCE {

sl-RemoteUE-ToAddModList-r17       SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-RemoteUE-ToAddMod-r17    OPTIONAL,    -- Need N

sl-RemoteUE-ToReleaseList-r17      SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-DestinationIdentity-r16  OPTIONAL,    -- Need N

...

}

SL-RemoteUE-ToAddMod-r17 ::=       SEQUENCE {

sl-L2IdentityRemote-r17            SL-DestinationIdentity-r16,

sl-SRAP-ConfigRelay-r17           SL-SRAP-Config-r17                                                    OPTIONAL,    -- Need M

...

}

-- TAG-SL-L2RELAYUE-CONFIG-STOP

-- ASN1STOP

–	SL-SRAP-Config

The IE SL-SRAP-Config is used to set the configurable SRAP parameters used by L2 U2N Relay UE and L2 U2N Remote UE as specified in TS 38.351 [66].

SL-SRAP-Config information element

-- ASN1START

-- TAG-SL-SRAP-CONFIG-START

SL-SRAP-Config-r17 ::=                  SEQUENCE {

sl-LocalIdentity-r17                    INTEGER (0..255)                                                 OPTIONAL, -- Need M

sl-MappingToAddModList-r17              SEQUENCE (SIZE (1..maxLC-ID)) OF SL-MappingToAddMod-r17          OPTIONAL, -- Need N

sl-MappingToReleaseList-r17             SEQUENCE (SIZE (1..maxLC-ID)) OF SL-RemoteUE-RB-Identity-r17     OPTIONAL, -- Need N

...

}

SL-MappingToAddMod-r17 ::=              SEQUENCE {

sl-RemoteUE-RB-Identity-r17             SL-RemoteUE-RB-Identity-r17,

sl-EgressRLC-ChannelUu-r17              Uu-RelayRLC-ChannelID-r17                                        OPTIONAL, -- Cond L2RelayUE

sl-EgressRLC-ChannelPC5-r17             SL-RLC-ChannelID-r17                                             OPTIONAL, -- Need N

...

}

SL-RemoteUE-RB-Identity-r17 ::=         CHOICE {

srb-Identity-r17                        INTEGER (0..3),

drb-Identity-r17                        DRB-Identity,

...

}

-- TAG-SL-SRAP-CONFIG-STOP

-- ASN1STOP

Proposal 12: Reuse existing IE sl-L2RelayUE-Config-r17 to configure the mapping of E2E bearer and Uu RLC Channel for Relay UE, with the following restrictions:

- gNB only provides the IE configuration: sl-RemoteUE-RB-Identity-r17, Uu-RelayRLC-ChannelID-r17

- Relay UE ignores the IE sl-L2IdentityRemote-r17 if received

For indirect path addition and removal, the procedure in scenario 1 can be reused. The gNB can add or remove the indirect path based on the candidate Relay UE information received from the Remote in MeasurementReport. And it is left to Remote UE implementation to determine the candidate Relay UEs.

Proposal 13: gNB can add or release the indirect path based on the candidate Relay UE information received from the Remote on MeasurementReport. And it is left to Remote UE implementation to determine the candidate Relay UEs.

Since SRAP is not used for scenario 2, and the gNB does not need to provide any configuration for ideal link, then the gNB needs to know the UE-UE link type info. Remote UE can indicate the UE-UE link type info, e.g. PC5 or ideal link in MeasurementReport message. Since the SL-MeasResult-r16 is mandatory IE, and ideal link measurement is out of 3GPP scope, then the gNB should ignore this IE.

SL-MeasResultRelay-r17 ::=                      SEQUENCE {

cellIdentity-r17                                CellAccessRelatedInfo,

sl-RelayUE-Identity-r17                         SL-SourceIdentity-r17,

sl-MeasResult-r17                               SL-MeasResult-r16,

...

}

Proposal 14: Remote UE should indicate the UE-UE link type, i.e. ideal link in MeasurementReport for gNB not provide configuration on ideal link.

# Conclusion

This contribution discusses the aspects of architecture, bearer management, path management for multi-path operation, and provides the proposals.

For basic control plane procedures,

Proposal 1: For MP relay, the control plane procedures are clarified as following.

- The SIs of the cells other than PCell are transmitted to the UE using RRCReconfiguration message.

- UE performs RRC re-establishment procedure on PCell according to clause 5.3.7 in TS 38.331 with MP relay context released.

- Inactive state, Remote UE camps on PCell or indicated cell in RRCRelease message in response to an RRCResumeRequest or an RRCResumeRequest1.

Proposal 2: SRB1 and SRB2 are configured on direct path.

Proposal 3: Split SRB1 can be configured as duplication and non-duplication case as defined in existing NR-DC; split SRB2 can only be used for duplication.

For user plane issues,

Proposal 4: One MAC entity is applied for both direct path and indirect path.

For mobility-related procedure,

Proposal 5: gNB can add the indirect/direct path based on the measurement report from the Remote UE triggered by existing event, e.g. Event Y2 for indirect path addition or Event A4 for direct path addition.

Proposal 6: It is up to gNB to select the target Relay UE whose serving cell is in the same gNB in case of indirect path addition; gNB selects the target cell which is in the same gNB as the Relay UE’s serving cell.

Proposal 7: Add an indication in RemoteUEInformationSidelink message to trigger the IDLE/Inactive target Relay UE to setup/resume RRC connection.

Proposal 8: gNB can remove the indirect or direct path based on the measurement report from the Remote UE triggered by existing events, e.g. Event X2 for indirect path removal or Event A2 for direct path removal.

Proposal 9: Service continuity on indirect path to indirect path for intra-gNB can be used as baseline for indirect path changes during MP relay operation with the same issue addressed in indirect path addition on how to trigger IDLE/Inactive Relay UE connection setup or resume.

Proposal 10: Exiting PCell change procedure on direct path for intra-gNB can be reused as baseline for direct path change in MP relay operation, i.e. achieved by intra-gNB handover with SCell removal and addition in one RRCReconfiguration message.

For scenario 2 specific

Proposal 11: Layer-2 ID is used to identify the Remote UE and Relay UE for scenario 2 and it is Remote UE and Relay UE implementation to generate and exchange Layer-2 ID.

Proposal 12: Reuse existing IE sl-L2RelayUE-Config-r17 to configure the mapping of E2E bearer and Uu RLC Channel for Relay UE, with the following restrictions:

- gNB only provides the IE configuration: sl-RemoteUE-RB-Identity-r17, Uu-RelayRLC-ChannelID-r17

- Relay UE ignores the IE sl-L2IdentityRemote-r17 if received

Proposal 13: gNB can add or release the indirect path based on the candidate Relay UE information received from the Remote on MeasurementReport. And it is left to Remote UE implementation to determine the candidate Relay UEs.

Proposal 14: Remote UE should indicate the UE-UE link type, i.e. ideal link in MeasurementReport for gNB not provide configuration on ideal link.

# Reference

[1]    RP-221262  New WID on NR sidelink relay enhancements

[2]    R2-2211101 RAN2#119bis-e Meeting Report	MCC