3GPP TSG-RAN WG2 Meeting #121	R2-2301025

Athens, Greece, 27th February- 3rd March, 2023

Agenda Item:	8.1.3

Source: 	Fujitsu

Title:	Discussion on NCR-Fwd’s behaviours in NCR-MT RRC_INACTIVE

Document for:	Discussion and Decision

- Introduction

In the RAN2#120 meeting, the following agreements were made [1].

Agreement: - On NCR-Fwd ON/OFF: - When NCR-MT is in RRC_CONNECTED mode, the NCR-Fwd can be ON or OFF following the side control information received from the gNB. - After NCR-MT enters RRC_INACTIVE mode, the NCR-Fwd can be ON or OFF following the last configuration received from the gNB. - Release to RRC-IDLE is FFS. - gNB cell that NCR-Fwd is forwarding is the same cell the NCR-MT is connected to. Whether the NCR-Fwd can forward other cells is up to implementation - NCR-MT mandatorily support cell reselection and RRM measurements in RRC_IDLE and RRC_INACTIVE.

In this document, we discuss how to keep the NCR-Fwd’s forwarding after NCR-MT enters RRC_INACTIVE under network’s control to improve performance and avoid unpredictable inferences.

- Discussions

## 2.1	Forwarded cell of NCR-Fwd

In the last RAN2 meeting, the following agreement was made for NCR-Fwd ON/OFF. It was agreed that NCR-Fwd can be ON or OFF following the last configuration received from the gNB after NCR-MT enters RRC_INACTIVE.

Agreement: - On NCR-Fwd ON/OFF: - When NCR-MT is in RRC_CONNECTED mode, the NCR-Fwd can be ON or OFF following the side control information received from the gNB. - After NCR-MT enters RRC_INACTIVE mode, the NCR-Fwd can be ON or OFF following the last configuration received from the gNB. - Release to RRC-IDLE is FFS.

Meanwhile, the following was made for clarification of the cell forwarded by the NCR-Fwd.

Agreement: - gNB cell that NCR-Fwd is forwarding is the same cell the NCR-MT is connected to. Whether the NCR-Fwd can forward other cells is up to implementation

However, for the case of NCR-Fwd forwarding when NCR-MT is in RRC_INACTIVE, because the NCR-MT is not connected to any cell (it just camps on a cell as in the legacy), it is unclear whether and how the latter agreement is inapplicable for this case. Therefore, some further clarification is needed.

In our view, 3 options can help to fix this issue:

- Option 1: The gNB cell that NCR-Fwd is forwarding is the same cell the NCR-MT is connected to before entering RRC_INACTIVE.

- Option 2: The gNB cell that NCR-Fwd is forwarding is the same cell the NCR-MT camps on.

- Option 3: The NCR-Fwd can forward any cells up to implementation.

Both Option 1 and Option 2 seems following the spirit of the agreement. Comparing them, Option 1 is preferred, since the cell the NCR-MT camps on might be unpredictable for the network if Option 2 is adopted. As for Option 3, it should be precluded since it may make the network totally lose control of the NCR-Fwd.

Observation 1: For the case where NCR-Fwd is forwarding after NCR-MT enters RRC_INACTIVE, it is not clear which cell NCR-Fwd is forwarding because the NCR-MT in RRC_INACTIVE is not connected to any cell.

Proposal 1a: For the case where the NCR-Fwd is forwarding after NCR-MT enters RRC_INACTIVE, RAN2 considers the following 3 options to clarify the cell NCR-Fwd is forwarding when the NCR-MT is in RRC_INACTIVE.

- Option 1: The gNB cell that NCR-Fwd is forwarding is the same cell the NCR-MT is connected to before entering RRC_INACTIVE.

- Option 2: The gNB cell that NCR-Fwd is forwarding is the same cell the NCR-MT camps on.

- Option 3: The NCR-Fwd can forward any cell(s) up to implementation.

Proposal 1b: If the NCR-Fwd is forwarding after the NCR-MT enters RRC_INACTIVE, the gNB cell that NCR-Fwd is forwarding is the same cell the NCR-MT is connected to before entering RRC_INACTIVE. (Adopt option 1 given in Proposal 1a)

## 2.2	Control of NCR-Fwd ON-OFF

The 2nd issue is about how the network knows whether the NCR-Fwd is ON or OFF during NCR-MT in RRC_INACTIVE. In the agreement, it says “NCR-Fwd can be ON or OFF following the last configuration received from the gNB”. However, it is not clear what the configuration is. There are several possible understandings on the exact meaning of the ‘configuration’. Hence further clarification/discussion and conclusion is necessary to clarify that how could the network control NCR-Fwd’s ON-OFF during NCR-MT in RRC_INACTIVE.

To facilitate the discussion, we summarize RAN1 conclusions/agreements which are relevant to the NCR-Fwd ON/OFF (more details can be found in the Appendix):

- The “ON” state is implicitly indicated by the beam indication (for access link). The NCR-Fwd is expected to be ON over the time domain resources indicated by the beam indication.

- Support both periodic beam indication by RRC signalling and aperiodic beam indication by DCI.

In our understanding, one reasonable assumption is that NCR-MT in RRC_INACTIVE cannot receive signalling other than system information and paging. This means the aperiodic beam indication by DCI cannot be used during NCR-MT in RRC_INACTIVE.

Therefore, only the periodic beam indications received before NCR-MT enters RRC_INACTIVE can be used to support NCR-Fwd’s forwarding during NCR-MT in RRC_INACTIVE, and the most straight forward manner is:

- When the NCR-MT is in RRC_INACTIVE, the NCR-Fwd is ON over the indicated time domain resources associated with corresponding beam(s) indicated by the periodic beam indications for access link unless the NCR-Fwd is indicated to OFF before NCR-MT in RRCINACTIVE.

  - The periodic beam indications are received by the NCR-MT before the NCR-MT entering the RRC_INACTIVE state.

Another open issue is how to support the NCR-Fwd OFF when the NCR-MT is in RRC_INACTIVE. The problem is there is no existing solution/conclusion which can directly be reused to support the conclusion we made in the last meeting (i.e. the NCR-Fwd can be ON or OFF following the last configuration received from the gNB). RAN1 has not concluded anything about how to indicate NCR-Fwd OFF yet. Moreover, we can also hardly expect an obvious progress in the relevant discussion in this parallel RAN1 meeting because the previous discussion on OFF indication was very struggling.

Considering that this is the last RAN2 meeting for NCR WI and we still have a lot of open issues which must be concluded in this meeting, we suggest RAN2 directly concludes a simple solution to support the NCR-Fwd OFF when the NCR-MT is in RRC-INACTIVE. For example, the following two options can be considered:

- Option 1: Support release of periodic beam indications for access link.

  - If all periodic beam indications for access link are released before reception of the RRC Release message transiting the NCR-MT to RRC_INACTIVE, the NCR-Fwd is OFF during NCR-MT in RRC_INACTIVE.

  - Otherwise, the NCR-Fwd is ON over the indicated time domain resources associated with corresponding beam(s)) indicated by the periodic beam indications for access link.

- Option 2: Introduce a new RRC signalling (e.g., in RRC Release message or other messages) dedicated for ON-OFF indication to control NCR-Fwd’s ON-OFF when NCR-MT enters RRC_INACTIVE.

  - If the new RRC signalling indicates the NCR-Fwd to be OFF, the NCR-Fwd is OFF during NCR-MT in RRC_INACTIVE.

  - Otherwise, the NCR-Fwd is ON over the indicated time domain resources associated with corresponding beam(s)) indicated by the periodic beam indications for access link.

Observation 2:  Further discussion on how to support NCR-Fwd ON/OFF during NCR-MT in RRC_INACTIVE is necessary.

- The ‘configuration’ in the agreement is ambiguous, i.e., “NCR-Fwd can be ON or OFF following the last configuration received from the gNB”.

Proposal 2: RAN2 further discusses how the network controls NCR-Fwd’s ON-OFF during NCR-MT in RRC_INACTIVE.

Proposal 3: Consider the following 2 options to support NCR-Fwd ON/OFF during NCR-MT in RRC_INACTIVE.

- Option 1: Support release of periodic beam indications for access link

  - If all periodic beam indications for access link are released before reception of the RRC Release message transiting the NCR-MT to RRC_INACTIVE, the NCR-Fwd is OFF during NCR-MT in RRC_INACTIVE.

  - Otherwise, the NCR-Fwd can be ON over the indicated time domain resources associated with corresponding beam(s)) indicated by the periodic beam indications for access link.

- Option 2: Introduce a new RRC signalling (in RRC Release message or other messages) dedicated for ON-OFF indication to control NCR-Fwd’s ON-OFF when NCR-MT enters RRC_INACTIVE.

  - If the new RRC signalling indicates the NCR-Fwd to be OFF, the NCR-Fwd is OFF during NCR-MT in RRC_INACTIVE.

  - Otherwise, the NCR-Fwd can be ON over the indicated time domain resources associated with corresponding beam(s)) indicated by the periodic beam indications for access link.

## 2.3	NCR-Fwd’s behaviour upon RAN paging

Following legacy UEs’ behaviour, NCR-MT monitors paging after entering RRC_INACTIVE. RAN paging is used for triggering transition from RRC_INACTIVE to RRC_CONNECTED.

In our view, if the NCR-Fwd is configured to forward when NCR-MT enters RRC_INACTIVE, when the NCR-Fwd’s forwarding based on the current configuration (e.g. backhaul link beam and access link beam configuration) causes severe interference to other devices and/or the performance is not good, the network may send a RAN paging to the NCR-MT, so that it can reconfigure the NCR-Fwd. And that is a typical motivation of network sending a RAN paging for NCR-MT. Therefore, to reduce interference, the NCR-Fwd is expected to be OFF upon NCR-MT’s reception of a RAN paging, until the NCR-MT transits to RRC_CONNECTED and NCR-Fwd is ON according to a new configuration received in RRC_CONNECED.

Based on the discussions above about paging for NCR-MT in RRC_INACTIVE, we have the following proposal.

Proposal 4: If the NCR-Fwd is forwarding after the NCR-MT enters RRC_INACTIVE, upon the NCR-MT’s reception of a RAN paging,

- The NCR-MT’s behaviour is same as the legacy UE, and,

- The NCR-Fwd is OFF, until the NCR-MT transits to RRC_CONNECTED and NCR-Fwd is ON according to a new configuration received in RRC_CONNECED.

- Conclusion

Forwarded cell of NCR-Fwd

Observation 1: For the case where NCR-Fwd is forwarding after NCR-MT enters RRC_INACTIVE, it is not clear which cell NCR-Fwd is forwarding because the NCR-MT in RRC_INACTIVE is not connected to any cell.

Proposal 1a: For the case where the NCR-Fwd is forwarding after NCR-MT enters RRC_INACTIVE, RAN2 considers the following 3 options to clarify the cell NCR-Fwd is forwarding when the NCR-MT is in RRC_INACTIVE.

- Option 1: The gNB cell that NCR-Fwd is forwarding is the same cell the NCR-MT is connected to before entering RRC_INACTIVE.

- Option 2: The gNB cell that NCR-Fwd is forwarding is the same cell the NCR-MT camps on.

- Option 3: The NCR-Fwd can forward any cell(s) up to implementation.

Proposal 1b: If the NCR-Fwd is forwarding after the NCR-MT enters RRC_INACTIVE, the gNB cell that NCR-Fwd is forwarding is the same cell the NCR-MT is connected to before entering RRC_INACTIVE. (Adopt option 1 given in Proposal 1a)

Control of NCR-Fwd ON-OFF

Observation 2:  Further discussion on how to support NCR-Fwd ON/OFF during NCR-MT in RRC_INACTIVE is necessary.

- The ‘configuration’ in the agreement is ambiguous, i.e., “NCR-Fwd can be ON or OFF following the last configuration received from the gNB”.

Proposal 2: RAN2 further discusses how the network controls NCR-Fwd’s ON-OFF during NCR-MT in RRC_INACTIVE.

Proposal 3: Consider the following 2 options to support NCR-Fwd ON/OFF during NCR-MT in RRC_INACTIVE.

- Option 1: Support release of periodic beam indications for access link

  - If all periodic beam indications for access link are released before reception of the RRC Release message transiting the NCR-MT to RRC_INACTIVE, the NCR-Fwd is OFF during NCR-MT in RRC_INACTIVE.

  - Otherwise, the NCR-Fwd can be ON over the indicated time domain resources associated with corresponding beam(s)) indicated by the periodic beam indications for access link.

- Option 2: Introduce a new RRC signalling (in RRC Release message or other messages) dedicated for ON-OFF indication to control NCR-Fwd’s ON-OFF when NCR-MT enters RRC_INACTIVE.

  - If the new RRC signalling indicates the NCR-Fwd to be OFF, the NCR-Fwd is OFF during NCR-MT in RRC_INACTIVE.

  - Otherwise, the NCR-Fwd can be ON over the indicated time domain resources associated with corresponding beam(s)) indicated by the periodic beam indications for access link.

NCR-Fwd’s behaviour upon RAN paging

Proposal 4: If the NCR-Fwd is forwarding after the NCR-MT enters RRC_INACTIVE, upon the NCR-MT’s reception of a RAN paging,

- The NCR-MT’s behaviour is same as the legacy UE, and,

- The NCR-Fwd is OFF, until the NCR-MT transits to RRC_CONNECTED and NCR-Fwd is ON according to a new configuration received in RRC_CONNECED.

# References

- RAN2#120, " Report from NC Repeater breakout session"

# Appendix: RAN1 agreements

Agreement For FR2, the “ON” state of NCR-Fwd is indicated: - Alt-2: Implicit indication via the beam indication (i.e., if there is beam indication, the NCR is assumed to be ON over the indicated time domain resource associated with corresponding beam(s)) Agreement For FR1, the “ON” state of NCR-Fwd is indicated: Alt-2: Indication via the beam indication (i.e., if there is beam indication, the NCR is assumed to be ON over the indicated time domain resource associated with corresponding beam(s)) - When there is only one beam, the sole purpose of the beam indication is for indicating “ON” state of NCR-Fwd Agreement For each periodic beam indication for access link, one RRC signalling is used including the information defined by the following: - A list of X (1≤X≤Xmax) forwarding resource, each is defined as {Beam index, time resource} - FFS: The value of Xmax, other details Each time resource is defined by {Starting slot defined as the slot offset in one period, starting symbol defined by symbol offset within the slot, duration defined by the number of symbols} with dedicated field. - The periodicity is configured as part of the RRC signaling for periodic beam indication - The same periodicity is assumed for all time resource(s) in one periodic beam indication. - The reference SCS is configured as part of the RRC signaling for periodic beam indication - The same reference SCS is assumed for all time resource(s) in one periodic beam indication. Agreement For each aperiodic beam indication for access link, one DCI is used with the information defined by Option-1: - Lmax fields are used to indicate the beam information and each field refers to one beam index ; - Note: The bitwidth of this field is determined by the number of beams used for access link. - Tmax fields to indicate the time resource; - Note: A list of time resource is pre-defined by RRC signalling. The bitwidth of this field for time resource indication is determined by the length of list. - FFS: The value of Tmax - Down-select between Tmax=1 or Tmax=Lmax. - FFS: How to define the association between time indication and beam indication - Each time resource is defined by {Starting slot defined as the slot offset, starting symbol defined by symbol offset within the slot, duration defined by the number of symbols} with dedicated field.