3GPP TSG-RAN2 Meeting #121	R2-2301050

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 4908 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on handling of T317 timer during HO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN-Core |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are two issues in the handover procedure for IoT NTN: 1. In current spec, after UE triggers a handover, there is no process for T317 in source cell. Therefore, T317 in source cell may expire and such expiration of T317 may trigger UE to reacquire SIB31 during handover. This is undesirable behavior. So T317 in source cell needs to be stopped after UE triggers a handover. This process is also aligned with the process on T430 during handover in NR NTN. 2. In current spec, during handover procedure, there is no process for dedicated SIB31 of target cell, that would cause the T317 cannot be started when handover to the target cell. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. To add the process of stopping T317 timer in 5.3.5.4. 2. To add the process of handling systemInformationBlockType31Dedicated during handover in 5.3.5.4. Impact Analysis Impacted functionality: Handover in IoT NTN. Inter-operability: These changes mainly have impacts on UE side processes. NW don’t need to implement these changes. So no inter-operability issue can be seen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. If the first change is not agreed, after UE triggers a handover, T317 in source cell may expire. 2. If the second change is not agreed, UE couldn’t start T317 for target cell, that may cause incorrect UE behaviour in target cell. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of the change>

5.3.5.4	Reception of an RRCConnectionReconfiguration including the mobilityControlInfo by the UE (handover)

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	if the RRCConnectionReconfiguration is applied due to a conditional reconfiguration execution upon cell selection performed while timer T311 was running, as defined in 5.3.7.3:

2>	remove all the entries within VarConditionalReconfiguration, if any;

1>	if the UE is in (NG)EN-DC and;

1>	if the RRCConnectionReconfiguration does not include the nr-SecondaryCellGroupConfig:

2>	if the RRCConnectionReconfiguration includes the scg-State:

3>	perform SCG deactivation as specified in TS 38.331 [82], clause 5.3.5.13b;

2>	else:

3>	perform SCG activation without SN message as specified in TS 38.331 [82], clause 5.3.5.13b1;

1>	if daps-HO is not configured for any DRB:

2>	stop timer T310, if running;

2>	if timer T316 is running:

3>	stop timer T316;

3>	clear the information included in VarRLF-Report, if any;

2>	resume MCG transmission, if suspended;

1>	stop timer T312, if running;

1>	start timer T304 with the timer value set to t304, as included in the mobilityControlInfo;

1>	stop timer T370, if running;

1> stop timer T317, if running;

1>	if the carrierFreq is included:

2>	consider the target PCell to be one on the frequency indicated by the carrierFreq with a physical cell identity indicated by the targetPhysCellId;

1>	else:

2>	consider the target PCell to be one on the frequency of the source PCell with a physical cell identity indicated by the targetPhysCellId;

1>	if T309 is running:

2>	stop timer T309 for all access categories;

2>	perform the actions as specified in 5.3.16.4.

1>	start synchronising to the DL of the target PCell;

NOTE 1:	The UE should perform the handover as soon as possible following the reception of the RRC message triggering the handover, which could be before confirming successful reception (HARQ and ARQ) of this message.

1>	if BL UE or UE in CE:

2>	if sameSFN-Indication is not present in mobilityControlInfo:

3>	acquire the MasterInformationBlock in the target PCell;

1>	if makeBeforeBreak is configured:

2>	perform the remainder of this procedure including and following resetting MAC after the UE has stopped the uplink transmission/downlink reception with the source PCell;

NOTE 1a:	It is up to UE implementation when to stop the uplink transmission/ downlink reception with the source PCell to initiate re-tuning for connection to the target cell, as specified in TS 36.133 [16], if makeBeforeBreak is configured.

NOTE 1b: It is up to UE implementation when to stop the uplink transmission/ downlink reception with the source SCell(s) after receiving RRCConnectionReconfiguration message.

1>	if daps-HO is configured for any DRB:

2>	establish a MAC entity for the target PCell, with the same configuration as the MAC entity for the source PCell;

2>	for each DRB configured with daps-HO:

3>	establish the RLC entity or entities and the associated DTCH logical channel for the target PCell, with the same configurations as for the source PCell;

3>	reconfigure the PDCP entity to configure DAPS as specified in TS36.323 [8].

2>	for each DRB not configured with daps-HO:

3>	re-establish PDCP;

3>	re-establish the RLC entity and associate it, and the associated DTCH logical channel, to the target PCell;

2>	for each SRB:

3>	establish a PDCP entity for the target PCell, with the same configuration as the PDCP entity for the source PCell;

3>	establish an RLC entity and an associated DCCH logical channel for the target PCell, with the same configuration as for the source PCell;

2>	suspend the SRBs for the source PCell;

NOTE 1c:	In order to understand if a daps-HO is configured, the UE needs to check the presence of the field daps-HO within the received RadioResourceConfigDedicated IE.

NOTE 1d:	In DAPS handover, the UE may re-establish PDCP and RLC entity for a DRB not configured with daps-HO when MAC successfully completes the random access procedure. In this case, the UE suspends data transmission and reception for all DRBs not configured with daps-HO in the source PCell for the duration of the DAPS handover.

1>	else (if daps-HO is not configured):

2>	reset MCG MAC and SCG MAC, if configured;

2>	release uplinkDataCompression, if configured;

2>	re-establish PDCP for all RBs configured with pdcp-config that are established;

NOTE 2:	The handling of the radio bearers after the successful completion of the PDCP re-establishment, e.g. the re-transmission of unacknowledged PDCP SDUs (as well as the associated status reporting), the handling of the SN and the HFN, is specified in TS 36.323 [8].

NOTE 2a:	At handover the reestablishPDCP flag will be set for all RBs configured with NR PDCP in nr-RadioBearerConfig1 or nr-RadioBearerConfig2 TS 38.331 [82] which will cause the PDCP entity to be re-established also for these RBs.

2>	re-establish MCG RLC and SCG RLC, if configured, for all RBs that are established;

1>	for each SCell configured for the UE other than the PSCell:

2>	if the received RRCConnectionReconfiguration message includes sCellState for the SCell and indicates activated:

3>	configure lower layers to consider the SCell to be in activated state;

2>	else if the received RRCConnectionReconfiguration message includes sCellState for the SCell and indicates dormant:

3>	configure lower layers to consider the SCell to be in dormant state;

2>	else:

3>	configure lower layers to consider the SCell to be in deactivated state;

1>	apply the value of the newUE-Identity as the C-RNTI in the target MCG;

1>	if the RRCConnectionReconfiguration message includes the fullConfig:

2>	perform the radio configuration procedure as specified in 5.3.5.8;

1>	configure lower layers in accordance with the received radioResourceConfigCommon;

1>	if the received RRCConnectionReconfiguration message includes the rach-Skip:

2>	configure lower layers to apply the rach-Skip for the target MCG, as specified in TS 36.213 [23] and 36.321 [6];

1>	if UE supports timing advance reporting and the received radioResourceConfigCommon includes the ta-Report:

2>	instruct the associated MAC entity to trigger Timing Advance reporting;

1>	configure lower layers in accordance with any additional fields, not covered in the previous, if included in the received mobilityControlInfo;

1>	if the received RRCConnectionReconfiguration includes the sCellToReleaseList:

2>	perform SCell release as specified in 5.3.10.3a;

1>	if the received RRCConnectionReconfiguration includes the sCellGroupToReleaseList:

2>	perform SCell group release as specified in 5.3.10.3d;

1>	if the received RRCConnectionReconfiguration includes the scg-Configuration; or

1>	if the current UE configuration includes one or more split DRBs and the received RRCConnectionReconfiguration includes radioResourceConfigDedicated including drb-ToAddModList:

2>	perform SCG reconfiguration as specified in 5.3.10.10;

1>	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

2>	perform the radio resource configuration procedure as specified in 5.3.10.0;

1>	if the securityConfigHO (without suffix) is included in the RRCConnectionReconfiguration:

2>	if the keyChangeIndicator received in the securityConfigHO is set to TRUE:

3>	update the KeNB key based on the KASME key taken into use with the latest successful NAS SMC procedure, as specified in TS 33.401 [32];

2>	else:

3>	update the KeNB key based on the current KeNB or the NH, using the nextHopChainingCount value indicated in the securityConfigHO, as specified in TS 33.401 [32];

NOTE 2b:	If the UE needs to update the S-KeNB key as specified in 5.3.10.10, the UE updates the S-KeNB after updating the KeNB key.

2>	store the nextHopChainingCount value;

2>	if the securityAlgorithmConfig is included in the securityConfigHO:

3>	derive the KRRCint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

3>	if connected as an RN; or

3>	if capable of user plane integrity protection:

4>	derive the KUPint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

3>	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm, as specified in TS 33.401 [32];

2>	else:

3>	derive the KRRCint key associated with the current integrity algorithm, as specified in TS 33.401 [32];

3>	if connected as an RN; or

3>	if capable of user plane integrity protection:

4>	derive the KUPint key associated with the current integrity algorithm, as specified in TS 33.401 [32];

3>	derive the KRRCenc key and the KUPenc key associated with the current ciphering algorithm, as specified in TS 33.401 [32];

2>	configure lower layers to apply the integrity protection algorithm and the KRRCint key, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

2>	configure lower layers to apply the ciphering algorithm, the KRRCenc key and the KUPenc key, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

NOTE 2c:	For a DRB configured for DAPS HO, the new ciphering algorithm and the KUPenc key is applied for traffic exchange between the UE and the target MCG while the old ciphering algorithm and KUPenc key is applied for traffic exchange between the UE and the source MCG.

1>	else if the securityConfigHO-v1530 is included in the RRCConnectionReconfiguration:

2>	if the nas-Container is received:

3>	forward the nas-Container to upper layers;

2>	if the keyChangeIndicator-r15 is received and is set to TRUE:

3>	update the KeNB key based on the KAMF key, as specified in TS 33.501 [86];

2>	else:

3>	update the KeNB key based on the current KeNB or the NH, using the received nextHopChainingCount-r15, as specified in TS 33.501 [86];

2>	store the nextHopChainingCount-r15 value;

2>	if the securityAlgorithmConfig-r15 is received:

3>	derive the KRRCint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

3>	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm, as specified in TS 33.401 [32];

2>	else:

3>	derive the KRRCint key associated with the current integrity algorithm, as specified in TS 33.401 [32];

3>	derive the KRRCenc key and the KUPenc key associated with the current ciphering algorithm, as specified in TS 33.401 [32];

1>	if the received RRCConnectionReconfiguration includes the nr-Config and it is set to release; or

1>	if the received RRCConnectionReconfiguration includes endc-ReleaseAndAdd and it is set to TRUE:

2>	perform MR-DC release as specified in TS 38.331 [82], clause 5.3.5.10;

1>	if the received RRCConnectionReconfiguration includes the sk-Counter:

2>	perform key update procedure as specified in in TS 38.331 [82], clause 5.3.5.7;

1>	if the received RRCConnectionReconfiguration includes the nr-SecondaryCellGroupConfig:

2>	perform NR RRC Reconfiguration as specified in TS 38.331 [82], clause 5.3.5.3.

1>	if the received RRCConnectionReconfiguration includes the nr-RadioBearerConfig1:

2>	perform radio bearer configuration as specified in TS 38.331 [82], clause 5.3.5.6;

1>	if the received RRCConnectionReconfiguration includes the nr-RadioBearerConfig2:

2>	perform radio bearer configuration as specified in TS 38.331 [82], clause 5.3.5.6.

1>	if connected as an RN:

2>	configure lower layers to apply the integrity protection algorithm and the KUPint key, for current or subsequently established DRBs that are configured to apply integrity protection, if any;

1>	if the received RRCConnectionReconfiguration includes the sCellToAddModList:

2>	perform SCell addition or modification as specified in 5.3.10.3b;

1>	if the received RRCConnectionReconfiguration includes the sCellGroupToAddModList:

2>	perform SCell group addition or modification as specified in 5.3.10.3e;

1>	if the received RRCConnectionReconfiguration includes the systemInformationBlockType1Dedicated:

2>	perfom the actions upon reception of the SystemInformationBlockType1 message as specified in 5.2.2.7;

1>	if the received RRCConnectionReconfiguration includes the systemInformationBlockType31Dedicated:

2>	perform the actions upon reception of the SystemInformationBlockType31 message as specified in 5.2.2.39;

1>	perform the measurement related actions as specified in 5.5.6.1;

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	perform the measurement identity autonomous removal as specified in 5.5.2.2a;

1>	release reportProximityConfig and clear any associated proximity status reporting timer;

1>	if the RRCConnectionReconfiguration message includes the otherConfig:

2>	perform the other configuration procedure as specified in 5.3.10.9;

1>	if the RRCConnectionReconfiguration message includes the sl-DiscConfig or sl-CommConfig:

2>	perform the sidelink dedicated configuration procedure as specified in 5.3.10.15;

1>	if the RRCConnectionReconfiguration message includes wlan-OffloadInfo:

2>	perform the dedicated WLAN offload configuration procedure as specified in 5.6.12.2;

1>	if handoverWithoutWT-Change is not configured:

2>	release the LWA configuration, if configured, as described in 5.6.14.3;

1>	release the LWIP configuration, if configured, as described in 5.6.17.3;

1>	if the RRCConnectionReconfiguration message includes rclwi-Configuration:

2>	perform the WLAN traffic steering command procedure as specified in 5.6.16.2;

1>	if the RRCConnectionReconfiguration message includes lwa-Configuration:

2>	perform the LWA configuration procedure as specified in 5.6.14.2;

1>	if the RRCConnectionReconfiguration message includes lwip-Configuration:

2>	perform the LWIP reconfiguration procedure as specified in 5.6.17.2;

1>	if the RRCConnectionReconfiguration message includes the sl-V2X-ConfigDedicated or mobilityControlInfoV2X:

2>	perform the V2X sidelink communication dedicated configuration procedure as specified in 5.3.10.15a;

NOTE 2d:	In case of conditional reconfiguration the text "if the received RRCConnectionReconfiguration. . ." corresponds to applying the stored RRCConnectionReconfiguration message (according to 5.3.5.9.5).

1>	if the UE is configured to operate in EN-DC as result of this procedure, forward upperLayerIndication, as if the UE receives this field from SIB2, to upper layers, otherwise indicate upper layers absence of this field;

1>	set the content of RRCConnectionReconfigurationComplete message as follows:

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3>	include rlf-InfoAvailable;

2>	if the UE has MBSFN logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and if T330 is not running:

3>	include logMeasAvailableMBSFN;

2>	else if the UE has logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable;

3>	if Bluetooth measurement results are included in the logged measurements the UE has available:

4>	include logMeasAvailableBT;

3>	if WLAN measurement results are included in the logged measurements the UE has available:

4>	include logMeasAvailableWLAN;

2>	if the UE has connection establishment failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

3>	include connEstFailInfoAvailable;

2>	if the RRCConnectionReconfiguration message includes perCC-GapIndicationRequest:

3>	include perCC-GapIndicationList and numFreqEffective;

2>	if the frequencies are configured for reduced measurement performance:

3>	include numFreqEffectiveReduced;

2>	if the UE has flight path information available:

3>	include flightPathInfoAvailable;

2>	if the received RRCConnectionReconfiguration message included nr-SecondaryCellGroupConfig:

3>	include scg-ConfigResponseNR in accordance with TS 38.331 [82], clause 5.3.5.3;

2>	if the UE is connected to NTN:

3>	include gnss-validityDuration in accordance with the remaining time of the GNSS validity duration;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission;

1>	if MAC successfully completes the random access procedure; or

1>	if MAC indicates the successful reception of a PDCCH transmission addressed to C-RNTI and if rach-Skip is configured:

2>	stop timer T304;

2>	if daps-HO is configured for any DRB:

3>	stop timer T310 for the source PCell, if running;

3>	for each DAPS bearer trigger UL data switching, as specified in TS 36.323 [8];

2>	release rach-Skip;

2>	apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the target PCell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target PCell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target PCell;

NOTE 3:	Whenever the UE shall setup or reconfigure a configuration in accordance with a field that is received it applies the new configuration, except for the cases addressed by the above statements.

2>	if the UE is configured to provide IDC indications:

3>	if the UE has initiated the transmission of an InDeviceCoexIndication message during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo; or

3>	if the RRCConnectionReconfiguration message is applied due to a conditional reconfiguration execution and the UE has initiated transmission of an InDeviceCoexIndication message since it was configured to do so in accordance with 5.6.9.2:

4>	initiate transmission of the InDeviceCoexIndication message in accordance with 5.6.9.3;

2>	if the UE is configured to provide power preference indications, overheating assistance information, SPS assistance information, delay budget report or maximum bandwidth preference indications:

3>	if the UE has initiated the transmission of a UEAssistanceInformation message during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo; or

3>	if the RRCConnectionReconfiguration message is applied due to a conditional reconfiguration execution, and the UE has initiated transmission of a UEAssistanceInformation message for the corresponding cell group since it was configured to do so in accordance with 5.6.10.2:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.6.10.3;

2>	if SystemInformationBlockType15 is broadcast by the PCell:

3>	if the UE has initiated the transmission of a MBMSInterestIndication message during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo; or

3>	if the RRCConnectionReconfiguration message is applied due to a conditional reconfiguration execution and the UE supports MBMS reception and the UE has initiated transmission of an MBMSInterestIndication message since it was configured to do so in accordance with 5.8.5.2:

4>	ensure having a valid version of SystemInformationBlockType15 for the PCell;

4>	determine the set of MBMS frequencies of interest in accordance with 5.8.5.3;

4>	determine the set of MBMS services of interest in accordance with 5.8.5.3a;

4>	initiate transmission of the MBMSInterestIndication message in accordance with 5.8.5.4;

2>	if SystemInformationBlockType18 is broadcast by the target PCell; and the UE initiated the transmission of a SidelinkUEInformation message indicating a change of sidelink communication related parameters relevant in target PCell (i.e. change of commRxInterestedFreq or commTxResourceReq, commTxResourceReqUC if SystemInformationBlockType18 includes commTxResourceUC-ReqAllowed or commTxResourceInfoReqRelay if PCell broadcasts SystemInformationBlockType19 including discConfigRelay) during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo; or

2>	if SystemInformationBlockType19 is broadcast by the target PCell; and the UE initiated the transmission of a SidelinkUEInformation message indicating a change of sidelink discovery related parameters relevant in target PCell (i.e. change of discRxInterest or discTxResourceReq, discTxResourceReqPS if SystemInformationBlockType19 includes discConfigPS or discRxGapReq or discTxGapReq if the UE is configured with gapRequestsAllowedDedicated set to true or if the UE is not configured with gapRequestsAllowedDedicated and SystemInformationBlockType19 includes gapRequestsAllowedCommon) during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo; or

2>	if SystemInformationBlockType21 is broadcast by the target PCell; and the UE initiated the transmission of a SidelinkUEInformation message indicating a change of V2X sidelink communication related parameters relevant in target PCell (i.e. change of v2x-CommRxInterestedFreqList or v2x-CommTxResourceReq) during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo; or

2>	if the RRCConnectionReconfiguration message is applied due to a conditional reconfiguration execution, and at least one of SystemInformationBlockType18, SystemInformationBlockType19, and SystemInformationBlockType21 is broadcast by the target PCell, and the UE has initiated transmission of a SidelinkUEInformation message since it was configured to do so in accordance with 5.10.2.2:

3>	initiate transmission of the SidelinkUEInformation message in accordance with 5.10.2.3;

2>	remove all the entries within VarConditionalReconfiguration, if any;

2>	for each measId, if the associated reportConfig is condReconfigurationTriggerEUTRA:

3>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

3>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

3>	if the measObjectId is only included in a MeasIdToAddMod:

4>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

2>	the procedure ends;

NOTE 4:	The UE is not required to determine the SFN of the target PCell by acquiring system information from that cell before performing RACH access in the target PCell, except for BL UEs or UEs in CE when sameSFN-Indication is not present in mobilityControlInfo.

<End of the change>