3GPP TSG-RAN WG2 Meeting #121	R2-2301061

Athens, Greece, 27th Feb – 3th Mar 2023

Agenda item:	8.19.3

Source:	ZTE

Title:	Other aspects for further reducing UE complexity

Document for:	Discussion / Decision

Introduction

In WID RP-223544[1], UE BB bandwidth for PDSCH(unicast and broadcast) and PUSCH is further reduced and peak rate is also further reduced as below.

Complexity/cost reduction - Further reduced UE complexity in FR1 [RAN1, RAN2, RAN4] - UE BB bandwidth reduction - 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH, with 20 MHz RF bandwidth for UL and DL - The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth. - UE peak data rate reduction - Relaxation of the constraint (vLayers·Qm·f ≥ 4) for peak data rate reduction - The relaxed constraint is, e.g., 1 (instead of 4). - The parameters (vLayers, Qm, f) can be as in Rel-17 RedCap. - Both 15 kHz SCS and 30 kHz SCS are supported. - Aim to define at most one Rel-18 RedCap UE type for further UE complexity reduction. - The existing UE capability framework is used, and changes to capability signalling are specified only if necessary. By default, all UE capabilities applicable to a Rel-17 RedCap UE are applicable unless otherwise specified.

In this contribution, we will discuss issues related to RAN2 based on the WID and provide our views for these issues.

Discussions

Separate initial BWP

Regarding additional separate initial DL/UL BWP for Rel-18 eRedCap UE, it is still FFS as below.

Agreement For a cell supporting both Rel-17 and Rel-18 RedCap UEs, - The Rel-18 RedCap UEs can share the same separate initial DL/UL BWP as the Rel-17 RedCap UEs. - FFS: whether to support an additional separate initial DL/UL BWP specific to Rel-18 RedCap UEs

Rel-18 eRedCap UE can support a BWP up to 20MHz bandwidth, which will not further restrict the bandwidth of Rel-17 separate initial DL/UL BWP. Besides, if a new separate initial DL/UL BWP is introduced for Rel-18 eRedCap UE, which can increase the fragmentation of the carrier bandwidth. Hence, there is no necessary to support an additional separate initial DL/UL BWP specific to Rel-18 eRedCap UE. Then Rel-18 eRedCap UE may share/reuse the separate initial DL/UL BWP configuration of  Rel-17 RedCap UE.

Proposal 1: For Rel-18 eRedCap UE, there is no necessary to support an additional separate initial DL/UL BWP and it may share/reuse the separate initial DL/UL BWP configuration of Rel-17 RedCap UE.

Access control

In Rel-17 RedCap, some access control parameters are introduced, such as cellBarredRedCap1Rx, cellBarredRedCap2Rx and halfDuplexRedCapAllowed. Since Rel-18 eRedCap is further reduced UE complexity on top of Rel-17 RedCap, access control mechanism for Rel-17 RedCap can also be used for Rel-18 eRedCap. Compared with Rel-17 RedCap UE, the allocated resource bandwidth of PUSCH and unicast PDSCH for Rel-18 eRedCap can be restricted within 5MHz, which only needs to make resource scheduling restriction by NW. So we think there is no necessary to introduce new access control parameter for Rel-18 eRedCap UE.

Proposal 2: Access control mechanism introduced in Rel-17 RedCap can also be used for Rel-18 eRedCap UE and there is no necessary to introduce additional access control parameter.

UE peak rate

In section 4.1 of TS 38.306[2], the component  vLayers(j)⋅Qmj⋅fj  in the calculation formula of data rate is restricted, i.e. it is no smaller than 4 as below.

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

For single carrier NR SA operation, the UE shall support a data rate for the carrier that is no smaller than the data rate computed using the above formula, with J=1 CC and component vLayers(j)⋅Qmj⋅fj is no smaller than 4.

NOTE 3: As an example, the value 4 in the component above can correspond to vLayers(j)=1, Qmj= 4 and fj=1

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

In Rel-18 eRedCap, in order to decrease the peak data rate, the constraint for the above component can be relaxed. In WID RP-223544, a possible value for the relaxed constraint is 1, which will instead of 4 above. Of course, the final relaxed constraint value is determined by RAN1. Once determined, it will be reflected TS 38.306.

Proposal 3: The relaxed constraint value for the component in the data rate formula will wait for RAN1’s progress.

UE type

Since Rel-18 eRedCap UE is further reduced complexity on top of Rel-17 RedCap, for example, scheduled bandwidth for PUSCH and unicast PDSCH is restricted within 5MHz and peak rate is restricted, a new UE type should be defined. Based on the WID and Rel-17 RedCap UE definition, Rel-18 eRedCap UE type may be defined as below:

eRedCap UE is the UE with reduced capability in FR1: - 5MHz BB bandwidth for PDSCH(unicast and broadcast) and PUSCH, with 20MHz RF bandwidth for UL and DL. The maximum bandwidth is 20MHz for the other physical layer channels and signals. UE features and corresponding capabilities related to UE bandwidths wider than 20 MHz in FR1 or wider than 100 MHz in FR2 are not supported by RedCap UEs; - The supported SCS is 15kHz and 30kHz; - The maximum mandatory supported DRB number is 8; - The mandatory supported PDCP SN length is 12 bits while 18 bits being optional; - The mandatory supported RLC AM SN length is 12 bits while 18 bits being optional; - 1 DL MIMO layer if 1 Rx branch is supported, and 2 DL MIMO layers if 2 Rx branches are supported; UE features and corresponding capabilities related to more than 2 UE Rx branches or more than 2 DL MIMO layers, as well as UE features and capabilities related to more than 1 UE Tx branch or more than 1 UL MIMO layer are not supported by eRedCap UEs; - CA, MR-DC, DAPS, CPAC and IAB (i.e., the eRedCap UE is not expected to act as IAB node) related UE features and corresponding capabilities are not supported by eRedCap UEs. All other feature groups or components of the feature groups as captured in TR 38.822 [24] as well as capabilities specified in this specification remain applicable for eRedCap UEs same as non-RedCap UEs, unless indicated otherwise.

Proposal 4: A new UE type for Rel-18 eRedCap UE is defined as above.

UE capability

In Rel-17 RedCap, UE capabilities for RedCap UEs are defined. We give a summary as shown in Table 1. In addition, based on WID and RAN1’s discussion, we also point out which UE capabilities can be supported for  Rel-18 eRedCap and which cannot. Firstly, for supported UE capabilities, some updates are needed, for example, This capability is only applicable for RedCap UEs and eRedCap UEs. For UE capability supportOfRedCap-r17, since some further complexity reductions are introduced, a new UE capability should be defined for Rel-18 eRedCap UEs.

Table 1 UE capabilities for RedCap

| General parameters |  |
| --- | --- |
| supportOf16DRB-RedCap-r17 Indicates whether the RedCap UE supports 16 DRBs. This capability is only applicable for RedCap UEs and eRedCap UEs. | Support, but with update |
| supportOfRedCap-r17 Indicates that the UE is a RedCap UE with comprised of at least the following functional components: - Maximum FR1 RedCap UE bandwidth is 20 MHz; - Maximum FR2 RedCap UE bandwidth is 100 MHz; - Support of RedCap early indication based on Msg1, MsgA (if UE indicated support of twoStepRACH-r16) and Msg3 for random access; - Separate initial UL BWP for RedCap UEs; - Separate initial DL BWP for RedCap UEs; - UE-specific RRC-configured DL BWP with CD-SSB or NCD-SSB; - NCD-SSB based measurements in RRC-configured DL BWP. A RedCap UE shall set the field to supported. | Not support. A new UE capability should be defined. |
| PDCP parameters |  |
| longSN-RedCap-r17 Indicates whether the RedCap UE supports 18 bit length of PDCP sequence number. This capability is only applicable for RedCap UEs and eRedCap UEs. | Support, but with update |
| RLC parameters |  |
| am-WithLongSN-RedCap-r17 Indicates whether the RedCap UE supports AM DRB with 18 bit length of RLC sequence number. This capability is only applicable for RedCap UEs and eRedCap UEs. | Support, but with update |
| MeasAndMobParameters |  |
| rrm-RelaxationRRC-ConnectedRedCap-r17 Indicates whether UE supports Rel-17 relaxed RRM measurements in RRC_CONNECTED as specified in TS 38.331 [9]. | Support |
| BandNR parameters |  |
| bwp-WithoutCD-SSB-OrNCD-SSB-RedCap-r17 Indicates support of RRC-configured DL BWP without CD-SSB or NCD-SSB. The UE can include this field only if the UE supports supportOfRedCap-r17 or supportOfeRedCap-r18. | Support, but with update |
| halfDuplexFDD-TypeA-RedCap-r17 Indicates support of Half-duplex FDD operation (instead of full-duplex FDD operation) type A for RedCap UE. The UE can include this field only if the UE supports supportOfRedCap-r17 or supportOfeRedCap-r18. | Support, but with update |

Proposal 5: Besides supportOfRedCap-r17, other UE capabilities introduced in Rel-17 RedCap can be used for Rel-18 eRedCap with some field description updates.

For Rel-18 eRedCap UEs, since UE BB bandwidth for PDSCH and PUSCH is restricted within 5MHz and UE peak rate is also further reduced, a new UE capability should be defined. Based on WID and Rel-17 definition, we give a possible definition as Table 2. For early indication, we can wait for the final conclusion.

Table 2 Separate UE capability for Rel-18 eRedCap UE

| Definitions for parameters | Per | M | FDD-TDD DIFF |
| --- | --- | --- | --- |
| supportOfeRedCap-r187 Indicates that the UE is an eRedCap UE with comprised of at least the following functional components: - Maximum eRedCap UE BB bandwidth is 5 MHz for PDSCH(unicast and broadcast) and PUSCH, with 20MHz RF bandwidth for UL and DL; - Maximum eRedCap UE bandwidth is 20 MHz for other physical channels and signals; ; - Support of RedCap early indication based on Msg1, MsgA (if UE indicated support of twoStepRACH-r16) and Msg3 for random access; - Separate initial UL BWP for eRedCap UEs; - Separate initial DL BWP for eRedCap UEs; - UE-specific RRC-configured DL BWP with CD-SSB or NCD-SSB; - NCD-SSB based measurements in RRC-configured DL BWP. A eRedCap UE shall set the field to supported. | UE | CY | No |

Proposal 6: For Rel-18 eRedCap UEs, a new UE capability indicating whether the UE is an eRedCap UE should be defined and the final functional components will wait for RAN1 and RAN2’s progress.

SCS

In WID RP-223544[1], for Rel-18 eRedCap UEs, only 15kHz SCS and 30kHz SCS are supported for all channels and signals in FR1, so a restriction should be added in the field description of SCS configuration of BWP, CSI-RS and SRS. Taking BWP configuration as an example, according to the current spec[3], 15, 30 or 60 may be configured in FR1 for BWP. In order to avoid configuring not supported value, the restriction marked by red should be added.

Proposal 7: For Rel-18 eRedCap UEs, a restriction should be added in the field description of SCS for BWP, CSI-RS and SRS due to 60kHz SCS is not supported in FR1.

The following is an example for change on spec:

BWP ::=                             SEQUENCE {

locationAndBandwidth                INTEGER (0..37949),

subcarrierSpacing                   SubcarrierSpacing,

cyclicPrefix                        ENUMERATED { extended }            OPTIONAL    -- Need R

}

BWP field descriptions

subcarrierSpacing Subcarrier spacing to be used in this BWP for all channels and reference signals unless explicitly configured elsewhere. Corresponds to subcarrier spacing according to TS 38.211 [16], table 4.2-1. The value kHz15 corresponds to µ=0, value kHz30 corresponds to µ=1, and so on. Only the following values are applicable depending on the used frequency: FR1: 15, 30, or 60 kHz FR2-1: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz For the initial DL BWP and operation in licensed spectrum this field has the same value as the field subCarrierSpacingCommon in MIB of the same serving cell. Except for SUL, the network ensures the same subcarrier spacing is used in active DL BWP and active UL BWP within a serving cell. For the initial DL BWP and operation with shared spectrum channel access, the value of this field corresponds to the subcarrier spacing of the SSB associated to the initial DL BWP. The network does not configure 60kHz for eRedCap UEs.

Conclusions

This contribution presents some of our views on DRX enhancements for XR. We have the following proposals:

Proposal 1: For Rel-18 eRedCap UE, there is no necessary to support an additional separate initial DL/UL BWP and it may share/reuse the separate initial DL/UL BWP configuration of Rel-17 RedCap UE.

Proposal 2: Access control mechanism introduced in Rel-17 RedCap can also be used for Rel-18 eRedCap UE and there is no necessary to introduce additional access control parameter.

Proposal 3: The relaxed constraint value for the component in the data rate formula will wait for RAN1’s progress.

Proposal 4: A new UE type for Rel-18 eRedCap UE is defined as above.

Proposal 5: Besides supportOfRedCap-r17, other UE capabilities introduced in Rel-17 RedCap can be used for Rel-18 eRedCap with some updates.

Proposal 6: For Rel-18 eRedCap UEs, a new UE capability indicating whether the UE is an eRedCap UE should be defined and the final functional components will wait for RAN1 and RAN2’s progress.

Proposal 7: For Rel-18 eRedCap UEs, a restriction should be added in the field description of SCS for BWP, CSI-RS and SRS due to 60kHz SCS is not supported in FR1.

References

- RP-223544 Revised WID on Enhanced support of reduced capability NR devices.

- TS 38306 h30.

- TS 38331 h30.