3GPP TSG RAN WG2 #121									                          R2-2301063

Athens, Greece, February 27th-March 3rd, 2023

Source: 			ZTE Corporation, Sanechips

Title: 	Preventing legacy UEs camping on NES cell

Agenda item:	      8.3.4

Document for:       Discussion and Decision

- Introduction

During the NES study item[1], the possible NES techniques have been discussed, and in the NES WID[2], the following objective has been included:

- Specify mechanism(s) to prevent legacy UEs camping on cells adopting the Rel-18 NES techniques, if necessary [RAN2]

In this paper, we will discuss the necessity and feasible solutions to prevent legacy UEs camping on cells adopting the Rel-18 NES techniques, and give our proposals.

- Discussion

- Based on the NES study item[1] and the NES WID objectives[2] meeting, the following cell specific features will be supported for NES purposes, which may impact on whether to prevent legacy UEs camping on cells adopting the Rel-18 NES techniques:

- Specify SSB-less SCell operation for inter-band CA for FR1 and co-located cells

- Specify enhancement on cell DTX/DRX mechanism

- Specify the NES techniques in spatial and power domains, including

- Specify necessary enhancements on CSI and beam management related procedures including measurement and report, and signaling to enable efficient adaptation of spatial elements (e.g. antenna ports, active transceiver chains) [RAN1, RAN2]

- Specify necessary enhancements on CSI related procedures including measurement and report, and signaling to enable efficient adaptation of power offset values between PDSCH and CSI-RS [RAN1, RAN2]

- Based on the current specification, UE couldn’t search a SSB-less cell, so UE shall not select or re-select a SSB-less cell during cell selection and cell re-selection procedure.

- Observation 1: During cell selection and cell re-selection procedure, UE shall not select or re-select a SSB-less cell.

- Proposal 1: For cell selection or cell re-selection procedure, SSB-less cell is not included for the NES mode definition.

- Based on the following notes for cell DTX/DRX mechanism in NES WID[2], the legacy UE may camp in the cell configured with cell DTX/DRX.

- Note: No change for SSB transmission due to cell DTX/DRX.

- Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided.

- Observation 2: The legacy UE may camp in the cell configured with cell DTX/DRX.

- Whether legacy UE can work in the cell configured with cell DTX/DRX depends on the detailed mechanism of cell DTX/DRX, e.g.

- Whether the HARQ timing will be impacted by the cell DTX/DRX mechanism.

- Whether SR/SPS/CG can be transmitted at the inactive slot when cell DTX/DRX is configured.

- Whether CSI-RS and CSI report can be transmitted at the inactive occasion when cell DTX/DRX is configured.

- If the cell DTX/DRX impacts the UE behaviour in RRC_CONNECTED, the legacy UE will not work in cell configured with cell DTX/DRX, and some mechanism will be necessary to prevent legacy UE camping on cells configured with cell DTX/DRX. Otherwise, it is not necessary to prevent legacy UE camping on cells configured with cell DTX/DRX。

- Observation 3: Whether to prevent legacy UE camping on cells configured with cell DTX/DRX depends on whether the legacy UE can work in the cells configured with cell DTX/DRX.

- Proposal 2: Pending the discussion on whether and how to prevent legacy UE camping on cells configured with cell DTX/DRX until the detailed mechanism of cell DTX/DRX has been defined.

- Based on the following notes for objective of techniques in spatial and power domains in NES WID[2], the enhancements on spatial and power domains for NES does not impact the legacy UE behaviour..

- Note: Above objectives are only for UE specific channels/signals

- Note: Legacy UE CSI/CSI-RS capabilities applies when considering total number of CSI reports and requirements

- Observation 4: The enhancements on spatial and power domains for NES will not impact the legacy UE behaviour.

- Conclusion

- Observation 1: During cell selection and cell re-selection procedure, UE shall not select or re-select a SSB-less cell.

- Proposal 1: For cell selection or cell re-selection procedure, SSB-less cell is not included for the NES mode definition.

- Observation 2: The legacy UE may camp in the cell configured with cell DTX/DRX.

- Observation 3: Whether to prevent legacy UE camping on cells configured with cell DTX/DRX depends on whether the legacy UE can work in the cells configured with cell DTX/DRX.

- Proposal 2: Pending the discussion on whether and how to prevent legacy UE camping on cells configured with cell DTX/DRX until the detailed mechanism of cell DTX/DRX has been defined.

- Observation 4: The enhancements on spatial and power domains for NES will not impact the legacy UE behaviour.

- References

- 3GPP TR 38.864 V1.0.0 (2022-12), Study on network energy savings for NR

- RP-223540, New WID: Network energy savings for NR