3GPP TSG-RAN WG2 Meeting #121	                                  R2-2301068

Athens, Greece, February 27th-March 3rd, 2023

Source: 			ZTE Corporation, Sanechips

Title: 	Discussion on SSB-less SCell operation for NES

Agenda item:		8.3.3

Document for: 		Discussion and Decision

- Introduction

In RAN#98, a new WID[1] of Network energy savings for NR has been agreed. And the objective about SSB-less Cell is the following:

- Specify SSB-less SCell operation for inter-band CA for FR1 and co-located cells, if found feasible by RAN4 study, where a UE measures SSB transmitted on PCell or another SCell for an SCell’s time/frequency synchronization (including downlink AGC), and L1/L3 measurements, including potential enhancement on SCell activation procedures if necessary [RAN4, RAN2]

In this paper, we will focus on the SSB-less SCell operation for inter-band CA, and discuss the impact on RAN2 specification.

- Discussion

During the discussion in RAN#98, there is a general consensus that SSB-less SCell for inter-band CA has the potential to provide large gain for network energy savings. To be more specific, according to the simulation results in TR 38.864, it can provide up to 80.4%~97.4% energy saving gain at empty load, up to 72.7% energy saving gain at low load. Therefore, SSB-less SCell operation for inter-band CA for FR1 and co-located cells is included in Rel-18 network energy saving (NES) WI scope.

In R15, the SSB-less SCell for intra-band CA already has been supported. The timing and frequency synchronization of SSB-less SCell could depend on the co-site SpCell. For TA maintenance, SSB-less SCell and co-site SpCell are in the same TAG, it can be ensured by NW implementation. For UL and DL relationship, the pathloss reference for power control of SRS, PUCCH and PUSCH in SCell could be derived from CSI-RS or SSB in SpCell via RRC configuration, and the spatial relationship of PUCCH and SRS in SCell could be derived from CSI-RS or SSB in SpCell via RRC configuration. For cross-carrier scheduling, PUSCH in SCell could be scheduled by PDCCH in SpCell. For RRM, in co-site scenario, the addition/change/release of SSB-less SCell could be performed based on the measurement results of the co-site SpCell, and this can be left to NW implementation, hence, no measurement object for SSB-less SCell could be configured.

As above, it can be observed that current specification has already supported the SSB-less SCell in intra-band CA case. And UE could obtain the timing, frequency synchronization of SSB-less SCell from the SpCell.

Observation1: SSB-less intra-band CA can be well supported in current specs and UE could obtain the following information of SSB-less SCell from other serving cell with SSB within the same band.

-  timing, frequency synchronization

-  TA maintenance

-  power control and pathloss reference

-  RRM measurement result

For the SSB-less SCell for inter-band CA, from RAN2 perspective, the difference between SSB-less SCell for inter-band CA and SSB-less SCell for intra-band CA is mainly the band of SCell, and the specification modification in RAN2 seems quite limited. In RAN2#119B, an email discussion[2] is scheduled for SSB-less SCell for inter-band CA, and the following impacts have been identified in RAN2:

-   introduce a new UE capability to indicate the support of inter-band SCell without SSB; and

-   small clarification in the specification (e.g. to extend the field description of the absoluteFrequencySSB IE to inter-band case).

At last, an agreement “RAN2 assumes that the SSB-less solution for inter-band CA in connected mode we can consider to use the intra-band CA mechanism as a baseline/starting point. FFS whether there are other impacts for RAN2 according to other WGs discussion” was reached.

Observation 2: Since the mechanism for intra-band SSB-less SCell can be used as baseline, the impact on RAN2 for inter-band SSB-less SCell is quite limited, and the impacts are mainly in the following two aspects:

-   New UE capability to indicate the support of inter-band SSB-less SCell

-   Clarifications/Changes to enable the configuration of inter-band SSB-less SCell (e.g. to extend the field description of the absoluteFrequencySSB IE to inter-band case).

According to the WID, since RAN4 will work on the feasibility study first and identify the feasible use case, scenarios and potential impact from RAN4 perspective. Also considering the potential changes on the configuration of inter-band SSB-less SCell mainly on the input from RAN4 on the feasible use case, scenarios and restrictions, we propose to postpone the discussion on the configuration of inter-band SSB-less SCell, and wait for the RAN4 progress first.

Observation 3: The potential changes on the configuration of inter-band SSB-less SCell mainly depends on the input from RAN4 on the feasible use case, scenarios and restrictions.

Proposal 1: Postpone the discussion on the configuration of inter-band SSB-less SCell (e.g. remove the restriction for inter-band SSB-less SCell in RAN2 specs) until RAN4 input has been received.

Although we propose to postpone the discussion on the configuration of inter-band SSB-less SCell, some early discussion on UE capability seems necessary to understand the general framework of UE capability signaling and identify the potential questions for RAN4 for the UE capability aspect.

For intra-band SSB-less SCell, a per band capability is introduced to indicate whether UE support SSB less in all SCell within a band combination. However, for the inter-band SSB-less SCell, since UE may support inter-band SSB-less SCell in a certain band combination of PCell and certain SCell, a per band capability seems not sufficient.

Observation 4: Different from intra-band SSB-less SCell, per band capability is not sufficient for inter-band SSB-less SCell.

According to the WID, for each inter-band SSB-less SCell, there should be a inter-band “reference serving cell”, and UE is expected to measures SSB transmitted in the “reference serving cell” for the SSB-less inter-band SCell’s time/frequency synchronization (including downlink AGC), and L1/L3 measurements. Therefore, from UE capability aspect, for each band for inter-band SSB-less SCell, some kind of per band combination or per band pair indication will be required.

For band combination, the fallback rule for the band combination capability is an important hypothesis. For example, for a band combination with BAND A+B+C, the NW will assume the UE support band combination BAND A+B, BAND A+C, BAND B+C. However, for inter-band SSB-less SCell, the legacy fallback rule isn’t applicable. As above, since the important information (e.g. timing, frequency synchronization and so on) of SSB-less SCell depends on the serving cell (e.g. PCell or SCell with SSB) on reference band. Once the reference band is not included in the fallback case, the SSB-less inter-band SCell can not be configured. For example, if BAND C is required as reference band for the inter-band SSB-less SCell on band B, then band combination A+B (with SSB-less SCell) can not be supported even in case A+B(with SSB-less SCell)+C can be supported.

To aovid the issue caused by fallback case, a reference band list for each SSB-less SCell capable band will be required, and inter-band SSB-less SCell can only be configured in case a serving cell with SSB on the corresponding potential ‘reference’ band is configured in the CA operation.

Proposal 2: For the UE capability of inter-band SSB-less SCell, for each band supported for inter-band SSB-less SCell, a potential ‘reference’ band list will be configured, and inter-band SSB-less SCell can only be configured in case a serving cell with SSB on the corresponding potential ‘reference’ band is configured in the CA operation.

For the ‘reference’ band list, one issue is whether the reference band list should be configured per band combination, or it can be configured independently (i.e. the reference band list is applicable to all band combination). Based on our understanding, the relationship between the band for SSB-less SCell and the reference band is mainly determined by hardware on UE side, which will not be changed even in case the two band are involved in a band combination with more band. Therefore, to reduce signaling overhead, we propose to have an independent reference band list.

Proposal 3: The ‘reference band list for each inter-band SSB-less SCell capable band’ should be configured independently (independent from band combination), and it is applicable to all the band combination.

- Conclusion and proposals

Based on the analysis in previous sections, the following observations and proposals are given:

Observation 1: SSB-less intra-band CA can be well supported in current specs and UE could obtain the following information of SSB-less SCell from other serving cell with SSB within the same band.

-  timing, frequency synchronization

-  TA maintenance

-  power control and pathloss reference

-  RRM measurement result

Observation 2: Since the mechanism for intra-band SSB-less SCell can be used as baseline, the impact on RAN2 for inter-band SSB-less SCell is quite limited, and the impacts are mainly in the following two aspects:

-   New UE capability to indicate the support of inter-band SSB-less SCell

-  Clarifications/Changes to enable the configuration of inter-band SSB-less SCell (e.g. to extend the field description of the absoluteFrequencySSB IE to inter-band case).

Observation 3: The potential changes on the configuration of inter-band SSB-less SCell mainly depends on the input from RAN4 on the feasible use case, scenarios and restrictions.

Proposal 1: Postpone the discussion on the configuration of inter-band SSB-less SCell (e.g. remove the restriction for inter-band SSB-less SCell in RAN2 specs) until RAN4 input has been received.

Observation 4: Different from intra-band SSB-less SCell, per band capability is not sufficient for inter-band SSB-less SCell.

Proposal 2: For the UE capability of inter-band SSB-less SCell, for each band supported for inter-band SSB-less SCell, a potential ‘reference’ band list will be configured, and inter-band SSB-less SCell can only be configured in case a serving cell with SSB on the corresponding potential ‘reference’ band is configured in the CA operation.

Proposal 3: The ‘reference band list for each inter-band SSB-less SCell capable band’ should be configured independently (independent from band combination), and it is applicable to all the band combination.

- References

[1] RP-223540 New WID proposal: Network energy savings for NR

[2] R2-2210995 Report of [Offline-302][NES] Cell Selection/Reselection and SSB/SIB-less (Huawei)