3GPP TSG-RAN WG2 Meeting #121 electronic	R2-2301072

Athens, 27th February – 3rd March, 2023

Agenda item:		8.9.4

Source:		Lenovo

Title:		Discussion on Multi-path relaying

Document for:		Discussion and Decision

# Introduction

The following agreements for multi-path relaying are achieved in RAN2#120.

Agreement:

Support PCell on the direct path only when the UE is in multi-path operation, for both scenario 1 and scenario 2.

Agreements:

Proposal 1	[Easy] RAN2 confirms the following WA for Scenario 2.

•	Bearer identification except LCID is not needed in L2 PDU over Uu link in Scenario 2. Only 1:1 bearer mapping is supported over Uu link for the indirect path. FFS how to configure the mapping.

•	Without the adaptation layer over Uu link in scenario 2, a PDCP PDU can be delivered to an intended PDCP entity or RLC entity for support of more than one RB over Uu link e.g. by configuring 1:1 bearer mapping and different Uu RLC channels for relay UE local traffic and relay traffic for PDU delivery.

•	Do not specify adaptation layer over Uu link for scenario 2 in RAN2.

Proposal 2	[Easy] How to configure 1:1 bearer mapping and potential spec impact can be discussed in normative phase.

Proposal 3	[Easy] In principle, Mode 1 RA can be supported for the remote UE configured with multi-path in Scenario 1.]

Agreements:

Proposal 7 (modified)	[Easy] R2 confirms that split SRB can be configured with or without duplication as a baseline, for both scenarios (assuming it is supported in scenario 2 as proposed elsewhere). Further restrictions can be discussed in normative phase.

Proposal 13.	[Easy]For scenario 2, non-split SRB1/2 is allowed to be configured on direct path.

Agreement:

Proposal 14 (modified)	[Easy] Remote UE storing indirect path configuration (e.g., SRAP and PC5-RLC channel configurations) and resuming directly into multi-path configuration is not supported for scenario 1.

Agreement:

Proposal 16 (modified)	[Easy] If CSS for SI is configured within the active BWP on PCell, the remote UE can perform direct system information acquisition on PCell as currently specified in 38.331; besides, dedicated signaling can be used to deliver SIB via SRB1 configured on direct and/or indirect path as currently specified in 38.331.

Agreement:

Proposal 17	[Easy] Upon detection of 3GPP-defined RLF failure in one path, remote UE (configured with MP) can report path failure via the alternative available path if SRB1 is configured on the alternative path or split SRB1 is configured.

Agreement:

Proposal 21 (modified)	[Easy] PDCP Control PDU is not duplicated.

Agreement:

RAN2 do not define a control plane primary path concept in the study phase; FFS if something needs to be defined in normative work, but it should be driven by functionality and technical benefits.

Agreements:

Proposal 6a	[RAN2 to discuss] case B and case D are not supported for Scenario 2.

Proposal 9 (modified)	[RAN2 to discuss] For Scenario 2, Case E is not supported.

For Scenario 2, whether to support Case G is discussed in normative phase, but RAN2 will not do additional work to enable it for Scenario 2 over Scenario 1.

Agreements:

Whether SRB1/2 can be configured in different path for Scenario 1 can be discussed in normative phase.

Whether non-split SRB1/2 is allowed to be configured on indirect path for scenario 2 and whether split SRB1/2 is supported for scenario 2 can be discussed in normative work.

Agreement:

Remote UE storing indirect path configuration or not and use it to resume to MP configuration in scenario 2 is not supported.

In this contribution, we further discuss the related issues in multi-path relaying case.

# Discussion

Mobility scenarios

It has been agreed that the relay change for the remote UE can be supported in multi-path relaying case. Considering that it has been included in the scope of WI to enhance service continuity for the scenarios e.g., intra-gNB and inter gNB indirect-to-indirect path switching, it is natural to support the intra-gNB Relay change via a single switch procedure. Correspondingly, the related legacy timer, e.g., T420 can also be reused in multi-path relaying case. Similarly, the intra-gNB cell change can also be via a single procedure.

Proposal 1: Intra-gNB Relay change via a single switch procedure while the remote UE keeps the direct path under the same gNB should be supported.

Proposal 2: Legacy timer T420 can be reused in the case of relay change.

Proposal 3: Intra-gNB cell change via a single procedure while the remote UE keeps the indirect path under the same gNB should be supported.

Similar to the discussion of mobility with DC, there are several potential cases when considering the inter-gNB handover in the multi-path relaying case. It could be one path or multi-path in the source side, and it could also be one path or multi-path in the target side. And for the indirect path, it can be further distinguished as same or different relays in the source side and target side. Therefore, we list the three potential use cases to be discussed in multi-path relaying case.

Proposal 4: The following Inter-gNB handover associated with Multi-path should be discussed.

- Use Case 1: Multi-path in source gNB-> Only one path in target gNB;

- Use Case 2: One path in source->Multi-path in target gNB;

- Use Case 3: Multi-path in source gNB-> Multi-path in target gNB (different relays in the source gNB and target gNB);

For scenario 2, the remote UE and relay UE is a pair, which could be pre-configured. That means the remote UE is allowed to select a relay UE based on the pre-configuration. One case is that the relay UE is in proximity of the remote UE. And they may move together. Therefore, Inter-gNB handover for remote UE with the same relay UE should be supported for scenario 2.

Proposal 5: Inter-gNB handover of a remote UE together with the same relay UE in Scenario 2 should be discussed.

Enhancement of throughput and reliability

Considering the existing mechanisms, when configuring duplication for a DRB, RRC also sets the state of PDCP duplication (either activated or deactivated) at the time of (re-)configuration. After the configuration, the PDCP duplication state can then be dynamically controlled by means of a MAC control element and in DC, the UE applies the MAC CE commands regardless of their origin (MCG or SCG). When duplication is configured for a SRB, the state is always active and cannot be dynamically controlled.

Since the purpose is similar to the legacy case, the existing activation/deactivation procedure can be used as the baseline for multi-path relaying case. Correspondingly, the Duplication Activation/Deactivation MAC CE can be reused to activation/deactivation PDCP DRB.

Proposal 6: The activation/deactivation of PDCP duplication for a DRB in multi-path relaying case can be dynamically controlled by the gNB.

Proposal 7: The Duplication Activation/Deactivation MAC CE can be reused.

With the similar consideration, the mechanism of data split can be used to allow the transmission to be switched among the direct path and indirect path in Rel-18 multi-path relaying case. The legacy configuration and transmission rule can be applied to Rel-18 multi-path relaying case as the baseline. Then the existing data volume threshold for split bearer can be considered as a baseline, and some enhancements may be needed to support MP split bearer in multi-path relaying case.

Proposal 8: Both legacy data volume threshold for split bearer and some enhancements can be considered to support MP split bearer in multi-path relaying case.

Sidelink Resource scheduling

Based on the above agreement, the following Figure 1 depicts the required scenario:

Figure 1: Inter cell - Intra gNB multipaths for a SL remote UE

The Figure 1 shows a scenario where the two “serving” cells of the Remote UE end up in the same gNB i.e., cell-1 and cell-2 Distributed Units (DU) are terminating in the same Centralized Unit (CU):

In legacy a sidelink transmitter is only provided sidelink resources from one serving cell. In multipath scenario however there are two serving cells available to schedule the UE which could come in handy in special circumstances like RLF on one link, or due to Consistent LBT failures or simply due to radio geometry.

Further, following sidelink data need to be transmitted by the remote UE:

- data that is destined to be sent to gNB and is configured to be carried in an indirect bearer i.e., a bearer configured only on the indirect path (data type-a)

- data that is destined to be sent to gNB and is configured to be carried in a split bearer i.e., data from this application will be carried in direct Uu link as well as in the indirect link/ path (data type-b)

- sidelink data that is destined for the U2N sidelink relay i.e., relay does not forward this data on the Uu interface to its serving cell (data type-c)

- sidelink data that is destined for another sidelink UE i.e., relay acts also as a UE to UE relay and forward this data on the sidelink interface to a destination sidelink remote UE (data type-d)

Figure 2

As shown in Figure 2, Bearer-1 is using only Uu resources of Cell-1. Bearer-2 is split bearer and remote UE may use both Uu and SL resources. For Bearer-3 the remote UE shall only use sidelink resources. Bearer-4 terminates in another SL device (U2N Relay or another SL UE). It is possible that different SL data are scheduled differently e.g., using Mode 1 or Mode 2 resources from Cell 1 or Cell 2. As one simple possibility, a UE would use sidelink resources (i.e., perform resource selection for transmission) provided in broadcast signalling (e.g., SIB12) by the cell with which UE had first link established. If a sl-ConfigDedicatedNR is received the UE uses the dedicated SL configuration for Mode 2. If Mode 1 scheduling is used, when the first link is the direct link, UE uses Mode 1 SL resources.

When the second path is added, following options are possible for SL scheduling:

- The first cell continues to schedule all SL resources for any SL data transmission of the remote UE.

- UE may use Mode 2 transmission resources from the indirect link in addition to Mode1/ Mode2 transmission resources from the direct link. Being able to use resources from both cells can be helpful in mobility situations.

- UE uses SL resources provided by the Primary path for all SL data type. If the primary path changes, the UE’s SL resources are scheduled by the new primary path.

As a first discussion, it can be beneficial to discuss if the remote UE can use mode 2 SL transmission resources from both the cells and if use of Mode 1 from direct link and Mode 2 from indirect must be restricted, we see no reason for this, and this does not mean a violation of simultaneous Mode 1 Mode 2 principle.

Proposal 9: Remote UE can use mode 2 SL transmission resources from both the serving cells in a multipath scenario.

Proposal 10: Remote UE can use mode 1 SL transmission resources from direct path and mode 2 SL transmission resources from the serving cell on the indirect path, in a multipath scenario.

# Conclusion

In this contribution, the mobility scenarios and enhancement of throughput and reliability for multi-path relaying are discussed. We have the following proposals:

Proposal 1: Intra-gNB Relay change via a single switch procedure while the remote UE keeps the direct path under the same gNB should be supported.

Proposal 2: Legacy timer T420 can be reused in the case of relay change.

Proposal 3: Intra-gNB cell change via a single procedure while the remote UE keeps the indirect path under the same gNB should be supported.

Proposal 4: The following Inter-gNB handover associated with Multi-path should be discussed.

- Use Case 1: Multi-path in source gNB-> Only one path in target gNB;

- Use Case 2: One path in source->Multi-path in target gNB;

- Use Case 3: Multi-path in source gNB-> Multi-path in target gNB (different relays in the source gNB and target gNB);

Proposal 5: Inter-gNB handover of a remote UE together with the same relay UE in Scenario 2 should be discussed.

Proposal 6: The activation/deactivation of PDCP duplication for a DRB in multi-path relaying case can be dynamically controlled by the gNB.

Proposal 7: The Duplication Activation/Deactivation MAC CE can be reused.

Proposal 8: Both legacy data volume threshold for split bearer and some enhancements can be considered to support MP split bearer in multi-path relaying case.

Proposal 9: Remote UE can use mode 2 SL transmission resources from both the serving cells in a multipath scenario.

Proposal 10: Remote UE can use mode 1 SL transmission resources from direct path and mode 2 SL transmission resources from the serving cell on the indirect path, in a multipath scenario.

# References

RAN2#120 chairman notes.