3GPP TSG-RAN WG2 Meeting #121  	                            R2-2301093

Athens, Greece, Februray 27 – March 3, 2023	(Revision of R2-2211929)

Agenda Item:	8.7.4.1.1

Source: 	Sony

Title:	Cell selection/reselection enhancements in NTN-TN

Document for:	Discussion

# Introduction

In RAN2#120 meeting, RAN2 discussed cell selection/reselection enhancement and we have reached the agreements as follows.

Agreements:

- RAN2 will first continue the investigation on the details of the TN coverage data (e.g. accuracy requirements for describing where TN network(s) is/are available) and UE storage overhead before deciding how to send the information to the UE.

- Continue the discussion on whether to introduce explicit indication to identify TN cells from inter-frequency list and inter-RAT frequency list (FFS on the granularity) or whether we rely on implicit information.

In this contribution, we address some further details on cell selection/reselection for NTN-TN enhancements.

# Discussion

When an NTN-capable UE moves from an NTN cell to a TN cell, handover/reselection to the terrestrial cell may not be triggered if only based on the serving cell quality because the link quality of the NTN cell may still be good enough. As a result, the cell reselection or handover based on serving cell RSRP measurement may not work even when UE physically moves into TN coverage. Therefore, network should make the UE aware of when to start performing the measurements on TN cells and not apply serving cell criteria, when moving from an NTN cell towards a TN cell.

For RRC_IDLE and RRC_INACTIVE UEs, One straightforward solution could be that the network will include in its e.g., system information to indicate that its coverage overlaps with a terrestrial cell’s coverage. So, for the UEs that are not in the NTN cells that overlap with terrestrial cell’s coverage, they won’t need to perform any other TN measurements.

One option discussed and proposed in Rel-17 was that If TN is on a different frequency then its priority should be set accordingly (higher than NTN) so that UE measures high priority frequency at regular intervals. But if a UE is in the sea for a long time but near the shore where NTN cell overlaps a TN cell coverage (NTN cell size being much bigger than TN cell) and it is still located away from TN coverage then such measurements will simply result in wasted UE power consumption and this scenario should be avoided.

On further details of the assistance information, in order to solve the problem as stated above, in addition to the indication that an NTN cell overlaps with a TN cell’s coverage, network can set a distance threshold to a reference location e.g. border to TN cell. Only when the UE moves beyond that threshold e.g. is closer to the border area with a TN cell, it will start performing TN measurement.

Proposal 1: For RRC_IDLE and RRC_INACTIVE UEs, an indication could be included in system information to indicate NTN cell’s coverage overlaps with terrestrial TN cell’s coverage. With this indication, UE may ignore serving cell thresholds and perform TN cell measurements.

Proposal 2: The assistance information could be a distance threshold to a reference location. If UE moves beyond that threshold then it will start performing TN measurement.

The radio link quality of a satellite cell is relatively low compared with that of a terrestrial cell. As a result, when a UE moves to an area that is covered by both satellite and terrestrial cells, the UE will tend to select the terrestrial cell as the satellite cell may have a lower RSRP value or switch off the NTN receiver. However, UE should start measurements for NTN network if it is at the edge of the terrestrial cell coverage. So, serving cell may provide the NTN neighbour cell information e.g., an indication that whether a neighbour cell is an NTN cell or not.

Proposal 3: Serving cell’s system information include an indication that whether a neighbour cell is an NTN cell or not.

# Conclusion

We propose RAN2 to discuss the proposals as follows.

Proposal 1: For RRC_IDLE and RRC_INACTIVE UEs, an indication could be included in system information to indicate NTN cell’s coverage overlaps with terrestrial TN cell’s coverage. With this indication, UE may ignore serving cell thresholds and perform TN cell measurements.

Proposal 2: The assistance information could be a distance threshold to a reference location. If UE moves beyond that threshold then it will start performing TN measurement.

Proposal 3: Serving cell’s system information include an indication that whether a neighbour cell is an NTN cell or not.