3GPP TSG RAN WG2 Meeting #121                                                        	R2-2301105

Athens, Greece, 27th Feb – 3rd March 2023

Agenda item:	8.16.2

Source:	Samsung

Title:	AI/ML Capability Reporting

Document for:	Discussion and Decision

# 1. Introduction

In RAN2#120 meeting, RAN2 discussed the Rel-18 study on AI/ML for NR air interface [1] and concluded the following assumptions and agreements on the AI/ML methods [2]:

### 8.16.2 AIML methods R2 assumes that model ID can be used to identify which AI/ML model is being used in LCM including model delivery. R2 assumes that model ID can be used to identify a model (or models) during model selection/activation/deactivation/switching (can later align with R1 if needed).

In this contribution, we discuss the need for AI/ML capability reporting between the network and UE, considering latest RAN1 agreements and discussion on this issue.

# 2. Discussion

2.1 AI/ML Capability Reporting

In RAN2#119bis-e meeting, RAN2 briefly discussed UE capability and agreed the following [3]:

### 8.16.1 Organizational - Assume that e.g. for the management of data and AI/ML models, RAN2 could start by focusing on data collection, model transfer, model update, model monitoring and model selection/(de)activation/switching/fallback (to the extent needed), whether UE capabilities has a role in this.

Observation 1: RAN2 to discuss the role of UE capabilities in supporting AI/ML LCM procedures.

In our view, both the network and the UE would need to exchange information related to their AI/ML capability support for generic AI/ML operation and also for specific AI/ML functionality (or use case). This awareness of UE capabilities would be useful for supporting a model LCM procedures.

Observation 2: It is beneficial to define AI/ML capability reporting between the UE and network.

Moreover, the network and UE may exchange information associated with supported AI/ML models, use cases, applicable scenario/configurations or model deployment side for training and inference (UE-side, network-side).

Observation 3: The AI/ML capability may be indicated as a generic or use-case-specific capability.

Proposal 1a: RAN2 to introduce AI/ML capability reporting between the UE and network.

Proposal 1b: RAN2 to discuss whether AI/ML capability is defined/reported as a generic AI/ML operation capability or a use-case-specific capability or both.

In RAN2#120 meeting, RAN2 agreed the use of a model-ID-based identification to identify a model (or models) in LCM procedures including a model delivery [2]:

R2 assumes that model ID can be used to identify which AI/ML model is being used in LCM including model delivery. R2 assumes that model ID can be used to identify a model (or models) during model selection/activation/deactivation/switching (can later align with R1 if needed).

In RAN1#111 meeting, RAN1 agreed to study two alternative mechanisms for indication of LCM procedures (i.e. indication of activation/deactivation/switching/fallback): (a) functionality-based indication, and (b) model-ID-based indication. However, RAN1 has not yet decided on whether or how to indicate functionality [4]:

Agreement For UE-part/UE-side models, study the following mechanisms for LCM procedures: - For functionality-based LCM procedure: indication of activation/deactivation/switching/fallback based on individual AI/ML functionality - Note: UE may have one AI/ML model for the functionality, or UE may have multiple AI/ML models for the functionality. - FFS: Whether or how to indicate Funtionality - For model-ID-based LCM procedure, indication of model selection/activation/deactivation/switching/fallback based on individual model IDs

In our understanding, considering the above agreements, there are two alternatives for exchanging AI/ML capability support for a model LCM procedures or model delivery:

- Alt. 1 (functionality-based): The UE and network exchange information on AI/ML capability support in relation to a given functionality (scenarios/configurations/use case) of a model.

- Alt. 2 (model-ID-based): The UE and network exchange information on AI/ML capability support in relation to a given model ID (or models IDs).

Proposal 2: RAN2 to discuss/study whether the network and UE can exchange information on AI/ML capability support for LCM procedures and model delivery, using either functionality-based or model-ID-based mechanism.

# 3. Conclusion

In this contribution, we discuss the need for AI/ML capability reporting between the network and UE, considering latest RAN1 agreements and discussion on this issue. The following are the observations and proposals in this document:

Observation 1: RAN2 to discuss the role of UE capabilities in supporting AI/ML LCM procedures.

Observation 2: It is beneficial to define AI/ML capability reporting between the UE and network.

Observation 3: The AI/ML capability may be indicated as a generic or use-case-specific capability.

Proposal 1a: RAN2 to introduce AI/ML capability reporting between the UE and network.

Proposal 1b: RAN2 to discuss whether AI/ML capability is defined/reported as a generic AI/ML operation capability or a use-case-specific capability or both.

Proposal 2: RAN2 to discuss/study whether the network and UE can exchange information on AI/ML capability support for LCM procedures and model delivery, using either functionality-based or model-ID-based mechanism.

# 4. Reference

RP-221348 Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface.

RAN2#120 Chairman report

RAN1#119bis-e Chairman report.

RAN1#111 Chairman report.