3GPP TSG-RAN WG2 Meeting #121	 										   R2-2301117

Athens, Greece, 27th February – 3rd March, 2023

Source: 	Xiaomi

Title:  	Discussion on the band conflicts for MUSIM

Agenda Item:	8.17.2

Document for:	Discussion and Decision

# Introduction

According to the WID [1] of the Rel-18 MUSIM, RAN2 needs to find solutions to for UE capability restriction issue of multiple SIMs:

1. Enhancements for MUSIM procedures to operate in RRC_CONNECTED state simultaneously in NW A and NW B. [RAN2, RAN3, RAN4]. - Specify mechanism to indicate preference on temporary UE capability restriction and removal of restriction (e.g. capability update, release of cells, (de)activation of configured resources) with NW A when UE needs transmission or reception (e.g., start/stop connection to NW B) for MUSIM purpose - RAT Concurrency: Network A is NR SA (with CA) or NR DC. Network B can either be LTE or NR. - Applicable UE architecture: Dual-RX/Dual-TX UE The work item shall identify whether the WI will have RAN3 or RAN4 impacts by RAN#99 [RAN2].

According to the discussion in the RAN2#119bis-e meeting [2], RAN2 made the following agreement:

- RAN2 aims to address at least the Scenario 1: the UE in network A in RRC_CONNECTED indicates (i.e. adds/removes) its preference on temporary UE capability due start/stop connection in NW B. This can be e.g. CA/DC capability restriction. - RAN2 to discuss whether the following UE capabilities (not a complete list) are impacted for dual-active MUSIM: MIMO layers, BC capabilities, Measurement capabilities, Bandwidth, srs-TxSwitch, UL tx power, Power Class.

According to the email discussion for MUSIM [3], the band conflict issues were discussed. In this contribution, we provide the capability sharing issue related to BC capability.

# Discussion

## UE capability restriction for BC capability

According to the current UE capability bits indicated for the band combination, the UE will indicate its supported bands for each band combination. According to the email discussion for MUSIM in [3], we could have the following options:

- Option 1: UE request for releasing SCell/SCG

- Option 2: UE request for deactivating SCell/SCG

Firstly, we would consider that the intention of releasing/deactivating SCell/SCG is to free some RF chains which will then be used by another SIM-B. However, deactivating SCell/SCG does not really free the RF chain occupied by SIM-A. As the deactivation/reactivation of SCell/SCG is to have faster control (i.e. faster on/off for power saving) of the SCell/SCG, the UE would still be required to perform L1/L3 measurements, including RRM measurement, CSI measurement and RLM/BFD. Then the RF chain would still be occupied for the deactivated SCell or SCG. Thus, we think that RAN2 should adopt Option 1.

Proposal 1: The UE can request the release of SCell or SCG.

With regards to the details of Option 1, we think that if a UE is requesting the release of a SCell, the UE also needs to indicate which SCell is to be release, as a UE could be configured with more than one SCells and the SCell index can be same across MCG and SCG.

Proposal 2: The UE indicates which SCell(s) are to be release, by indicating SCell ID of MCG/SCG.

After freeing some RF chains for the use of SIM-B, the SIM-B could also change its required RF chains, due to the configuration change in SIM-B. Thus, the UE should be able to indicate the latest status of its request SCell RF chain for SIM-B.

Proposal 3: The UE can indicate whether the previous request for releasing SCell or SCG is not valid.

Proposal 4: The UE can update the SCells requested for releasing.

# Conclusions

According to the analysis given above, we have the following proposals:

Proposal 1: The UE can request the release of SCell or SCG.

Proposal 2: The UE indicates which SCell(s) are to be release, by indicating SCell ID of MCG/SCG.

Proposal 3: The UE can indicate whether the previous request for releasing SCell or SCG is not valid.

Proposal 4: The UE can update the SCells requested for releasing.

# Reference

[1] RP-220955, vivo, “Revised WID: Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR”

[2] RAN2#119bis-e meeting minutes

[3] [Post119bis-e][212][MUSIM] Rel-18 MUSIM solutions (QC - Vivo)