3GPP TSG-RAN WG2 #121	R2-2301154

Athens, Greece, 27 February – 3 March, 2022

Agenda item:		8.4.2.2 RRC

Title:		RRC aspects in LTM

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In RAN2#119bis-e, some important agreements were made as basics of the L1/L2-triggered mobility (LTM). Further in RAN2#120, there were additional agreements as followings [1].

P1 RAN2 to confirm that the CellGroupConfig IE is (mandatory) needed within an LTM candidate cell configuration. P3 The RadioBearerConfig IE can be optionally supported in an LTM candidate configuration P5 The MeasConfig IE can be optionally supported in an LTM candidate configuration. P8 The OtherConfig IE is not required to be part of the LTM candidate cell configuration. P9 The LTM candidate cell configuration should be designed as a To AddMod/ToRelease structure. P10 The LTM candidate cell configuration ASN.1 structure comprises at least a CellGroupConfig IE and a configuration ID. Chair: We Attempt to decide further at next meeting.

On Delta Configuration A UE stores the reference configuration as a separate configuration. The reference configuration is managed separately

As per the conclusion in the last meeting, we consider it should be decided which RRC model is used for the LTM. In this contribution, we firstly discuss the RRC model and then discuss other open issues.

# 2. Discussion

2.1	RRC model

After RAN2#119bis-e, there were two RRC models on the table:

- Model 1: One RRCReconfiguration message (or FFS RRCReconfiguration IEs) for each candidate target configuration

- Model 2: One CellGroupConfig IE (FFS additional IEs) for each candidate target configuration

In RAN2#120, it was agreed that the CellGroupConfig IE is mandatory, while the RadioBearerConfig IE and the MeasConfig IE can be optional in an LTM candidate configuration. For those optional IEs, we understand that the signalling needs to support to include them at least. In other words, given that RAN2 should specify a single RRC model, it has to be possible to include the optional IEs as well from signalling perspective.

The model 1 can already cover the agreements so far. Regarding the FFS (i.e., either RRCReconfiguration message or IE), we assume no strong motivation to select the message rather than the IE due to potential signalling overhead. In addition, the RRCReconfiguration IE is more suitable to follow the other agreement that the LTM candidate cell configuration should be designed as a ToAddMod/ToRelease structure. On the other hand, the model 2 can be simply modified to include the optional IEs. With this understanding, the current options of RRC models can be modified as follows. Both models also need the configuration ID.

- Model 1’: One RRCReconfiguration IE for each candidate target configuration

- Model 2’: One CellGroupConfig IE (and optionally RadioBearerConfig IE and the MeasConfig IE) for each candidate target configuration

Comparing the models, the model 1’ is much simpler than the model 2’ which needs to define new ASN.1 structure for the legacy IEs (e.g., as exemplified in [2]). One may still have concern on potential signalling overhead in the model 1’. However, RAN2 can apply some restrictions so that the RRCReconfiguration IE only includes the necessary IEs which are exactly the same regardless of the models. We assume that for this restriction, there is no need to check one by one for IEs in RRCReconfiguration message. Instead of that, RAN2 can consider which IEs are necessary as done in RAN2#120, if anything more is necessary in addition to the CellGroupConfig IE, the RadioBearerConfig IE and the MeasConfig IE)

Proposal 1: RAN2 to select the model 1’ (One RRCReconfiguration IE for each candidate target configuration) as the RRC model in the LTM.

Proposal 2: RAN2 to assume that the RRCReconfiguration IE in the model 1’ can include only the CellGroupConfig IE, the RadioBearerConfig IE and the MeasConfig IE. FFS for other IEs.

2.2	Delta configurations

The LTM can support the delta configuration. In RAN2#120, the handling of the reference configuration was clarified. The UE stores the reference configuration, which is managed separately from other configurations. We discuss how the delta configuration works in the LTM.

Firstly, it is not yet clear what can be the reference configuration. We assume that the source cell/gNB can configure the reference configuration and provide it to candidate target cells/gNBs during the LTM preparation for the SpCell change (e.g. HO). It is straightforward to consider that the reference configuration is the current configuration in the source SpCell (i.e. source configuration at the LTM preparation) just like normal mobility procedure, not another specific configuration. On the other hand, if a specific configuration is used as the reference configuration, the gNB has to send also the reference configuration to the UE, which increases a signalling overhead.

Proposal 3: RAN2 to agree that the source configuration at the LTM preparation can be used as the reference configuration for delta configuration in LTM.

Based on the proposal 3, the delta configuration in the first LTM after configuration is the same as the legacy mobility procedure. The difference will be seen in the subsequent mobility. For example, in the second LTM (i.e. the first subsequent LTM), the UE performs the delta configuration based on the stored reference configuration. In other words, the UE performs the mobility procedure by using the stored reference configuration and the target cell configuration. In the following subsequent LTM, the same way is applied.

For the delta configuration in the subsequent LTM, one issue is how the UE actually does such delta configuration. For example, it is not clear whether the UE firstly applies the reference configuration and then perform the delta configuration on top of the reference configuration, or the UE jointly applies the reference configuration and the target configuration with leaving detail processing up to UE implementation. In any case, this can be discussed in Stage 3 phase.

Proposal 4: RAN2 to confirm that the delta configuration in the subsequent LTM is performed based on the stored reference configuration and the target cell configuration.

Proposal 5: RAN2 to discuss how the delta configuration in the subsequent LTM is done in Stage 3.

2.3	Decoding and compliance check

In RAN2#119bis-e, it was also discussed whether ASN.1 decoding and validity/compliance check of candidate cell configuration are performed upon reception of the candidate cells configuration, which remains as FFS.

FFS whether ASN.1 decoding and validity/compliance check of candidate cell configuration are performed upon reception of the candidate cells configuration. FFS if this need to be specified.

The point here is that if we follow the conditional mobility in Rel-16/17, it is up to UE implementation exactly when it is performed. The reason was to avoid unnecessary failure indication. However, the LTM is intended only intra-CU scenarios and it is/should be much less possibility to (e.g.) excess UE capabilities. In addition, the intention behind the LTM is to extremely reduce mobility interruption. Therefore, if the possible latency reduction can be expected, RAN2 should take such way, i.e. ASN.1 decoding and validity/compliance check of candidate cell configuration are performed upon reception of the candidate cells configuration.

Proposal 6: RAN2 to agree that ASN.1 decoding and validity/compliance check of candidate cell configuration are performed upon reception of the candidate cells configuration.

# 3. Conclusion

In this contribution we discussed the RRC model in the LTM, the delta configuration and the ASN.1 decoding issue. Then we made the following proposals.

Proposal 1: RAN2 to select the model 1’ (One RRCReconfiguration IE for each candidate target configuration) as the RRC model in the LTM.

Proposal 2: RAN2 to assume that the RRCReconfiguration IE in the model 1’ can include only the CellGroupConfig IE, the RadioBearerConfig IE and the MeasConfig IE. FFS for other IEs.

Proposal 3: RAN2 to agree that the source configuration at the LTM preparation can be used as the reference configuration for delta configuration in LTM.

Proposal 4: RAN2 to confirm that the delta configuration in the subsequent LTM is performed based on the stored reference configuration and the target cell configuration.

Proposal 5: RAN2 to discuss how the delta configuration in the subsequent LTM is done in Stage 3.

Proposal 6: RAN2 to agree that ASN.1 decoding and validity/compliance check of candidate cell configuration are performed upon reception of the candidate cells configuration.

# References

[1] RAN2#120 Chair Notes

[2] R2-2212438, Qualitative analysis on what to include in the RRC model for LTM, Ericsson