3GPP TSG-RAN WG2 Meeting #121	R2-2301159

Athens, Greece, 27th February – 3rd March, 2023

Agenda item:	6.2.2

Source:	Huawei, CBN, Hisilicon

Title:	Discussion on the plmn-index usage for multicast

Document for:	Discussion

- Introduction

For MBS multicast, if plmn-index is used in the TMGI for an MBS session, the target gNB may not be able to recognize the plmn-index when the RRC context is forwarded to target gNB during handover. In the RAN2#120 meeting, this issue was discussed but postponed due to limit of time. In this contribution, we will further analyze this issue.

- Discussion

A similar issue for MBS broadcast MII transfer was discussed in RAN2#119e and the following agreement was made:

When UE reports plmn-index in the MII, the source gNB decodes the MII, translates the plmn-index to explicit PLMN ID and replaces the plmn-index with the explicit PLMN ID when sending MII to target gNB.

However, for MBS multicast, the similar issue exists, given that the TMGI is also included in the multicast MRB configuration and the RRC context transferred to the target gNB during handover. In the last RAN2 meeting, two options were proposed for this issue:

Option 1: reuse the agreement for MII for multicast MRB, i.e. the network replaces the plmn-index by the explicit PLMN ID in the RRC context during the handover.

Option 2: clarify that the “plmn-Index” is not used for multicast MRB configuration (i.e. explicit PLMN ID is used)

For option 1, it is true that the network can replace the plmn-index by the explicit PLMN ID. But the problem is that the UE would still keep the plmn-index in the UE’s RRC context. And when the UE receives the handover command, the UE would mistakenly interpret the plmn-index based on the PLMN ID list signalled in the SIB1 of the target cell. One may argue that a smart UE should already replace the plmn-index by the explicit PLMN ID before handover. However, that is not guaranteed since it is not clearly specified. Furthermore, it is not appropriated for the UE to tamper with the RRC configuration received from the network as it could lead to configuration misalignment between the UE and the network.

In fact, the motivation of introducing plmn-index in TMGI was to reduce the overhead in MCCH message for MBS broadcast. For MBS multicast, the benefit of using plmn-index is really marginal. So option 2 is a better solution to avoid the issues mentioned above.

Another related issue may be how to configure the TMGI for multicast in SNPN case. To our understanding, the NID is mainly used to identify the MBS session ID in the NG interface. As one UE is registered to only one PLMN/SNPN at a time, using the PLMN ID in the TMGI for multicast configuration is sufficient even for the SNPN case. There is no need to introduce the NID for multicast configuration in uu interface.

Given the above analysis, we think the simplest way should be to just use the explicit PLMN ID for multicast configuration.

Proposal 1: clarify that the explicit PLMN ID is used in TMGI for multicast configuration.

- Conclusion

Based on the above discussion we have the following proposal:

Proposal 1: clarify that the explicit PLMN ID is used in TMGI for multicast configuration.

- Text proposal

START OF CHANGE

### 6.3.6	MBS information elements

*****Text omitted*****

#### –	TMGI

The IE TMGI is used to identify the MBS session.

TMGI information element

-- ASN1START

-- TAG-TMGI-START

TMGI-r17 ::=                     SEQUENCE {

plmn-Id-r17                      CHOICE {

plmn-Index                       INTEGER (1..maxPLMN),

explicitValue                    PLMN-Identity

},

serviceId-r17                    OCTET STRING (SIZE (3))

}

-- TAG-TMGI-STOP

-- ASN1STOP

TMGI field descriptions

serviceId Uniquely identifies the identity of an MBS service within a PLMN. The field contains octet 3- 5 of the IE Temporary Mobile Group Identity (TMGI) as defined in TS 24.008 [38]. The first octet contains the third octet of the TMGI, the second octet contains the fourth octet of the TMGI and so on. When configured in mbs-SessionId in MRB-ToAddMod, or indicated to upper layer, the explicitValue of PLMN ID is used.

END OF CHANGE