3GPP TSG-RAN WG2 Meeting #121	R2-2301163

Athens, Greece, 27th February – 3rd March, 2023

Agenda item:	8.11.2.2

Source:	Huawei, Hisilicon

Title:	Notification and RRC state transition for multicast reception in RRC_INACTIVE

Document for:	Discussion

- Introduction

In the RAN2#120 meeting, the following agreements were made regarding PTM configuration delivery:

We will have a mixed approach and we start with the following: - When NW configures UE to continue the multicast reception in INACTIVE state, NW provides the PTM configuration for the activated multicast session via the RRC dedicated signalling, at least for the serving cell (FFS other cases). - MCCH is used in case there is a need to indicate a PTM configuration in case there is a need for change in PTM config or during mobility beyond serving cell / gNB. FFS session status change and other indications. - We assume that the UE can only receive multicast service after it joined the session. - FFS whether MCCH configuration is initially provided to the UE via dedicated signalling.

In the RAN2#119bis meeting, the following agreements were made regarding state transitions and corresponding notifications:

For both option 1 and option 2, as a baseline, group paging can be used to switch UEs receiving multicast from RRC_INACTIVE to RRC_CONNECTED, and UEs continue the multicast reception in CONNECTED. FFS if there is any potential issue if Rel-17 group paging is reused. FFS if there are other cases when UE triggers resume. FFS if MCCH can also be used in case of option 2. FFS how UE determines whether it can receive the multicast session in RRC_INACTIVE or not when the session is activated, taking into account the following solutions (can further update the descriptions if needed, and several solutions may be needed, some solutions may apply only for certain configuration options) - 1. When the multicast session is activated, UE can receive the multicast session in RRC_INACTIVE if the PTM configuration used in RRC_INACTIVE for the session is available to the UE and the UE has joined the session already (e.g., configuration provided to UE via dedicated RRC signaling or via MCCH), otherwise it goes back to RRC_CONNECTED to receive the multicast session. - 2. When the multicast session is activated, UE is indicated by group paging whether it can receive the multicast session in RRC_INACTIVE or not (detailed signaling FFS). - 3. UE is configured "whether it can receive the multicast session in RRC_INACTIVE" by dedicated signaling before UE is released. When the multicast session is activated, UE stays in RRC_INACTIVE or resumes RRC connection accordingly (detailed signaling FFS).

In this contribution, we will further analyze the state transitions for multicast reception in RRC_INACTIVE and corresponding notifications.

- Discussion

## 2.1 Notification based on the UE level MBS assistance information

In SA2 TR, there are the following conclusions on new UE level MBS assistance information:

- For UE level MBS assistance information: - The UE level MBS assistance information is an optional new parameter and set per MBS session. It is indicated by the AF to inform the network whether from the expected traffic pattern of the UE the indicated UE is preferred to be kept in the RRC Connected state even if it is able according to its radio capabilities to receive the MBS session data in RRC_INACTIVE state, e.g. a frequent talker.

- NG-RAN use the MBS session level and UE level MBS assistance information as help for the decisions on whether to enable delivery for reception in RRC_INACTIVE state for an MBS session and/or on which UEs to keep in RRC_CONNECTED or. RRC_INACTIVE state. How NG-RAN performs those decisions is up to NG-RAN implementation.

From our point of view, we think the UE level MBS assistance information is helpful for the gNB to make decision about which UEs can be released to RRC_INACTIVE for multicast reception. Specifically, if one UE is associated with the UE level MBS assistance information, the gNB shouldn't configure PTM configuration to the UE when releasing the UE to RRC_INACTIVE, e.g. when there is no data transmission or when the MBS session is inactive. In this case, when the MBS session is active or there is data transmission, the UE must go back to RRC_CONNECTED to receive the multicast. Otherwise, if one UE is not associated with the UE level MBS assistance information, the gNB can configure PTM configuration to the UE when releasing the UE to RRC_INACTIVE, to enable the UE to receive the multicast in RRC_INACTIVE.

Proposal 1: Whether the UE is allowed to receive multicast in RRC_INACTIVE depends on whether the PTM configuration is configured during RRC release.

## 2.2 Notification of MBS session activation

The following Group Paging related conclusions have been reached in the SA2 TR, with some aspects to be decided by RAN WGs.

- When the MBS session is activated, the UEs in RRC Inactive state in cells, where the MBS session is delivered allowing RRC-inactive reception, should be able to remain in RRC Inactive state for receiving the MBS session data. - For group paging, the network notifies which MBS session is to be activated. For the UE in RRC_INACTIVE state how the group paging is handled will be decided by RAN WGs. - How NG-RAN notifies the UE that the MBS session is re-activated and whether the MBS session is allowed to be received in RRC-inactive state will be decided by RAN WGs.

It was agreed that the R18 UE in RRC_INACTIVE can be informed by group paging when the session is activated. However, there are different views on whether additional indication is needed in the group paging message to inform the UE to perform the reception in RRC_INACTIVE. The notification options discussed in the RAN2#119bis meeting are extracted as below:

Option 1. When the multicast session is activated, UE can receive the multicast session in RRC_INACTIVE if the PTM configuration used in RRC_INACTIVE for the session is available to the UE and the UE has joined the session already (e.g., configuration provided to UE via dedicated RRC signaling or via MCCH), otherwise it goes back to RRC_CONNECTED to receive the multicast session. Option 2. When the multicast session is activated, UE is indicated by group paging whether it can receive the multicast session in RRC_INACTIVE or not (detailed signaling FFS). Option 3. UE is configured "whether it can receive the multicast session in RRC_INACTIVE" by dedicated signaling before UE is released. When the multicast session is activated, UE stays in RRC_INACTIVE or resumes RRC connection accordingly (detailed signaling FFS).

To our understanding, option1/3 has no essential difference in case that the PTM configuration of the serving cell is provided by RRC dedicated signaling. The presence of PTM configuration of the serving cell in RRCRelease for RRC_INACTIVE would equal to the indication of “whether it can receive the multicast session in RRC_INACTIVE”. However, since we have already agreed the mix solution, which means the UE will acquire PTM configuration by MCCH after mobility, option3 might not work after mobility. The UE cannot receive the multicast in RRC_INACTIVE without resume only based on the indication in option3. The UE still has to have a valid PTM configuration and then decides not to resume, which is basically option1.

For option1, the issue is that the network cannot indicate the UEs in RRC_INACTIVE (configured with PTM configuration during RRC release) whether to resume based on the latest load status. If the NW indicates this implicitly by MCCH, the UEs will need to wait for MCCH to see whether the previous PTM configuration is still valid and then decide whether to resume. This will further lead to the following issues:

1) Delay to resume: Since the UE doesn’t need to monitor MCCH during session deactivation for power saving, upon reception of paging, the UE has to additionally acquire the MCCH before it decides whether to resume, leading to extra delay.

2) Load for access: even though the UEs receive paging in different POs, they might have to wait for the same MCCH transmission occasion. This might cause RACH congestion due to many UEs resuming simultaneously, if the PTM configuration is absent in MCCH for the concerned service.

Therefore, option2 is a better choice, i.e., the network indicates the UEs (configured with PTM configuration during RRC release) whether to stay in RRC_INACTIVE for multicast reception via group paging.

Proposal 2: When the multicast session is activated, the NW indicates the UEs (configured with PTM configuration during RRC release) whether to stay in RRC_INACTIVE or go to RRC_CONNECTED for multicast reception via group paging.

## 2.3 Notification of MBS session deactivation

In the RAN2#119bis meeting, the following agreement was made about the notification of MBS session deactivation to the RRC_INACTIVE UEs:

If a UE is in RRC_INACTIVE and is configured to receive a multicast session in RRC_INACTIVE, the UE may be notified when the multicast session is deactivated. FFS how (e.g., informed via group paging, MCCH, or other ways).

For the notification of MBS session deactivation, the following candidate options can be considered:

Option 1: indicate the session deactivation in paging message

For this option, the UE can be notified in the group paging message with an additional indicator. When the UE receive this indicator, it can stop monitoring the PDCCH addressed by the corresponding G-RNTI. However, the legacy R17 UEs may be impacted by this paging message. Based on procedure defined in [TS 38.300], when there is temporarily no data to be sent to the UEs for an active multicast session, the gNB may also move the UE to RRC_INACTIVE. If, later, the session is deactivated by CN, the gNB will sent group paging to UEs in RRC_INACTIVE. In such scenario, the R17 RRC_INACTIVE UEs released due to temporarily no data will also resume as the new indicator of deactivation cannot be understood by the legacy UEs. In addition, the UE may miss this notification in paging message in some cases such as during cell reselection and will not know the session has been deactivated. Furthermore, this option would somehow increase the load for paging to send the service deactivation for MBS multicast. Thus, it is not desirable to indicate the session deactivation in the paging message.

Option 2: indicate the session deactivation in a new MAC CE

In LTE, the SC-PTM Stop Indication MAC CE is used to indicate that the transmission associated with a G-RNTI is stopped. However, if we design a new MAC CE to indicate the session deactivation, there is also a probability that the MAC CE is missed during cell reselection. For example, when the UE moves to a new cell, it cannot receive the MAC CE scrambled by the new G-RNTI until it gets the updated PTM configuration. Thus, it is not good choice to introduce a new MAC CE to indicate the session deactivation.

Option 3: indicate the session deactivation in MCCH message

In R17, when the MBS broadcast service stops, the UE is notified by the MCCH change notification and the UE can know whether the concerned service is stopped or not by MCCH. Similar mechanism can be reused for multicast session deactivation, i.e. the UE can learn that the session is deactivated by MCCH. For this option, the issues for option1 and option2 can be naturally avoided.

Proposal 3: MCCH is used to notify the UEs in RRC_INACTIVE about the multicast session deactivation similar as broadcast.

## 2.4 RRC Resume due to bad reception quality in RRC_INACTIVE

Generally, the quality of reception in RRC_INACTIVE is also needed to be guaranteed to meet the QoS requirement. Normally, the network can use lower MCS order and code rate for scheduling to take the UEs at the cell edge into account. However, this can’t be 100% ensured considering the varying radio environment and the lack of feedback from the RRC_INACTIVE UEs. The NW will not be able to know even if an RRC_INACTIVE UE has encountered consecutive packet loss and consequently the service is not available to the UE. There should be a fallback way for the UE to send resume request to continue the multicast reception in such failure cases, e.g. based on the link quality.

Proposal 4: the UE should be allowed to trigger RRC resume due to bad quality of multicast reception in RRC_INACTIVE.

- Conclusion

Based on the above discussion we have the following observations and proposals:

Notification based on the UE level MBS assistance information

Proposal 1: Whether the UE is allowed to receive multicast in RRC_INACTIVE depends on whether the PTM configuration is configured during RRC release.

Notification of MBS session activation

Proposal 2: When the multicast session is activated, the NW indicates the UEs (configured with PTM configuration during RRC release) whether to stay in RRC_INACTIVE or go to RRC_CONNECTED for multicast reception via group paging.

Notification of MBS session deactivation

Proposal 3: MCCH is used to notify the UEs in RRC_INACTIVE about the multicast session deactivation similar as broadcast.

RRC Resume due to bad reception quality in RRC_INACTIVE

Proposal 4: the UE should be allowed to trigger RRC resume due to bad quality of multicast reception in RRC_INACTIVE.