3GPP TSG-RAN WG2 Meeting #121                              R2-2301192

Athens, Greece, 27th February – 3rd March, 2023

Agenda item:	8.13.3

Source:	Samsung

Title:	Inter-RAT signaling based logged MDT override protection

Document for:	Discussion & Decision

# Introduction

We have an objective in Rel-18 SON/MDT [1] to handle Inter-RAT scenarios for signaling based logged MDT override protection as below.

Support of signaling based logged MDT override protection to address the scenario where the signaling based MDT is configured in E-UTRAN when [RAN2, RAN3]:

- UE reselects to NR while logged measurements are collected

- UE reselects to NR after logged measurements are collected and before uploading the logged MDT report.

In RAN2#119-e, we discussed various aspects of the above objective and the following agreements are made [2].

Agreement:

1	RAN2 confirms the valid scenario for Rel-18 inter-RAT scenario for signalling based logged MDT override protection is set by the WID:

a.Logged MDT is configured in E-UTRAN, the UE reselects to NR.

2	Rel-17 mechanism for signalling based logged MDT override protection in intra-NR scenario is the baseline for Rel-18 inter-RAT scenario.

We discussed further on the objective in RAN2#119bis-e and the following agreements are made [3].

=>	RAN2 will investigate UE and NW impacts due to EUTRA MDT configuration override protection in inter-RAT scenario realized by simultaneous LTE and NR configuration in the UE.

=> FFS if the extension of the LTE LoggedMeasurementConfiguration (with Logged MDT type indication) is needed.

=>	FFS Cross-RAT reporting for Logged MDT results (i.e. UE reports E-UTRAN logged MDT results in NR) is whether supported in R18.

=>	Intra-EUTRA case will not be considered.

We discuss further on the above in this paper.

# Discussion

A key principle of existing MDT frameworks is that there is only one RAT-specific logged measurement configuration in the UE. It means that whenever the network provides a configuration, any previously configured logged measurement configuration will be entirely replaced by the new one and logged measurements corresponding to the previous configuration will be cleared at the same time in the UE.

Observation 1: A key principle of existing MDT frameworks is that there is only one logged measurement configuration and corresponding logged measurement information in the UE.

During RAN2#119bis-e [3], we discussed a proposal to realize the override protection in inter-RAT scenario by simultaneous LTE and NR configuration in the UE. The solution proposed may work if a UE is capable of maintaining separate memory of the logged measurement configuration and information in each respective RAT signals this capability to the network.

Observation 2: Simultaneous LTE and NR configuration for MDT requires a UE to maintain separate memory of the logged measurement configuration and information in each respective RAT.

However, maintaining simultaneous LTE and NR configuration for MDT in the UE is a big deviation from above existing MDT principles, and this may cause critical UE burden. In Rel-17, we already specified the solution of signalling based logged MDT override protection in intra-NR scenario while keeping them. Such solution is quite simpler and does not cause critical UE burden i.e. UE just signal two bits (e.g. sigLogMeasConfigAvailable) to network.

Observation 3: Current specification already supports signalling based logged MDT override protection in intra-NR scenario without deviating from existing MDT principles.

Thus, we think that it is quite strange when specification supports signalling based logged MDT override protection in inter-RAT scenario only if a UE is capable of maintaining simultaneous LTE and NR configuration for MDT. If we want to handle different types of the UE, then network will need to support both the solutions (i.e. Rel-17 mechanism and simultaneous LTE and NR configuration for MDT in the UE), if the WI objectives have to be met. We don’t see any good reason to support multiple solutions for this problem.

Observation 4: Simultaneous LTE and NR configuration in the UE may force the network to support two different solutions on signalling based logged MDT override protection in inter-RAT scenario.

Based on the above observations and analysis, we propose that RAN2 doesn’t consider inter-RAT override protection by simultaneous LTE and NR configuration in the UE.

Proposal 1: RAN2 doesn’t consider inter-RAT override protection by simultaneous LTE and NR configuration in the UE.

As we already agreed to consider Rel-17 mechanism as the baseline for Rel-18 inter-RAT scenario, eNB can inform the UE with Logged MDT type indication by extending LTE LoggedMeasurementConfiguration

Proposal 2: Extend LTE LoggedMeasurementConfiguration with Logged MDT type indication.

As in intra-NR case, we can also consider UE assistance support to assist the network in preventing management based logged MDT overwriting signaling based logged MDT for inter-RAT case. A UE which has been configured with signaling based logged MDT by eNB and has moved to NR can indicate to gNB whether signaling based MDT has been configured in RRC messages like RRCSetupComplete,RRCReconfigurationComplete etc.

Proposal 3: UE informs gNB whether signaling based MDT is configured when it is configured by E-UTRA.

Since WI objective (also confirmed by RAN2) doesn’t require any RAT based prioritization, we think that it would be possible to reuse the existing NR signaling introduced in Rel 17 for the UE to inform the network whether it is configured for signaling based MDT in LTE. gNB can prevent overwrite of signalling based MDT irrespective of whether it was configured by LTE or NR.

Proposal 4: R17 NR signaling can be reused by the UE to inform gNB whether signaling based MDT is configured even when it is configured by E-UTRA.

Regarding the Cross-RAT reporting for Logged MDT results, we think that the benefits are not very obvious. This could lead to nontrivial changes in the ASN.1. So we don’t see any strong reason to support this.

Proposal 5: Cross RAT reporting is not supported.

# Conclusion

In the previous sections, we have discussed Inter-RAT signalling based logged MDT override protection and have made following observations.

Observation 1: A key principle of existing MDT frameworks is that there is only one logged measurement configuration and corresponding logged measurement information in the UE.

Observation 2: Simultaneous LTE and NR configuration for MDT requires a UE to maintain separate memory of the logged measurement configuration and information in each respective RAT.

Observation 3: Current specification already supports signalling based logged MDT override protection in intra-NR scenario without deviating from existing MDT principles.

Observation 4: Simultaneous LTE and NR configuration in the UE may force the network to support two different solutions on signalling based logged MDT override protection in inter-RAT scenario.

We made following proposals based on the observations

Proposal 1: RAN2 doesn’t consider inter-RAT override protection by simultaneous LTE and NR configuration in the UE.

Proposal 2: Extend LTE LoggedMeasurementConfiguration with Logged MDT type indication.

Proposal 3: UE informs gNB whether signaling based MDT is configured when it is configured by E-UTRA.

Proposal 4: R17 NR signaling can be reused by the UE to inform gNB whether signaling based MDT is configured even when it is configured by E-UTRA.

Proposal 5: Cross RAT reporting is not supported.

# References

- RP-221371	Revised WID: Further enhancement of data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity).

- R2-2209301 RAN2#119-e meeting report

- R2-2211101 RAN2#119bis-e meeting report.