3GPP TSG-RAN WG2 #121	R2-2301194

Athens, Greece, 27 February – 03 March 2023

Agenda item:	8.12.3

Source: 	Samsung

Title: 	TAC handling for mIAB

Document for:	Discussion and decision

# Introduction

At the RAN2#120 meeting (Toulouse, November 2022), the topic of TAC handling was singled out as a candidate for treatment at the following (i.e. the ongoing RAN2#121) meeting. This tdoc addresses this issue by identifying the key underlying problem – frequent and massive Tracking Area Updates as the mIAB node moves – and proposes solutions to alleviate the problem while bearing in mind the need to serve non mIAB enhanced UEs.

# Background and RAN3/SA2 latest

A UE can in general perform IDLE mode mobility without indicating to a new cell that it is camping on it. In order for the core network to be able to reach a UE when it is in IDLE mode, whenever a UE has entered a new Tracking Area, or TA, it will be required to announce its presence using a so-called Tracking Area Update (or TAU; sometimes also referred to as mobility registration). This allows the network to be aware within which cell the UE is located so that it remains reachable. The Tracking Area Codes (TACs) are signalled per cell. When the UE receives reads the SIB1 containing the Tracking Area, the UE will forward the Tracking Areas to upper layers (NAS), which then decide on the course of action.

In the SA2 Technical Report TR23.700-05 (v18.0.0), “Study on architecture enhancements for vehicle-mounted relays”, the following Key Issue is identified: “Key Issue #3: Efficient mobility and service continuity when served by mobile base station relay”. In the conclusion, the following is stated:

----------------- 23.700-05, V18.0.0 -----------------

8.3	Conclusions for KI#3

For KI#3, the interim conclusions are as follows:

-	The UE's mobility management is performed using the legacy mechanism as defined in the TS 23.501 [2] and TS 23.502 [5]. The UE in CM-Idle shall follow legacy procedure when detecting a TAC which is not in the TA list.

-	The TAC broadcasted by the MBSR cell(s) is configured by the Donor gNB and whether this is the same as the one of the cell of the Donor gNB serving the MBSR, or not, will be based on alignment with RAN WGs and SA2 may align specifications if SA WG2 specifications impact is identified.

NOTE:	Normative work will be based on RAN decisions.

-	Each UE connected via the MBSR may have different serving AMFs e.g., due to slicing and individual PDU sessions/QoS service flows configured. UE context handling and path switching would be handled per each individual UE.

-	No normative work for Group mobility of the UEs served by the MBSR. Alignment of specifications may be done only if RAN WGs decide to support this.

----------------- 23.700-05, V18.0.0 -----------------

Additionally, at RAN3#118 the following was agreed:

Static TAC solution is not pursued.

RAN3 assumes that dynamic TAC solution should be supported.

RAN3 to continue discussions on impacts (if any) of dynamic TAC solutions on RAN3 specs

The notion of a dynamic TAC as opposed to a static TAC means that the Tracking Area can be changed for a cell and by extension for the mobile IAB node. The purpose of this is to ensure that the Tracking Area remains fixed within a certain geographical area as the mobile IAB moves into it. (This is in contrast to static TAC, where the TAC would remain fixed for the mIAB node cell as it moves around.)

One major issue with having the TAC dynamic is that when the Tracking Area changes as the mIAB node moves, the UEs all have to perform TAUs at the same time when the cell changes its Tracking Area as the network is attempting to keep the Tracking Area fixed within a given geographical area. This might be challenging for a network to handle, causing core network congestion and potentially loss of service. An example of this problem can be seen in Figure 1:

Figure 1

One major issue with dynamic TAC is that when the Tracking Area changes as the mIAB node moves, the UEs all have to perform TAUs at the same time when the cell changes its Tracking Area as the network is attempting to keep the Tracking Area fixed within a given geographical area.

Another issue is that for the mIAB work, the following is one of the guiding principles (RP-220843): Mobile IAB-nodes should be able to serve legacy UEs. This means that the network will still have to support UEs that are not enhanced, thus any mIAB related enhancements introduced should not “break” the support of non-enhanced UEs.

Solutions on tracking areas to issue identified in Observation 1 should still be able to serve legacy UEs.

# Use of multiple TA fields: concept and use-cases

When it comes to alleviating TAU storms, one possible option (as discussed in RAN2#120) could be to have the mIAB-served UEs multi-registered in many applicable TAs. While this is certainly a possibility, the TAUs will still be needed and – while perhaps less frequent – they may be more sizable in terms of signaling amount due to UE being (re)registered to many TAs.

Another solution suggested in the last meeting was to introduce multiple tracking areas as is done for NTN. This has some issues for mIAB as 1) the trackingAreaList-r17 is only readable by Rel-17 UEs and we have to support Rel-15 UEs, and 2) the NTN solution on tracking areas rely on mapping the identifiers used over Uu, to another identifier that is used over NGAP (gNB to AMF) – this is likely not the solution that RAN3/SA2 is looking at.

The tracking area signaling is done as part of PLMN-Identity, which can be seen below:

PLMN-IdentityInfoList information element

-- ASN1START

-- TAG-PLMN-IDENTITYINFOLIST-START

PLMN-IdentityInfoList ::=       SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-IdentityInfo

PLMN-IdentityInfo ::=           SEQUENCE {

plmn-IdentityList               SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity,

trackingAreaCode                TrackingAreaCode         OPTIONAL,       -- Need R

ranac                           RAN-AreaCode             OPTIONAL,       -- Need R

cellIdentity                    CellIdentity,

cellReservedForOperatorUse      ENUMERATED {reserved, notReserved},

...,

[[

iab-Support-r16                 ENUMERATED {true}        OPTIONAL       -- Need S

]],

[[

trackingAreaList-r17            SEQUENCE (SIZE (1..maxTAC-r17)) OF TrackingAreaCode             OPTIONAL,      -- Need R

gNB-ID-Length-r17               INTEGER (22..32)         OPTIONAL       -- Need R

]]

}

-- TAG-PLMN-IDENTITYINFOLIST-STOP

-- ASN1STOP

PLMN-IdentityInfo field descriptions

cellReservedForOperatorUse Indicates whether the cell is reserved for operator use (per PLMN), as defined in TS 38.304 [20]. This field is ignored by IAB-MT.

gNB-ID-Length Indicates the length of the gNB ID out of the 36-bit long cellIdentity.

iab-Support This field combines both the support of IAB and the cell status for IAB. If the field is present, the cell supports IAB and the cell is also considered as a candidate for cell (re)selection for IAB-node; if the field is absent, the cell does not support IAB and/or the cell is barred for IAB-node.

trackingAreaCode Indicates Tracking Area Code to which the cell indicated by cellIdentity field belongs. The absence of the field indicates that the cell only supports PSCell/SCell functionality (per PLMN) or is an NTN cell.

trackingAreaList List of Tracking Areas to which the cell indicated by cellIdentity field belongs. If this field is present, network does not configure trackingAreaCode. Total number of different TACs across different PLMN-IdentityInfos shall not exceed maxTAC. This field is only present in an NTN cell.

The newly introduced trackingAreaList in PLMN-Identity offers some interesting opportunities for mIAB. One possibility would be to signal the Rel-15 trackingAreaCode and the trackingAreaList-r17 in the PLMN-Identity that can then be utilized by different UEs. As shown in Figure 2 below, if the mIAB node signals two different Tracking Area fields, one using legacy tracking area trackingAreaCode for legacy UEs and one using new field trackingAreaList-r17, this can help smoothen out the transition of moving in-between two different Tracking Areas. In this case, the mobile IAB will signal the tracking areas that are adjacent to current tracking areas.

Figure 2

The benefit of this approach becomes even clearer when a UE attaching to an mIAB node moves (together with the mIAB node) for a long time in-between multiple cells belonging to two different tracking areas (Figure 3), where without any solutions there would be a constant amount of tracking area updates.

Figure 3

The gist of the proposed approach is therefore: the mIAB broadcasts two separate TACs in two fields. The first TAC field is used by legacy UEs while the other TAC field is used by mIAB-enhanced UEs.

More specifically, when the mIAB node moves from a first fixed cell (cell#1 with TAC=TA1) to a second fixed cell (cell#2 with TAC=TA2), the two TACs broadcast by the mIAB both change from TA1 to TA2 at the boundary between cell#1 and cell#2. However, the TAC broadcast in the first field changes before (or after) the TAC broadcast in the second field. This means that the TAUs of the legacy UEs are performed before the TAUs of the enhanced UEs. This in turn distributes the TAUs over a time window, reducing the risk of network congestion.

Based on the above we propose the following:

mIAB node shall signal two different Tracking Area fields, one indicating TA for legacy UEs and one using a new field indicating TAs for mIAB-enhanced UEs.

RAN2 shall discuss UE behaviour upon reception of multiple TA fields.

# Conclusions

On the topic of dynamic TAC and its impact on RAN2, based on RAN3/SA2 agreements we provided an analysis of potential solutions and observed the following:

- One major issue with dynamic TAC is that when the Tracking Area changes as the mIAB node moves, the UEs all have to perform TAUs at the same time when the cell changes its Tracking Area as the network is attempting to keep the Tracking Area fixed within a given geographical area.

Solutions on tracking areas to issue identified in Observation 1 should still be able to serve legacy UEs.

We then made the following proposals for RAN2’s consideration:

- mIAB node shall signal two different Tracking Area fields, one indicating TA for legacy UEs and one using a new field indicating TAs for mIAB-enhanced UEs.

RAN2 shall discuss UE behaviour upon reception of multiple TA fields.