3GPP TSG-RAN WG2 #121	R2-2301196

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	8.4.2.1

Source:	Ericsson

Title:	Discussion on procedures for LTM

Document for:	Discussion, Decision

# 1	Introduction

This contribution addresses multiple stage-2 related aspects of the LTM procedures such as:

- Handling of LTM candidate cells during LTM configuration and execution

- LTM execution for intra-DU vs inter-DU and the impact on the overall signaling

- Triggering of L2 reset (MAC reset, RLC re-establishment, PDCP recovery) during LTM execution

# 2	Discussion

## 2.1	The stage-2 signalling procedures for LTM

As reference for the discussion in this contribution we use the overall stage-2 procedure in the RAN2 stage-2 running CR to TS 38.300 [4], as illustrated in Figure 1 below.

Figure 1. Signalling procedure for LTM (from 38.300 running CR)

The overall procedure for LTM is divided into these phases / procedures:

- LTM preparation: The network configures the UE with LTM candidate cells, based on measurement reports and the UE starts to perform L1 measurements on those cells.

- Early sync: The UE may perform DL synchronization and TA acquisition with candidate target cell(s) before receiving the LTM cell switch command.

- LTM execution: Based on a received L1 measurement report, LTM cell switch is triggered and an LTM cell switch command is sent to the UE which switches to the configuration of the LTM candidate cell indicated by the cell switch command and performs RA in target if TA is not available.

- LTM completion: The UE indicates successful completion of the LTM cell switch towards target cell.

As an additional reference for the discussion, also RAN3 maintains a baseline CR for stage-2 [5] as well to be included in TS 38.401 to specify the CU/DU split for the LTM functionality as depicted in Figure 2 and Figure 3 below.

Figure 2. Intra-gNB-DU L1/L2 based inter-cell Mobility (from 38.401 baseline CR)

Figure 3. Inter gNB-DU L1/L2 Triggered Mobility (from 38.401 baseline CR)

## 2.2	Handling of LTM candidate cells

### 2.2.1	Support for sequential LTM cell switch

At RAN2#119bis we agreed to support sequential LTM cell switch and to avoid RRC reconfiguration between each cell switch:

RAN2 assumes that sequential L1L2 cell change between Candidates without RRC reconfiguration can be supported.

Figure 4. Examples of sequential LTM cell switch

For the purpose of supporting the discussion here, some examples for sequential LTM cell switch are illustrated in Figure 4. The UE is first in cell A and performs an LTM cell switch (1) to cell B. Then it performs and LTM cell switch (2) to cell C followed by an LTM cell switch (3) back to the previous source cell B.

To support sequential LTM cell switch without RRC reconfiguration, the UE (at least by default) should keep the configured LTM candidate cells after LTM cell switch. It is also implicitly assumed that the UE may be configured with multiple LTM candidate cells.

The UE keeps the LTM candidate cell configurations after LTM cell switch.

The UE may be configured with multiple LTM candidate cell configurations.

Another thing is how to handle the LTM candidate cell configuration the UE has just switched to during an LTM cell switch. In the examples in Figure 4 above, e.g. after LTM cell switch (1) to cell B, how to deal with the candidate cell configuration corresponding to cell B? Considering that the UE may need later to switch (2) to cell C and then back (3) to cell B, without RRC reconfiguration, by default the UE should keep also the LTM candidate cell configuration for cell B after the LTM cell switch (2). We propose:

After performing an LTM cell switch, the UE keeps the LTM candidate cell configuration it just switched to.

Another thing is whether the “source configuration” may be an “LTM candidate cell configuration”. Illustrated in the examples in Figure 4 above, to support subsequent LTM cell switch (3) back to the cell B from cell C without RRC reconfiguration, the previous source cell B needs to be configured as an LTM candidate cell configuration while in cell C. In our view the simplest solution is to explicitly provide an LTM candidate cell configuration for cell B, e.g. while in cell A, to support this scenario. But optimizations can be discussed if needed, e.g. in the example, if the serving cell configuration while in cell B changed and avoiding reconfiguration of the cell B candidate cell configuration previously received.

The previous source SpCell after an LTM cell switch can be an LTM candidate cell by means of explicitly configure it as an LTM candidate cell (e.g. before the cell switch).

RAN2 to discuss whether further optimizations are needed to avoid explicit reconfiguration to support sequential LTM cell switch back to the previous source SpCell.

### 2.2.2	Delta and reference configuration for LTM candidate cells

During LTM preparation, the UE is configured with LTM candidate cells, containing an RRC configuration the UE applies during LTM cell switch. In the last RAN2 meeting, the following agreements have been taken regarding the support of reference and delta configuration for LTM candidate cells:

On Delta Configuration

A UE stores the reference configuration as a separate configuration.

The reference configuration is managed separately

Given these agreements, a first aspect that need to be considered is which entity is in charge of building up and providing the reference configuration to the UE. The reference configuration is a configuration that it should work with all the LTM candidate cell configuration configured but these configurations may be generated by different DUs. First, to make it more clear in the specifications what is the reference configuration for the delta configuration of LTM candidate cells, the reference configuration should be provided to the UE by the network as a separate configuration. The only network node that has the overall knowledge on what is configured at the UE is the gNB-CU. Also, RAN3 has agreed that the gNB-CU initiates the LTM candidate cell configuration, and it is natural that the gNB-CU also provides the reference configuration to the UE (e.g. during the LTM candidate cell configuration using RRC signalling). Therefore, we propose:

The reference configuration used to enable delta configuration in LTM is provided to the UE in a separate configuration by the gNB-CU.

In [6], we discuss the RRC aspects of delta and reference configuration for LTM candidate cells.

### 2.2.3	Support of measurements for configuration of LTM candidate cells

At RAN2#119-e it was agreed that

Assume that we rely on L1 measurements to trigger L1L2 mobility (still measurement for preparation could be L3, FFS)

Our understanding is that the triggering for configuring LTM at the UE can be the reception of the L3 measurement reports. This makes sense as L3 measurements already support intra-frequency and inter-frequency scenarios and thus the network may configure the UE to measure the interested frequency in anticipation that some cell(s) may become candidate cells for LTM. These are also assumed to be used as input to identify CHO target candidate cells, which to some extent, could be similar to the process of identifying LTM candidate cells.

L3 measurements already support intra-frequency and inter-frequency scenarios and thus the network may configure the UE to measure the interested frequency in anticipation that some cell(s) may become candidate cells for LTM.

Therefore, using L3 measurements for triggering the configuration of LTM it seems to be the most straightforward and simple solution. Having said that, the triggering of LTM configuration is up to network implementation and there is no requirement to use any measurements to configure LTM. Thus, we propose:

The network may determine to configure LTM candidate cells base don RRC measurement reports.

## 2.3	LTM execution and completion

### 2.3.1	Intra-DU LTM cell switch

RAN2#119-e agreed to support both intra-DU and inter-DU LTM (as already stated in the WID). At the same meeting, RAN2 also concluded:

The design for intra-DU and inter-DU L1/L2-based mobility should share as much commonality as reasonable. FFS which aspects need to be different.

While we acknowledge the above RAN2 agreement, it is anyway expected that the intra-DU case provides typically shorter latency and interruption time than the inter-DU case for LTM cell switch. This is at least clear since no RLC re-establishment is needed and only a partial MAC reset is performed for intra-DU. Further, we think that the interruption time can be even further reduced at least for intra-DU, by providing in the LTM cell switch command some additional information to the UE (e.g. information for TCI states to be activates and/or SCell to activated, etc.) related to the UE operation in the LTM target candidate cell.

Below we detail some examples of such dynamic information that may further shorten the interruption time for the intra-DU case.

One example of such dynamic information is the SCell activation/deactivation state as discussed in the previous section. Without this, the performance of LTM cell switch procedure becomes worse than a legacy L3 handover, in which the RRCReconfiguration including the reconfigurationWithSync, may include SCell state(s) for operation in the target after the handover based on the latest traffic conditions. Without this, the alternative would have been to first let the UE move to the target candidate cell upon reception of a first MAC CE with an LTM cell switch, and, only after the UE is in the target cell the UE could receive another MAC CE for activating/ deactivating SCells. Another possibility would be to rely on SCell state settings in the LTM candidate cell configuration, but as these LTM candidate cells are configured much earlier than the moment of LTM execution, traffic conditions may change and it is probably not accurate to set SCell states when LTM candidate cells are being configured. Thus, the UE should be able to receive SCell state information for the LTM target candidate cell during LTM cell switch, so the performance becomes as good as in a legacy handover, from that perspective.

Another example of dynamic information to be included in the LTM cell switch command is information enabling the UE to determine the TCI state(s) to be activated in the LTM target candidate cell during the LTM cell switch, in the case the UE does not perform random access during the LTM cell switch. Upon reception of the LTM cell switch command, the UE needs to be able to determine the TCI state to be activated in the LTM target candidate cell e.g. for transmitting on PUCCH/PUSCH and/or receive on PDCCH.

RAN1 is working on the exact content of the so called “beam indication”, and RAN2 needs to await further progress before concluding. However, one can assume that some information enabling the UE to determine the TCI state to be activated in the target LTM candidate cell is needed e.g. SSB identifier of the SSB of the TCI state to be activated, or the TCI state identifier.

UE may receive during LTM cell switch dynamic information on target candidate cell e.g. info enabling the UE to determine the TCI state to activate in the target candidate in the LTM cell switch command and SCell state (at least for intra-DU).

It is clear the in LTM we aim for short latency and interruption time during the LTM cell switch, and for that we have agreed to support LTM cell switch without the need of a random access during LTM cell switch. In that case, the UE needs to know the Timing Advance (TA) of the target cell during the cell switch and the first UL transmissions. In this scenario, the time before UE performs the first DL/UL reception/ transmission on the indicated beam of the target cell (Tfirst-data) can be further reduced by providing UL grants to the UE before it accesses the target cell. This saves one round-trip (and thus a few milliseconds) as it avoids a scheduling request.

The UL grant and dedicated preambles could either be in the LTM candidate cell configuration or sent in the LTM cell switch command. By providing this information in LTM candidate configuration we could rely on the RRC signalling. On the other hand, the related radio resources would need to be tied up in the LTM candidate cells in advance, during the LTM preparation phase, and including those cells towards the UE will not switch to, which is a waste of resources.

The time before UE performs the first DL/UL reception/ transmission on the indicated beam of the target cell can be reduced by providing dedicated RACH preambles and/or configured UL grants to the UE (either in the LTM candidate configuration or in the LTM cell switch command).

Also, for the case LTM cell switch relies on a RACH, the latency can also be reduced by providing dedicated RACH preambles to the UE beforehand. In principle, the dedicated RACH configuration for LTM cell switch could also be provided in the LTM candidate cell configuration. However, as discussed for the case of UL grant configuration(s), that would not be efficient in terms of resource usage for the network, as that would need to be configured during the LTM preparation phase, and the time the UE executes LTM may be much alter, and not to all cells for which the resources have been configured. As there are still some open question regarding the LTM cell switch relying on random access, the usage of dedicated RACH and the indication of RACH resources in the LTM cell switch command may be left FFS.

We propose:

UE may receive an UL grant in LTM cell switch command. FFS include information about dedicated RACH configuration for LTM execution.

In the discussion above about TA management and potential impact, we noted that one of the solutions studied by RAN1 is to provide the timing advance from the source cell, in the LTM cell switch command. For the intra-DU case, this value can be provided without any inter-node signalling support.

A TA value may be included in the LTM cell switch command. RAN2 needs to await RAN1 conclusion on TA management.

### 2.3.2	Inter-DU LTM cell switch

The discussion above shows that there is some dynamic information that can be provided in the LTM cell switch command, at least for the intra-DU case, which reduces the interruption time for LTM.

At least for the intra-DU case, there is dynamic information that can be provided in the LTM cell switch command, which reduces the interruption time for LTM.

If the content of the LTM cell switch command is specified to include this information, the next question is whether and how the inter-DU case can also support this.

Note that another issue is that some of the information (TCI state, SCell state, etc.) is typically determined and set by the target DU, while the LTM cell switch command is to be transmitted by the source DU. In a legacy handover this is not an issue, as the preparation and the execution happen shortly after the other.

As we also expressed in previous contributions, we see two main signalling approaches for execution of LTM cell switch, where the main difference between the approaches is in which order the serving DU transmits the LTM cell switch command to the UE and informs the target candidate DU via the CU about the execution. We are aware these are somewhat RAN3 issues, but it seems relevant to discuss them here so we understand the impact to the design of the LTM cell switch command. If we acknowledge that there is dynamic information that can be sent to the UE to speed up the LTM cell switch for the intra-DU case, the two approaches provide different level of supporting this information also for the inter-DU case.

- In approach 1 – LTM triggering without target candidate DU involvement, the serving DU first transmits the LTM cell switch command to the UE, and only after that, it indicates the execution to the target candidate DU via the CU.

- In approach 2 – LTM triggering with target candidate DU involvement, the serving DU first indicates the execution to the target DU via the CU and only after it has received a response from target candidate DU via the CU, it transmits the LTM cell switch command to the UE.

The signalling flows for these two approaches are illustrated in Figure 5 and Figure 6 below.

Figure 5. Approach 1 for LTM cell switch procedure (LTM triggering without target candidate DU involvement)

Figure 6. Approach 2 for LTM cell switch procedure (LTM triggering with target candidate DU involvement)

In approach 2, the triggering of LTM cell switch may be somewhat delayed compared to approach 1, at least if the serving DU in approach 2 waits for the response from the target candidate DU via CU before transmitting the LTM cell switch command. Some benefits of this approach are: possibility for the CU and target candidate DU to potentially reject execution; and possibility to include dynamic information in the LTM cell switch command, which should not be decided by the serving DU when it triggers LTM execution.

In approach 1, on the other hand, the information that can provided in the LTM cell switch command to the UE would be limited to the information already known by the serving DU before it makes the decision to trigger LTM cell switch, and information that is allowed to be set. We assume that in approach 1, for example, the Serving DU should not be allowed to set state related information of the target candidate DU, such as SCell state(s), which would break a basic architecture principle.

Referring to the RAN2#119-e agreement: “The design for intra-DU and inter-DU L1/L2-based mobility should share as much commonality as reasonable. FFS which aspects need to be different”, if we have an LTM cell switch command including the dynamic information (such as UL grant or TA), we think that the inter-node signalling should support the inclusion of this information also for the inter-DU case. This implies that approach 2 for execution (LTM triggering with target candidate DU involvement) should be supported.

The discussion above illustrates that there are advantages to use approach 2 execution (LTM triggering with target candidate DU involvement).

Approach 2 for execution (LTM triggering with target candidate DU involvement) facilitates the transfer of dynamic or state information, to be used in the LTM cell switch command, such as SCell state, TA and UL grant, from the target candidate DU to the serving DU.

As both approaches have their use and set of advantages and disadvantages, we think that both can quite easily be supported by the specifications and which approach to use may be up to network implementation.

The interfaces could quite easily support multiple approaches for execution of the LTM cell switch. Which approach to use may be up to network implementation.

For the execution of LTM cell switch, the following approaches are supported:

  - Approach 1 - LTM triggering without target candidate DU involvement: Serving DU decides execution of LTM and send the LTM cell switch MAC CE to the UE.

  - Approach 2 - LTM triggering with target candidate DU involvement: Serving DU decides execution of LTM, but it contacts first the target candidate DU (via the CU) before sending the LTM cell switch MAC CE to the UE.

We understand that RAN3 may also discuss these 2 approaches for the LTM cell switch execution. However, as some of the implications are very much related to what is possible or not to include in the LTM cell switch command, it is beneficial that RAN2 discuss these issues.

### 2.3.3	Triggering of L2 reset during LTM execution

RAN2 has agreed:

RAN2 assumes that sequential L1L2 cell change between Candidates without RRC reconfiguration can be supported. R2 assumes that L2 is continued whenever possible (e.g. intra-DU), without Reset, with the target to avoid data loss, and the additional delay of data recovery. R2 assumes that at L1L2 cell switch: Whether the UE performs partial or full MAC reset (FFS what partial reset is, e.g. to avoid data loss), re-establish RLC, perform data recovery with PDCP is explicitly controlled by the network. R2 assumes that this can be configured by RRC. FFS if MAC CE indication(s) is/are needed.

As RAN2 has agreed that the LTM candidate cell configuration includes the RRC IEs CellGroupConfig and RadioBearerConfig, triggering of RLC re-establishment and PDCP data recovery is performed by RRC in the LTM candidate cell configuration. And MAC reset can possibly be triggered as well by including reconfigurationWithSync (part of CellGroupConfig) in the LTM candidate cell configuration.

As also discussed in section 2.2.1 of this contribution, we propose that to support sequential LTM cell switch without RRC reconfiguration, the UE keeps the LTM candidate cell configurations after each LTM cell switch. However, by controlling L2 reset (MAC reset, RLC re-establishment and PDCP data recovery) using the LTM candidate cell configuration, it is not completely clear how to avoid RRC reconfiguration during subsequent LTM cell switch(es).

In the example illustrated in Figure 7 below, the UE is configured with LTM candidate cell configuration(s) (e.g. cell A, cell B and Cell C) while connected to a first cell (e.g. Cell X), e.g., with these cells associated to different DU(s) as in the figure below. Then, the UE performs LTM cell switch to a candidate cell (e.g. from cell X to Cell A, in the same DU as Cell X), but still considers the previous LTM candidate cells (e.g. Cell B and Cell C) as possible candidate cells for subsequent LTM executions, as agreed. Assuming Cell X and Cell A are in the same DU, such a cell switch would not require a L2 reset.

Figure 7. Example of a scenario where L2 reset may be needed or not.

Now, if the UE in cell A moves to cell B, a L2 reset is required. However, if the UE receives the LTM cell switch command (e.g. MAC CE) indicating a change to cell C (from cell B) and L2 reset, in principle, would not be required (since cell B and C are served by the same DU), though when the cell C was configured, a L2 reset indication was included (as the UE was in cell X when cell C was configured as a candidate). This results in the UE performing an unnecessary MAC reset, which increases the LTM cell switch delay. Similarly, if the UE subsequently receives an LTM switch command indicating a switch back to cell A while being in cell C, the stored LTM candidate cell configuration would not include the MAC reset (as it was not needed for the LTM switch from cell X to cell A), but in this case MAC reset would be needed since cell C and cell A are served by different DUs. This means that the LTM cell switch will fail.

Thus, at the time for LTM cell switch, the UE and the network should know whether an L2 reset would be needed or not. For that, one solution is the UE to receive from the network the cells for which L2 reset is not needed during if LTM execution, when performed from/ to them; for example, the UE is indicated during LTM configuration that no L2 reset is needed between cells A, X and between cells B, C; otherwise, L2 reset is needed. Another solution is that the triggering of the L2 reset, is instead carried in the LTM cell switch MAC CE.

What we aim for is to be able to control the MAC reset but also the RLC and PDCP per cell switch basis without RRC reconfiguration after cell switch. We see two main alternatives how to achieve this:

- Alternative 1: The MAC layer carries a single flag. When the flag is set the UE performs an overall L2 reset i.e. a full MAC reset, RLC re-establishment and PDCP data recovery for all bearers. If this flag is not set, only a partial MAC reset is performed. In this alternative, to avoid conflicts between the settings in MAC and RRC, the existing RRC flags for RLC re-establishment and PDCP data recovery are not used or the fields are ignored in LTM. In this alternative, some inter-layer interaction between MAC and upper layers (RRC or RLC/PDCP) needs to be specified for the triggering of the L2 reset. One benefit of this solution is the simplicity, while one drawback is perhaps the lack of flexibility to perform L2 reset per bearer, existing in legacy handovers, and the lack of future proofness, in case one wants this further flexibility in the future.

- Alternative 2: The RRC configuration of the LTM candidate cell includes the information about MAC reset, RLC re-establishment and PDCP data recovery, but the UE uses this information to determine whether to perform L2 reset based on the UE configuration before the switch, for example the SpCell configured before the switch. The information about L2 reset in the LTM candidate cell configuration would then be configured per source SpCell. This alternative avoids the need for RRC reconfiguration after LTM cell switch while still give flexibility to control L2 reset on source-target cell pair basis without exposing the network deployment and allowing L2 reset being configured even for intra-DU cases if that is required based on e.g. target configuration. In this alternative, a flag is also added in RRC to trigger full or partial MAC reset. This alternative would be the most flexible one as it allows RLC and PDCP to be controlled per bearer basis (as we can do today) and independent also from MAC. In this alternative, the functionality for the determination of L2 reset is kept within RRC and we reuse the existing inter-layer interactions for the actual triggering. One benefit of this solution is the flexibility, but a clear drawback is its complexity.

- Alternative 3: UE receives in the RRC Reconfiguration, during the LTM preparation, the cells for which full L2 reset is not required (i.e. partial reset) e.g. cells A, X; cells B,C, as in the example above of Figure 7. This solution is also simple, as alternative 1, but instead of defining a flag in the MAC CE for L2 reset we define it in RRC, to allow the subsequent executions. In one hand, it may not have the same flexibility as solution 2, but it is more extendable for the future than Alternative 1.

While we think both these alternatives are feasible and fulfils the objective to not requiring RRC reconfiguration after LTM cell switch, we have a slight preference for alternative 2 as this is also in line with the existing RAN2 agreement. However, if RAN2 determines the alternative 2 is not preferred (e.g. due to complexity) we suggest going for alternative 1. We propose:

RAN2 to select between these three alternatives for triggering of MAC reset, RLC re-establishment and PDCP data recovery:

Alternative 1: UE determines whether to perform L2 reset based on a single flag in the LTM cell switch command MAC CE. The flag indicates whether to perform a partial MAC reset or a full MAC reset+RLC re-establishment+PDCP data recovery.

Alternative 2: UE determines whether to perform a partial MAC reset or a full MAC reset+RLC re-establishment+PDCP data recovery based on the source SpCell and the LTM candidate cell configuration.

- Alternative 3: UE determines whether to perform a partial MAC reset or a full MAC reset+RLC re-establishment+PDCP data recovery based on lists of cells received in RRC for which reset is not needed.

### 2.3.4	Indication of UE arrival on the target cell during LTM completion

In RAN2#120, the following was agreed:

The MAC CE agreed to carry LTM related information for cell switch is used for LTM triggering of the cell switch. LTM cell switch is supervised by a timer UE arrival in the target cell need to be indicated (somehow)

It was agreed to have an indication in the target cell to confirm that the UE has completed the procedure for execution of LTM cell switch and has started using the target cell configuration. This enables the network to start the scheduling of uplink and downlink data as soon as possible (considering also somewhat unpredictable network delays).

From the sequences in section 2.1 in this contribution from the RAN3 baseline 38.401 CR [5], it is clear that RAN3 also expects an indication that triggers the target DU to transmit an ACCESS SUCCESS message to the CU.

In this section we discuss further the format and content of this indication from the UE.

In legacy L3 handover, the MSG1+MSG3 and RRCReconfigurationComplete serve the same purpose. It is not clear we can reuse the same solution for L1/L2-triggered mobility, because as discussed below, execution of L1/L2-triggered mobility without random access should also be supported and as the execution of L1/L2 inter-cell mobility is a lower layer signalling procedure, it would be awkward to have a L3 response. When the LTM cell switch is triggered by reception of a MAC CE, it is natural for the response to also be transmitted in MAC.

Even if the clearest solution would be to have an explicit indication by the UE that the execution of LTM cell switch had been successfully performed, yet another alternative is to also consider an implicit indication by the UE. However, as we have agreed to support RACH-less LTM cell switch, we can’t use MSG1 or MSG3 as implicit indication. Therefore, an implicit indication may be either the transmission of an SR request, or even the sending of UL data. But this relies on that there is some UL data that is sent immediately after the switch. And we propose in a previous section in this contribution to have the possibility to send an UL grant to the UE in the LTM cell switch command, and in this case the UE will not transmit a scheduling request.

In case of RACH-based LTM cell switch, MAC needs a trigger to transmit RACH, which may be e.g. a signalling message, uplink data or a MAC CE. In legacy L3 handover, this trigger is the RRCReconfigurationComplete.

Therefore we think that an explicit message is the preferred alternative.

We have not agreed yet on the RRC model for the LTM candidate target cell configuration. If the RRC model is an RRCReconfiguration message, it argues for transmitting an RRCReconfigurationComplete in the target.

The need to specify an explicit UL signal (e.g. a MAC CE) to the target to confirm the execution of LTM cell switch depends mainly on the RRC model.

RAN2 to discuss what the UL signal to the target cell would be to indicate the successful execution of the LTM cell switch:

  - An existing or new MAC CE

  - An RRCReconfigurationComplete message

  - An implicit indication, such as a scheduling request

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	The previous source SpCell after an LTM cell switch can be an LTM candidate cell by means of explicitly configure it as an LTM candidate cell (e.g. before the cell switch).

Observation 2	L3 measurements already support intra-frequency and inter-frequency scenarios and thus the network may configure the UE to measure the interested frequency in anticipation that some cell(s) may become candidate cells for LTM.

Observation 3	The time before UE performs the first DL/UL reception/ transmission on the indicated beam of the target cell can be reduced by providing dedicated RACH preambles and/or configured UL grants to the UE (either in the LTM candidate configuration or in the LTM cell switch command).

Observation 4	A TA value may be included in the LTM cell switch command. RAN2 needs to await RAN1 conclusion on TA management.

Observation 5	At least for the intra-DU case, there is dynamic information that can be provided in the LTM cell switch command, which reduces the interruption time for LTM.

Observation 6	Approach 2 for execution (LTM triggering with target candidate DU involvement) facilitates the transfer of dynamic or state information, to be used in the LTM cell switch command, such as SCell state, TA and UL grant, from the target candidate DU to the serving DU.

Observation 7	The interfaces could quite easily support multiple approaches for execution of the LTM cell switch. Which approach to use may be up to network implementation.

Observation 8	The need to specify an explicit UL signal (e.g. a MAC CE) to the target to confirm the execution of LTM cell switch depends mainly on the RRC model.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The UE keeps the LTM candidate cell configurations after LTM cell switch.

Proposal 2	The UE may be configured with multiple LTM candidate cell configurations.

Proposal 3	After performing an LTM cell switch, the UE keeps the LTM candidate cell configuration it just switched to.

Proposal 4	RAN2 to discuss whether further optimizations are needed to avoid explicit reconfiguration to support sequential LTM cell switch back to the previous source SpCell.

Proposal 5	The reference configuration used to enable delta configuration in LTM is provided to the UE in a separate configuration by the gNB-CU.

Proposal 6	The network may determine to configure LTM candidate cells base don RRC measurement reports.

Proposal 7	UE may receive during LTM cell switch dynamic information on target candidate cell e.g. info enabling the UE to determine the TCI state to activate in the target candidate in the LTM cell switch command and SCell state (at least for intra-DU).

Proposal 8	UE may receive an UL grant in LTM cell switch command. FFS include information about dedicated RACH configuration for LTM execution.

Proposal 9	For the execution of LTM cell switch, the following approaches are supported:

a.	Approach 1 - LTM triggering without target candidate DU involvement: Serving DU decides execution of LTM and send the LTM cell switch MAC CE to the UE.

b.	Approach 2 - LTM triggering with target candidate DU involvement: Serving DU decides execution of LTM, but it contacts first the target candidate DU (via the CU) before sending the LTM cell switch MAC CE to the UE.

Proposal 10	RAN2 to select between these three alternatives for triggering of MAC reset, RLC re-establishment and PDCP data recovery:  Alternative 1: UE determines whether to perform L2 reset based on a single flag in the LTM cell switch command MAC CE. The flag indicates whether to perform a partial MAC reset or a full MAC reset+RLC re-establishment+PDCP data recovery.  Alternative 2: UE determines whether to perform a partial MAC reset or a full MAC reset+RLC re-establishment+PDCP data recovery based on the source SpCell and the LTM candidate cell configuration.

Alternative 3: UE determines whether to perform a partial MAC reset or a full MAC reset+RLC re-establishment+PDCP data recovery based on lists of cells received in RRC for which reset is not needed.

Proposal 11	RAN2 to discuss what the UL signal to the target cell would be to indicate the successful execution of the LTM cell switch:

a.	An existing or new MAC CE

b.	An RRCReconfigurationComplete message

c.	An implicit indication, such as a scheduling request

# 4	References

R2-2211154, LS on L1 intra- and inter- frequency measurement and configurations for L1/L2-based inter-cell mobility, RAN1, 3GPP TSG-RAN WG2 #120, Toulouse, France, 14 – 18 November 2022

R2-2212988, Reply LS on L1 measurement and configurations for LTM, RAN2, 3GPP TSG-RAN WG2 #120, Toulouse, France, 14 – 18 November 2022

R3-226829, Reply LS on L1 intra- and inter- frequency measurement and configurations for L1/L2-based inter-cell mobility, RAN3, 3GPP TSG-RAN WG3 #118, Toulouse, 14-18 November 2022

R2-2213332, 38.300 running CR for introduction of NR further mobility enhancements, MediaTek Inc., 3GPP TSG-RAN WG2 Meeting #120 Electronic, Toulouse, France, November 14 – 18, 2022

R3-226930, Introduction of L1/L2 inter-cell mobility, Huawei, Ericsson, Nokia, Nokia Shanghai Bell, 3GPP TSG-RAN WG3 Meeting #118, Toulouse, France, 14-18 November 2022

R2-2301197, Discussion on RRC aspects for LTM, Ericsson, 3GPP TSG-RAN WG2 #121, Athens, Greece, February 27th – March 3rd, 2023

R2-2301199, Partial and full MAC reset in LTM, Ericsson, 3GPP TSG-RAN WG2 #121, Athens, Greece, February 27th – March 3rd, 2023