3GPP TSG-RAN WG2 #121	R2-2301199

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	8.4.2.3

Source:	Ericsson

Title:	Partial and full MAC reset in LTM

Document for:	Discussion, Decision

# 1	Introduction

RAN2#119bis-e made these agreements:

R2 assumes that at L1L2 cell switch: Whether the UE performs partial or full MAC reset (FFS what partial reset is, e.g. to avoid data loss), re-establish RLC, perform data recovery with PDCP is explicitly controlled by the network. R2 assumes that this can be configured by RRC. FFS if MAC CE indication(s) is/are needed. For UE processing, the following (not exhaustive) is assumed to be performed after receiving the cell switch command: - MAC/RLC reset (when configured) - RF retuning (e.g. needed for inter-frequency), baseband retuning

Then in RAN2#120, Partial MAC reset was further discussed in Offline#33, discussion summarized in [1]. Based on online discussion, the following was agreed:

RAN2 to have the mindset to have a common design for partial MAC reset for different cell change cases in intra-DU scenario (as far as reasonable) The summary in [R2-2213336] could be considered as the starting point for partial reset in intra-DU.

In this contribution we continue the discussion on the MAC actions at partial and full MAC reset for LTM.

# 2	Discussion

According to the agreement from RAN2#119bis meeting, LTM will support both full and partial MAC reset.

LTM supports both full and partial MAC reset.

So far there was however no detailed discussion of what actions the UE performs in these so called “full” and “partial” MAC reset for LTM, so this is discussed in the following sections of this contribution. We also include a section to discuss whether a LTM cell switch with no MAC reset is feasible or not.

## 2.1	“Full” MAC reset for LTM

The starting point for the “full” MAC reset for LTM should be the MAC reset triggered at reconfiguration with sync (e.g. handover), in which all MAC counters and timers are reset and HARQ buffers are flushed (see TS 38.321, section 5.12 for details). The MAC reset provides a clean state for applying the new configuration (like the target cell configuration in a handover) and allows moving the MAC termination point on the network side as part of the handover. Since LTM shall support both intra-DU and inter-DU operation, there appears to be a need to support also full MAC reset for LTM. In addition, supporting LTM with full MAC reset will allow full flexibility when it comes to MAC parameters that can be reconfigured at mobility.

Full MAC reset as in reconfiguration with sync is needed in order to reset MAC timers and counters, allowing that these can be reconfigured as part of the LTM cell switch target configuration.

There are however a couple characteristics for LTM that have not been extensively discussed yet and could call for exceptions from MAC reset triggered by reconfiguration with sync:

- Beam failure detection/recovery. Full MAC reset involves the reset of beamFailureDetectionTimer and BFI_COUNTER. The beamFailureDetectionTimer is configured per cell or per BFD-RS set (if there are two configured). It is restarted and BFI_COUNTER is incremented at each beam failure instance indication for a BFD-RS. When beamFailureDetectionTimer expires or any of the reference signals used for beam failure detection is reconfigured by upper layers or by the BFD-RS Indication MAC CE associated with a BFD-RS set, then BFI_COUNTER is set to 0. If BFI_COUNTER => beamFailureInstanceMaxCount, BFD is declared, and RA is triggered on the SpCell. For LTM, since the UE may already have a beam associated with the target candidate cell, one could argue that beamFailureDetectionTimer and BFI_COUNTER should not be reset in this case. On the other hand, if the target configuration includes a reconfiguration of the reference signals used for beam failure detection, then a reset may be needed. Furthermore, for the case where LTM involves random access to determine the TA with target cell, the reset is motivated as the random access involves a new beam selection anyway. However, the actions at MAC reset will also depend on whether BFD/BFR is only performed on source cell or also candidate target cells for LTM. We have a discussion on this in [2]. As BFD/ BFR for LTM has not been discussed yet, we propose to keep the handling of BFD related counters and timers FFS for the time being.

- TA management. In case the UE has already established TA with an LTM candidate cell (which may become a target cell), in order to support LTM without random access, it is not clear whether the timeAlignmentTimer should be handled as in legacy during a full MAC reset. RAN1 is still discussing LTM without random access, so we propose to keep this FFS until a solution for how the TA of candidate target LTM cells is established and maintained is agreed.

Considering the above points, we propose the following:

Full MAC reset for LTM includes the same actions as MAC reset at reconfiguration with sync, except that the handling of time alignment, BFD and BFR is left FFS.

## 2.2	Partial MAC reset for LTM

In addition to the full MAC reset, RAN2 has agreed to support also a partial MAC reset, in which data loss is minimized by not resetting HARQ. So far, the following has been agreed:

In RAN2#119:

R2 assumes that L2 is continued whenever possible (e.g. intra-DU), without Reset, with the target to avoid data loss, and the additional delay of data recovery.

In RAN2#120:

RAN2 to have the mindset to have a common design for partial MAC reset for different cell change cases in intra-DU scenario (as far as reasonable) The summary in [R2-2213336] could be considered as the starting point for partial reset in intra-DU.

According to the agreements so far, the purpose of the partial MAC reset is to avoid data loss at LTM. It is also agreed to target a common design for partial MAC reset, as far as reasonable. Another aspect to consider for the partial MAC reset is that whatever exception is added, i.e. whatever counter or timer that is not reset, also means that the value of this counter or timer cannot be reconfigured in the LTM target configuration. The reset of timers and counters in MAC reset is there to clear the old state before a new configuration can be applied. This means that for each timer or counter that is not reset during partial MAC reset, it introduces restrictions to the cases where partial MAC reset can be applied.

Actions skipped during partial MAC reset for LTM may limit the cases in which partial MAC reset can be applied.

With the above in mind, it seems reasonable that the starting point for discussing partial MAC reset is a full MAC reset, and then actions to be skipped are discussed case by case, with the focus to minimize data loss at LTM. In other words, the focus should be skipping actions that contribute to data loss and/or interruption at LTM, and other actions should preferably be kept.

Actions to be skipped during partial MAC reset for LTM compared to full MAC reset for LTM should contribute towards reducing data loss and/or interruption time.

In RAN2#120, the actions of partial MAC reset were discussed in Offline#033 and the outcome summarized in [1]. Though no agreements were made regarding the actions for partial MAC reset, it was agreed that the summary in [1] should be the starting point for partial MAC reset. Here we continue the discussion on the MAC actions and provide our input case by case.

HARQ

The main contributor to data loss during MAC reset is the reset of UL/DL HARQ and flushing of the HARQ buffers. At least in some scenarios, e.g. in intra-DU LTM cell switch, it is feasible to keep the UL/DL HARQ processes, soft buffers and not reset NDIs for the cases where the network termination point is not moved. This will significantly reduce the data loss during LTM.

UL/DL HARQ processes, soft buffers, NDIs and HARQ related timers are not reset during partial MAC reset for LTM.

BSR

At MAC reset in reconfiguration with sync, any triggered BSR (i.e. with the source cell) is cancelled. In legacy L3 handover, RLC is re-established and HARQ buffers are reset, meaning that all data buffered on RLC and MAC is lost. This means that when data is again passed down to RLC and MAC in the target cell, this new data will trigger a BSR in the target. Assuming however that the HARQ buffers are not flushed for partial MAC reset, there is no such automatic trigger for BSR as the UE switches to the target cell. Therefore, since the L2 buffers are not flushed, it makes sense not to cancel any triggered BSR during partial MAC reset, since that may just delay the time until next BSR can be provided. Same goes for timers related to BSR reporting, they should not be stopped at partial MAC reset.

BSR is not cancelled and related timers not stopped during partial MAC reset for LTM.

Time alignment

With the same reasoning as for the full MAC reset in section 2.1, details around the handling of the timeAlignmentTimers during partial MAC reset will need to wait until RAN1 makes some progress on LTM without random access and the TA establishment/ maintenance procedures. This will depend on at which point of time the TA for candidate targets are provided to the UE. Thus. the handling of the timeAlignmentTimers should be left FFS until RAN1 has concluded.

Handling of timeAlignmentTimers during partial MAC reset for LTM is left FFS pending RAN1 progress.

Beam Failure Detection/Recovery (BFD/BFR)

With the same reasoning as for the full MAC reset in section 2.1, details around the handling of counters and timers related to beam failure detection and recovery need to wait until progress is made on how BFD/BFR is performed during LTM.

FFS whether any triggered BFR is cancelled, related timers stopped and counters reset during partial MAC reset for LTM.

Bj

Bj is kept per logical channel and used by the UE to prioritize between LCHs during uplink transmissions. Bj is increased according to PBR until bucket size is reached. Bj is decreased according to total size of MAC SDUs served to logical channel j at each Tx. At MAC reset, HARQ buffers are flushed and also Bj is initialized to 0 to reset scheduling history. At partial MAC reset, since the HARQ buffers are kept, Bj could also be kept. On the other hand, resetting Bj will not impact the data loss or interruption time during LTM, as it only affects which logical channels that are selected for UL transmission by the UE. Furthermore, there will likely be a short interruption also during LTM, even if shorter compared to legacy L3 handover, Bj could also be reset during partial MAC reset. Resetting Bj will also allow the network to reconfigure prioritisedBitRate and bucketSizeDuration for each logical channel during LTM with partial MAC reset. In summary, we don’t have a strong view whether to reset Bj or not during partial MAC reset, but since there is no impact on data loss or interruption, we have a slight preference to reset Bj, in order not to restrict the possibility for reconfiguration of prioritisedBitRate and bucketSizeDuration during LTM.

Bj is reset during partial MAC reset for LTM.

Recommended bit rate query

The recommended bit rate procedure is used to provide the MAC entity with information about the bit rate which the gNB recommends. The query procedure is used by the UE to request the gNB to indicate the recommended bit rate for a specific logical channel and a specific direction. The query indicates the desired bit rate. Considering that the network termination point may not change during LTM with partial MAC reset (intra-DU case), it may seem unnecessary to cancel any triggered query procedure. However, since the recommended bit rate may differ from one serving cell to the other, depending on the cell load, it would be a reason to cancel any triggered procedure to avoid that the UE operates on a value relevant for the source cell. In addition, cancelling any triggered recommended bit rate query procedure is not expected to cause any data loss or data interruption, and so the action can be kept for partial MAC reset.

Any triggered Recommended bit rate query procedure is cancelled during partial MAC reset for LTM.

The following set of actions can all be regarded as cell specific, which motivates to keep the actions during partial MAC reset during LTM, since there serving PCell is changed from source to target.

RACH

During MAC reset, the UE cancels any ongoing random access, flushes MsgA/Msg3 buffer, discards contention free random access resources and releases any temporary C-RNTI. Since random access configuration and resources for contention free random access are cell specific, it makes sense to keep the actions during partial MAC reset.

Any ongoing random access is stopped, MsgA/Msg3 buffer flushed, contention free random access resources discarded, timers stopped and any temporary C-RNTI is released during partial MAC reset for LTM.

Power Headroom Reporting

During MAC reset, the UE cancels any ongoing power headroom reporting procedure. Since the source and the target cells may have different path loss, it makes sense to cancel any ongoing power headroom reporting procedure and stop related timers also upon LTM with partial MAC reset.

Any triggered power headroom reporting procedure is cancelled and related timers stopped during partial MAC reset for LTM.

Scheduling Request

The scheduling request (SR) is a request for scheduling resources, which are cell specific, so it makes sense to cancel any ongoing SR procedure and stop related timers also upon LTM with partial MAC reset.

Any triggered scheduling request reporting procedure is cancelled and related timers stopped during partial MAC reset for LTM.

Listen Before Talk

Listen before talk (LBT) is a channel sensing operation that is used for unlicenced band operation to ensure fairness with other users of the unlicenced band. As such, the operation is cell specific, and at LTM cell change it makes sense to cancel any ongoing consistent LBT failure and reset all LBT_COUNTERS and stop related timers.

Any triggered consistent LBT failure is cancelled and all LBT_COUNTERS reset and related timers stopped during partial MAC reset for LTM.

Configured grants

Configured grants are cell specific, so when the PCell is changed from source to target it makes sense that the UE cancels any triggered configured uplink grant confirmation and stops any related timers, same as for normal MAC reset.

Any triggered configured uplink grant confirmation is cancelled and related timers stopped during partial MAC reset for LTM.

## 2.3	No MAC reset during LTM cell switch?

Based on the discussion in previous section on the different actions for partial MAC reset, is seems an LTM with no MAC reset is not feasible, since even if the network L2 protocol termination point does not change, there are several MAC procedures that are affected by the fact that the serving cell is changed, and thus need a reset. It therefore does not seem feasible to support LTM with no MAC reset, and there is no need to support this. This is also in line with the agreement from last meeting to have a common design for the partial MAC reset. So based on the proposals in this contribution there would be support for LTM with full MAC reset and LTM with partial MAC reset.

LTM cell switch with no MAC reset is not supported

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	LTM supports both full and partial MAC reset.

Observation 2	Full MAC reset as in reconfiguration with sync is needed in order to reset MAC timers and counters, allowing that these can be reconfigured as part of the LTM cell switch target configuration.

Observation 3	Actions skipped during partial MAC reset for LTM may limit the cases in which partial MAC reset can be applied.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Full MAC reset for LTM includes the same actions as MAC reset at reconfiguration with sync, except that the handling of time alignment, BFD and BFR is left FFS.

Proposal 2	Actions to be skipped during partial MAC reset for LTM compared to full MAC reset for LTM should contribute towards reducing data loss and/or interruption time.

Proposal 3	UL/DL HARQ processes, soft buffers, NDIs and HARQ related timers are not reset during partial MAC reset for LTM.

Proposal 4	BSR is not cancelled and related timers not stopped during partial MAC reset for LTM.

Proposal 5	Handling of timeAlignmentTimers during partial MAC reset for LTM is left FFS pending RAN1 progress.

Proposal 6	FFS whether any triggered BFR is cancelled, related timers stopped and counters reset during partial MAC reset for LTM.

Proposal 7	Bj is reset during partial MAC reset for LTM.

Proposal 8	Any triggered Recommended bit rate query procedure is cancelled during partial MAC reset for LTM.

Proposal 9	Any ongoing random access is stopped, MsgA/Msg3 buffer flushed, contention free random access resources discarded, timers stopped and any temporary C-RNTI is released during partial MAC reset for LTM.

Proposal 10	Any triggered power headroom reporting procedure is cancelled and related timers stopped during partial MAC reset for LTM.

Proposal 11	Any triggered scheduling request reporting procedure is cancelled and related timers stopped during partial MAC reset for LTM.

Proposal 12	Any triggered consistent LBT failure is cancelled and all LBT_COUNTERS reset and related timers stopped during partial MAC reset for LTM.

Proposal 13	Any triggered configured uplink grant confirmation is cancelled and related timers stopped during partial MAC reset for LTM.

Proposal 14	LTM cell switch with no MAC reset is not supported

# 4	References

R2-2213336, Potential Partial MAC Reset for intra-DU LTM, vivo

R2-2301197, Discussion on RRC aspects for LTM, Ericsson