3GPP TSG-RAN2 Meeting #121	R2-2301203

Athens, Greece, 27 February – 3 March 2023

Agenda Item:	6.2.2 CP corrections

Source: 	Ericsson

Title:  	MBS and NPN

Document for:	Discussion and Decision

# Introduction

In this contribution the open issues with MBS and NPN are discussed further.

# Background

RAN2#120

MBS and NPNs were discussed during RAN2#120 [1, 2], and it was agreed:

- MBS should be supported within SNPN. FFS if some change is needed. FFS CAG (offline to discuss FFSes - Nokia)

- RAN2 specs allow to use plmn-Index to indicate PLMN+NID (=SNPN ID).

- RAN2 specs allow to use plmn-Index to indicate CAG.

- FFS whether this works without specification changes for all the required scenarios.

- FFS whether/what changes are needed for inter-node messages

Furthermore in the reply LS to RAN3 [3] RAN2 replied:

Question (3): Is there any significant limitation from RRC point of view if the TMGI as received by the 5GC contains a PLMN/SNPN ID not broadcast in SIB1?

Answer to Question (3): From RRC point of view there is no restriction that the TMGIs for the broadcast services that UE is interested to receive or is receiving should contain PLMN ID broadcasted in SIB1. RAN2 has agreed that MBS should be supported within SNPN in Rel-17, and it is FFS whether some specification change is needed (e.g., changes related to TMGI and SNPN ID).

SA2

The SA2 WID objective for eNPNs, among others, is to introduce equivalent SNPNs [4,5]:

1.	Support for enhanced mobility by enabling support for idle and connected mode mobility between SNPNs without new network selection, including:

a.	UE and AMF support of equivalent SNPN list in NAS.

b.	NG-RAN and AMF support of equivalent SNPNs in NGAP.

c.	UE/NG-RAN/AMF take equivalent SNPN list into consideration, for supporting relevant functions, e.g. idle/connected mode mobility.

NOTE 1:	The above excludes support for equivalent SNPNs, with different SNPN IDs, within an RA.

# Discussion

## PNI-NPN

The plmn-Index was introduced in Rel-16 [6]:

Agreements via email (from [104][PRN]):

- UAC parameter set for a PNI-NPN is selected based on the PLMN ID of PNI-NPNs. There is no need to broadcast CAG ID specific UAC parameter sets.

- The PNI-NPNs belonging to the same PLMN have a common (shared) index value.

The UAC parameter were not needed per CAG and the PNI-NPN identity is not included in MSG5:

UAC-BarringPerPLMN ::=              SEQUENCE {

plmn-IdentityIndex                  INTEGER (1..maxPLMN),

…

plmn-IdentityIndex Index of the PLMN or SNPN across the plmn-IdentityInfoList and npn-IdentityInfoList fields included in SIB1.

RRCResumeComplete-IEs ::=               SEQUENCE {

selectedPLMN-Identity                   INTEGER (1..maxPLMN)    OPTIONAL,

…

selectedPLMN-Identity Index of the PLMN selected by the UE from the plmn-IdentityInfoList or npn-IdentityInfoList fields included in SIB1.

RRCSetupComplete-IEs ::=            SEQUENCE {

selectedPLMN-Identity               INTEGER (1..maxPLMN),

…

selectedPLMN-Identity Index of the PLMN or SNPN selected by the UE from the plmn-IdentityInfoList or npn-IdentityInfoList fields included in SIB1.

But a separate intra-/inter-frequency cells list can be configured per NPI-NPN identity:

IntraFreqCAG-CellListPerPLMN-r16 ::= SEQUENCE {

plmn-IdentityIndex-r16               INTEGER (1..maxPLMN),

cag-CellList-r16                     SEQUENCE (SIZE (1..maxCAG-Cell-r16)) OF PCI-Range

…

InterFreqCAG-CellListPerPLMN-r16 ::= SEQUENCE {

plmn-IdentityIndex-r16              INTEGER (1..maxPLMN),

cag-CellList-r16                    SEQUENCE (SIZE (1..maxCAG-Cell-r16)) OF PCI-Range

…

The TMGI uniquely identifies an MBS session, except for SNPN where the NID is needed as well (23.247):

TMGI (Temporary Mobile Group Identity) is defined in TS 23.003 [12] and is used to be able to identify a broadcast MBS Session or a multicast MBS session.

In SNPN (Stand-alone Non-Public Network), TMGI is used together with NID (Network Identifier) defined in TS 23.003 [12] to identify an MBS Session.

For NPI-NPNs it is thus implicitly specified that the CAG is not needed. It might be beneficial to have a note in 23.247 about NPI-NPNs to avoid any confusion.

In SNPNs the pairs <TMGI1, NID1> and <TMGI1, NID2> can indicate different sessions. While in NPI-NPNs <TMGI1, CAGx> and <TMGI1, CAGy> always indicate the same session. So the CAG identity is not needed to identify an MBS session:

Proposal 1: No specification changes are needed to support NPI-NPNs in MBS.

## SNPN

The TMGI is included in the following messages on the Uu interface:

- MBS broadcast:

  - MBSBroadcastConfiguration message on MCCH (mbs-SessionInfoList IE)

  - MBSInterestIndication message (mbs-ServiceList IE)

- MBS multicast:

  - Paging message (PagingGroupList IE)

  - RRCReconfiguration message (mrb-ToAddMod IE)

In Uu signalling the plmn-Index uniquely identifies an SNPN (NPN Identity) and the NID is not explicitly signalled, as specified in 38.331:

npn-IdentityInfoList The npn-IdentityInfoList is used to configure a set of NPN-IdentityInfo elements. Each of those elements contains a list of one or more NPN Identities and additional information associated with those NPNs. The total number of PLMNs (identified by a PLMN identity in plmn -IdentityList), PNI-NPNs (identified by a PLMN identity and a CAG-ID), and SNPNs (identified by a PLMN identity and a NID) together in the PLMN-IdentityInfoList and NPN-IdentityInfoList does not exceed 12, except for the NPN-only cells. A PNI-NPN and SNPN can be included only once, and in only one entry of the NPN-IdentityInfoList. In case of NPN-only cells the PLMN-IdentityList contains a single element that does not count to the limit of 12 and the cellIdentity of the first entry of the PLMN-IdentityInfoList is set to the same value as the cellIdentity-r16 of the first entry of the NPN-IdentityInfoList. The NPN index is defined as B+c1+c2+…+c(n-1)+d1+d2+…+d(m-1)+e(i) for the NPN identity included in the n-th entry of NPN-IdentityInfoList and in the m-th entry of npn-Identitylist within that NPN-IdentityInfoList entry, and the i-th entry of its corresponding NPN-Identity, where - B is the index used for the last PLMN in the PLMN-IdentittyInfoList; in NPN-only cells B is considered 0; - c(j) is the number of NPN index values used in the j-th NPN-IdentityInfoList entry; - d(k) is the number of NPN index values used in the k-th npn-IdentityList entry within the n-th NPN-IdentityInfoList entry; - e(i) is - i if the n-th entry of NPN-IdentityInfoList entry is for SNPN(s); - 1 if the n-th entry of NPN-IdentityInfoList entry is for PNI-NPN(s).

For example plmn-Index 6 indicates SNPN identified by <PLMN x1, NID2>. The plmn-Index does not uniquely indicate an PNI-NPN identity:

Observation 1: In Uu signalling the plmn-Index is used to uniquely identify an SNPN (NPN Identity).

A TMGI+NID that is not included in SIB1 cannot be signalled, but there is also no need to do so:

Observation 2: In case the SNPN indicated by <PLMN identity, NID> is not present in SIB1 the MBS session indicated by <TMGI (PLMN identity, Session ID), NID> is not supported on Uu interface (MBS multicast: Paging, RRCReconfiguration and MBS broadcast: MCCH, MBSInterestIndication).

## plmn-Index in HandoverPreparationInformation message

The plmn-Index (4 bits) is a significant Uu optimization when used instead of the full TMGI (48 bits), and it is needed in case of SNPNs [7]:

Observation 3: It is beneficial to use the plmn-Index in Uu signalling and in case of SNPNs the plmn-Index is the only option.

The plmn-Index can be included in the HandoverPreparationInformation message when:

- UE included mbs-ServiceList IE in MBSInterestIndication message

- A multicast MRB is configured in the AS-Config (mrb-ToAddMod IE)

RAN2 agreed that in case the UE included the plmn-Index in the TMGI in the mbsInterestIndication message, that the gNB would substitute this with the PLMN ID:

AS-Context field descriptions

mbsInterestIndication Includes the information last reported by the UE in the NR MBSInterestIndication message, where the plmn-Index (if included by the UE in tmgi) is replaced by the PLMN ID, if any.

If the Broadcast PLMN Identity Info List NR IE in the Served Cell Information NR IE is included in the XN SETUP REQUEST message, then the plmn-index could be used in the target cell. In some network deployments the plmn-index may refer to the same PLMN ID, but that is something that we cannot rely on from a specification perspective.

The Broadcast SNPN ID List IE and Broadcast PNI-NPN ID Information, i.e. copies of the npn-IdentityInfoList in SIB1, are distributed to neighbour cells when the source cell supports NPN and Xn is configured. But the required information is available at the target gNB in all cases, i.e. thus the source gNB needs to signal the PLMN ID explicitly.

Observation 4: The target gNB is not aware of the SIB1 configuration on the source gNB in all cases, and therefore the plmn-Index cannot be used in the HandoverPreparationInformation message to indicate the PLMN and SNPN.

The UE is required to store the configuration (including multicast MRB) in the AS-context. The UE is also required to have a valid SIB1 stored, including the PLMN/NPN info. One can perhaps argue whether it is stored in the AS-context or not, but the UE also stores the PLMN ID and NID.

In our understanding the UE does not act on the stored plmn-Index, but on the PLMN ID (+ NID):

- The UE does not trigger the MBSInterestIndication message on the target cell, when a different plmn-Index is pointing to the same PLMN ID or PLMN ID + NID in the source and target cell, because in such case the UE did not change its mind, but only a different SIB1 configuration is used in source and target cell.

- When the UE includes the plmn-Index in the MBSInterestIndication message, then the UE does not show interest in the plmn-Index as such, but in the TMGI that it represents according to the SIB1 configuration.

- The UE does not use the plmn-Index included in the mrb-ToAdMod but the TMGI to identify the MRB.

We are not sure if further clarification is needed:

Proposal 2: RAN2 to discuss if further clarification for the use of plmn-Index is needed.

In case Xn is configured the serving NID in the NPN Mobility Information in the HANDOVER REQUEST indicates the registered SNPN (PLMN ID + NID) of the UE. And the serving NID is included in the handover request between SNPNs, otherwise the handover might fail.

For SNPNs in Rel-17 the following can be assumed:

- The UE is only interested in MBS broadcast services that belong to the registered SNPN

- The TMGI of the multicast MRB belongs to the registered SNPN

In our view the serving NID in the NPN Mobility Information in the HANDOVER REQUEST can be used by the target, in case the plmn-Index is included in the HandoverPreparationInformation message

Proposal 3: The gNB replaces the plmn-Index in the mrb-ToAddMod, if included, in the HandoverPreparationInformation message and the serving NID in the NPN Mobility Information is used in case the PLMN ID indicates an SNPN.

A TP is provided in the Annex for discussion and agreement.

## Equivalent PLMNs and SNPNs

In Rel-18 SA2 will introduce equivalent SNPNs [4,5] which means that the UE can re-select to an equivalent SNPNs (different <PLMN ID, NID>) without NAS involvement. It is understanding that both PLMN ID and/or NID can be different in an equivalent SNPN, but that is perhaps something that needs to be clarified.

A UE can be configured by NAS with an equivalent PLMN list (38.304 Equivalent PLMN list). The UE can consider a cell suitable to camp on when the PLMN ID in SIB1 belongs to the equivalent PLMN list, and there is no inter-action needed between AS cell re-selection and NAS PLMN selection. A similar principle applies to the equivalent SNPNs in Rel-18 [8,9].

Concerning MBS sessions and equivalent PLMNs the following needs to be clarified:

- The same MBS broadcast service(s) are provided in the equivalent PLMNs and an MBS broadcast service is identified by the same TMGI (or TMGI+NID in case of SNPNs) across the equivalent PLMNs.

- The same MBS broadcast service(s) are provided in the equivalent PLMNs but an MBS broadcast service may use a different TMGI (or TMGI+NID in case of SNPNs) across the equivalent PLMNs.

- The same MBS broadcast service(s) may not be provided in the equivalent PLMNs and an MBS broadcast service may have the same or different TMGI (or TMGI+NID in case of SNPNs) across the equivalent PLMNs.

It is proposed to send an LS to SA2 and RAN3 concerning those questions. Based on the reply, RAN2 can discuss further whether any signalling changes are needed:

Proposal 4: Ask SA2 and RAN3 for clarification of MBS sessions across equivalent PLMNs and SNPNs.

Potentially a NID-list is added to the HandoverPreparationInformation message or NPN Mobility Information. See Appendix for information.

# Summary

RAN2 is kindly asked to discuss MSB and NPNs:

Proposal 1: No specification changes are needed to support NPI-NPNs in MBS.

Observation 1: In Uu signalling the plmn-Index is used to uniquely identify an SNPN (NPN Identity).

Observation 2: In case the SNPN indicated by <PLMN identity, NID> is not present in SIB1 the MBS session indicated by <TMGI (PLMN identity, Session ID), NID> is not supported on Uu interface (MBS multicast: Paging, RRCReconfiguration and MBS broadcast: MCCH, MBSInterestIndication).

Observation 3: It is beneficial to use the plmn-Index in Uu signalling and in case of SNPNs the plmn-Index is the only option.

Observation 4: The target gNB is not aware of the SIB1 configuration on the source gNB in all cases, and therefore the plmn-Index cannot be used in the HandoverPreparationInformation message to indicate the PLMN and SNPN.

Proposal 2: RAN2 to discuss if further clarification for the use of plmn-Index is needed.

Proposal 3: The gNB replaces the plmn-Index in the mrb-ToAddMod, if included, in the HandoverPreparationInformation message and the serving NID in the NPN Mobility Information is used in case the PLMN ID indicates an SNPN.

Proposal 4: Ask SA2 and RAN3 for clarification of MBS sessions across equivalent PLMNs and SNPNs.

# References

- R2-2211974 SNPN and MBS broadcast, Nokia, DISC, RAN2#120

- R2-2213330 Report of [AT120][607][MBS-R17] FFSes for MBS in NPN scenario, Nokia, Report, RAN2#120

- R2-2213109 Reply LS on resource efficiency for MBS reception in RAN sharing scenario, LS out, To:RAN3 Cc:SA2, RAN2#120

- SP-221125, Enhanced support of Non-Public Networks Phase 2 (eNPN_Ph2), Revised WID, SA#98E

- SP-221087, CRs on eNPN_Ph2, Rel-18, Source: SA WG2, SA#98E

- R2-2005794, Offline discussion 104: PRN - RRC open issues - first round, Report, Nokia, RAN2#110-e

- R2-2211511, Corrections on RRC, Huawei, CR 38.331, RAN2#120

- R2-2301443, Discussion on NPN Rel-18 work, Ericsson, DISC, RAN2#121

- R2-2301444, (draft) LS on how to proceed with specification work on eNPN in Rel-18, LS out, Ericsson, RAN2#121

# Appendix: TP 38.331

<TEXT OMITTED>

TMGI-r17 ::=                     SEQUENCE {

plmn-Id-r17                      CHOICE {

plmn-Index                       INTEGER (1..maxPLMN),

explicitValue                    PLMN-Identity

},

serviceId-r17                    OCTET STRING (SIZE (3))

}

-- TAG-TMGI-STOP

-- ASN1STOP

TMGI field descriptions

plmn-Index The PLMN index in plmn-IdentityInfoList and if present NPN index in npn-IdentityInfoList in SIB1.

serviceId Uniquely identifies the identity of an MBS service within a PLMN. The field contains octet 3- 5 of the IE Temporary Mobile Group Identity (TMGI) as defined in TS 24.008 [38]. The first octet contains the third octet of the TMGI, the second octet contains the fourth octet of the TMGI and so on.

<TEXT OMITTED>

#### –	HandoverPreparationInformation

This message is used to transfer the NR RRC information used by the target gNB during handover preparation or UE context retrieval, e.g. in case of resume or re-establishment, including UE capability information. This message is also used for transferring the information between the CU and DU.

Direction: source gNB/source RAN to target gNB or CU to DU.

HandoverPreparationInformation message

-- ASN1START

-- TAG-HANDOVER-PREPARATION-INFORMATION-START

HandoverPreparationInformation ::=      SEQUENCE {

<TEXT OMITTED>

HandoverPreparationInformation-IEs ::=  SEQUENCE {

ue-CapabilityRAT-List                   UE-CapabilityRAT-ContainerList,

sourceConfig                            AS-Config                                       OPTIONAL, -- Cond HO

rrm-Config                              RRM-Config                                      OPTIONAL,

as-Context                              AS-Context                                      OPTIONAL,

nonCriticalExtension                    SEQUENCE {}                                     OPTIONAL

}

AS-Config ::=                           SEQUENCE {

rrcReconfiguration                      OCTET STRING (CONTAINING RRCReconfiguration),

<TEXT OMITTED>

AS-Context ::=                          SEQUENCE {

reestablishmentInfo                     ReestablishmentInfo                                 OPTIONAL,

<TEXT OMITTED>

mbsInterestIndication-r17               OCTET STRING (CONTAINING MBSInterestIndication-r17) OPTIONAL

]],

[[

mrb-ToAddModSNPN-r17                    MRB-ToAddModSNPN-r17                                OPTIONAL,  –- Cond SNPN-Config

mbs-ServiceListSNPN-r17                 MBS-ServiceListSNPN-r17                             OPTIONAL   –- Cond SNPN-MII

]]

}

MRB-ToAddModSNPN-r17 ::= SEQUENCE (SIZE (1..maxNrofMBS-ServiceListPerUE-r17)) OF MBS-ServiceInfoSNPN-r17

MBS-ServiceListSNPN-r17 ::= SEQUENCE (SIZE (1..maxNrofMBS-ServiceListPerUE-r17)) OF MBS-ServiceInfoSNPN-r17

MBS-ServiceInfoSNPN-r17 ::= SEQUENCE {

snpn-NID-r17                        NID-r16                                                 OPTIONAL

}

<TEXT OMITTED>

-- TAG-HANDOVER-PREPARATION-INFORMATION-STOP

-- ASN1STOP

<TEXT OMITTED>

AS-Config field descriptions

rrcReconfiguration Contains the RRCReconfiguration configuration as generated entirely by the MN, where the plmn-Index (if included in mrb-ToAddModList) is replaced by the PLMN ID.

sdt-Config Contains the IE SDT-Config as generated entirely by the last serving gNB. This field is only used during the SDT procedure with UE context relocation as defined in TS 38.300 [2], clause 18.2.

sourceRB-SN-Config Contains the IE RadioBearerConfig as generated entirely by the SN. This field is only used when the UE is configured with SN terminated RB(s).

sourceSCG-Configured Value true indicates that the UE is configured with NR or EUTRA SCG in source configuration. The field is only used in NR-DC and NE-DC and is included only if the fields sourceSCG-NR-Config and sourceSCG-EUTRA-Config are absent.

sourceSCG-EUTRA-Config Contains the current dedicated SCG configuration in RRCConnectionReconfiguration message as specified in TS 36.331 [10] and generated entirely by the SN. In this version of the specification, the E-UTRA RRCConnectionReconfiguration message can only include the field scg-Configuration . This field is only used in NE-DC.

sourceSCG-NR-Config Contains the current dedicated SCG configuration in RRCReconfiguration message as generated entirely by the SN. In this version of the specification, the RRCReconfiguration message can only include fields secondaryCellGroup and measConfig. This field is only used in NR-DC.

AS-Context field descriptions

configRestrictInfoDAPS Includes fields for which source cell explicitly indicates the restriction to be observed by target cell during DAPS handover.

mrb-ToAddModSNPN Includes the NID in the i-th entry in case the i-th entry in the mrb-ToAddModList is an SNPN.

mbs-ServiceListSNPN Includes the NID in the i-th entry in case the i-th entry in the mbs-Servicelist is an SNPN.

mbsInterestIndication Includes the information last reported by the UE in the NR MBSInterestIndication message, where the plmn-Index (if included by the UE in tmgi) is replaced by the PLMN ID.

<TEXT OMITTED>

| Conditional Presence | Explanation |
| --- | --- |
| HO | The field is mandatory present in case of handover within NR or UE context retrieval, e.g. in case of resume or re-establishment. The field is optionally present in case of handover from E-UTRA/5GC. Otherwise the field is absent. |
| HO2 | The field is optionally present in case of handover within NR; otherwise the field is absent. |
| SNPN-Config | The field is optionally present in case the IE rrcReconfiguration in IE as-Config is present and the mrb-ToAddModList includes one or more MBS services for which the plmn-Identity indicates an SNPN. Otherwise the field is absent. |
| SNPN-MII | The field is optionally present in case the IE mbs-ServiceList in IE mbsInterestIndication is present and the IE mbs-ServiceList includes one or more MBS services for which the plmn-Identity indicates an SNPN. Otherwise the field is absent. |

<TEXT OMITTED>

#### –	MBS-ServiceList

The IE MBS- ServiceList is used to inform the network of the MBS services that the UE is receiving or interested to receive.

MBS-ServiceList information element

-- ASN1START

-- TAG-MBS-SERVICELIST-START

MBS-ServiceList-r17 ::=         SEQUENCE (SIZE (1..maxNrofMBS-ServiceListPerUE-r17)) OF MBS-ServiceInfo-r17

MBS-ServiceInfo-r17 ::=         SEQUENCE {

tmgi-r17                        TMGI-r17

}

-- TAG-MBS-SERVICELIST-STOP

-- ASN1STOP

<TEXT OMITTED>