3GPP TSG-RAN WG2 Meeting #121	R2-2301217

Athens, Greece, 27th of Feb – 3rd of Mar 2023

Source: 			ZTE Corporation, Sanechips

Title: 	Remaining issues for LTM execution

Agenda item:		8.4.2.3

Document for:		Discussion and Decision

- Introduction

At last meeting, RAN2 discussed dynamic cell switch and made the following agreements [1]:

- The MAC CE agreed to carry LTM related information for cell switch is used for LTM triggering of the cell switch. - LTM cell switch is supervised by a timer - UE arrival in the target cell need to be indicated (somehow)

In this contribution, we discussed some remaining issues for LTM execution.

- Discussion

  - 2.1 Notification of the UE Arrival

At last meeting, RAN2 discussed the general procedure for LTM and agreed that the UE arrival in the target cell need to be indicated. Considering that both RACH-based and RACH-less procedures are supported for LTM cell switch, a general flow chart for LTM execution is shown as below.

Fig. 1 The flow chart of LTM execution

A general flow chart for LTM execution comprises the following three steps:

- STEP 1: The source cell send the cell switch command (MAC CE) to the UE, based on the L1 measurement result.

- STEP 2: The UE performs the operations for LTM according to the pre-configured RRC configuration of the target cell.

- STEP 3: If there is no valid TA for the target cell, the UE initiate the RACH-based procedure for executing LTM (i.e. case 1), otherwise, RACH-less LTM is executed (i.e. case 2).

For case 1, if CFRA is configured, the UE shall initiate the CFRA on the target cell. Since the preamble is dedicated RACH resources for a UE, the target cell can identify the UE identity according to the received preamble, and hence the the notification of the UE arrival can be the preamble in the case of the CFRA. Assuming the UE shall initiate the CBRA on the target cell, the NW can not identify the UE identity with the received preamble, but the C-RNTI MAC CE will be contained in the Msg.3 to the target cell, which can be used for identifying the UE identity.

- Observation 1: In the case of the RACH-based LTM: if CBRA is initiated for LTM, the notification of UE arrival is the C-RNTI MAC CE contained in the Msg.3; if CFRA is initiated for LTM, the notification of UE arrival is the preamble.

For case 2, since RACH procedure is skipped, the UE needs to send the target cell with a notification of the arrival in other way. However, without the RACH, in the case of inter-DU LTM, the target cell may have no idea about the beam that would be used by the UE when the UE accessing to it. That means, if the NW cannot find the suitable beam to receive the notification form the UE, the LTM failure is inevitable.

Therefore, the notification message to the target cell needs to be indicated explicitly or implicitly. Besides, the notification message should indicate at least two important information: 1) UE identity and 2) the beam information. In this sense, the following alternatives can be taken into account:

- Alt. 1: The UE uses the preallocated UL grant (e.g. configured grant type 1) to transmit the C-RNTI MAC CE to the target cell, each preallocated UL grant is associated with a reference signal of the target cell (e.g. CG-SDT like solution).

- Alt. 2: The UE sends to the target cell with an SRS which is associated with a reference signal in the SRS-Config of the target cell.

- Observation 2: In the case of the RACH-less LTM, to guarantee the notification of the UE arrival that can be received successfully by target cell, at least two information shall be indicated in either explicit or implicit way: 1)the arrived UE identity and 2) the beam information when accessing to the target cell.

Upon reception of the cell switch command with the reference signaling (RS) or beam information of the target cell, the UE can send the UL signaling associated with the RS or beam indicated in the cell switch command to notify the target cell about UE arrival.

Above all, according to the observation 2, for notification of UE arrival, it shall be discussed separately, one is for RACH-based LTM, the other one is for RACH-less LTM, so we make a proposal as below:

Proposal 1: For RACH-based LTM:

- For CFRA based LTM, reception of the Preamble is taken as the notification of the UE arrival.

- For CBRA based LTM, reception of the C-RNTI MAC CE in Msg.3 is taken as the notification of the UE arrival.

- Proposal 2: For RACH-less LTM, RAN2 to discuss options for the notification of the UE arrival:

- Option 1: The UE sends the C-RNTI MAC CE to the target cell with the preallocated UL grant which is associated with an indicated RS/beam by the cell switch command;

- Option 2: The UE sends to the target cell with an SRS which is associated with a RS/beam indicated by the cell switch command.

After the notification of UE arrival, the NW needs to send the UE with the ACK for the notification, in order to confirm the successful completion of LTM execution. For RACH-based LTM, there is a direct way that the successful completion of the RACH is an ACK to the notification of the UE arrival. As for the RACH-less LTM, the reception of the DCI addressed to the new C-RNTI for dynamic scheduling of the UL grant or DL assignment can be considered as the ACK to the notification of UE arrival. So we propose：

Proposal 3: The UE considers the successful completion of LTM execution by:

- For RACH based LTM: The successful completion of the RACH procedure.

- For RACH-less LTM: The reception of the DCI addressed to the new C-RNTI for dynamic scheduling of the UL grant or DL assignment from the target cell.

  - 2.2 Failure detection and handling

At last meeting, it’s agreed that LTM cell switch is supervised by a timer. Similar to legacy HO, a T304-like timer can be used in LTM for failure detection. The UE starts the timer upon triggering the LTM execution, i.e. upon reception of the cell switch command. And the UE stops the timer upon successful completion of LTM execution, i.e. successful completion of RACH procedure or upon reception of the DCI addressed to the new C-RNTI for dynamic scheduling of the UL grant or DL assignment from the target cell. If the timer expires, the UE shall consider that the LTM fails.

Proposal 4: A T304-like timer is introduced for LTM procedure, with the following principle:

- The UE starts the timer upon triggering the LTM execution, i.e. reception of the cell switch command.

- The UE stops the timer upon successful completion of LTM execution, i.e. for RACH-based LTM, successful completion of RACH procedure; for RACH-less LTM, upon reception of the DCI addressed to the new C-RNTI for dynamic scheduling of the UL grant or DL assignment from the target cell.

- When the timer expires, the UE considers the failure of LTM.

Regarding whether to introduce a new timer for LTM or reuse the current T304 timer, we think the main point is whether new values are required for the supervision of LTM execution. The value range for the current T304 timer is from 50ms to 10000ms, which is used for RACH-based handover. Considering that RACH-less LTM is to be supported and the LTM is targeted for a quick cell switch with less interruption time, a short timer may be useful for the fast failure detection. Thus we can further consider about the smaller value of the LTM timer, e.g. 10ms, 20ms, 40ms.

Observation 3: The value range for the current T304 timer is from 50ms to 10000ms, which is used for RACH-based handover. Considering that RACH-less LTM is to be supported and LTM is targeted for a quick cell switch with less interruption time, a short timer could be useful for the fast failure detection.

Proposal 5: RAN2 to discuss whether smaller values are required for the LTM timer, e.g. 10ms, 20ms, 40ms.

In the legacy handover, upon T304 expires, the UE shall trigger RRC re-establishment procedure, which shall cause the longer data interruption. Considering that the LTM may happen frequently, we think the RRC re-establishment should be avoided as much as possible. Regrading how to improve the LTM failure handling, there are several options that can be considered:

- Option 1: the UE selects one of another LTM candidate cells to trigger LTM execution autonomously based on a NW-configured condition/threshold, i.e. when the condition for that candidate cell is met;

- Option 2: the UE falls back to the source cell and reports the LTM failure information to the source cell;

In option 1, a possible way is to reuse a CHO based recovery during RRC re-establishment procedure. Namely, the UE performs the cell re-selection, if the selected cell is a LTM candidate cell, the UE can trigger the LTM execution to the selected candidate cell, based on the stored LTM candidate cell configuration. Otherwise, the UE shall perform the legacy re-establishment procedure. This solution can reduce the data interruption time compared to the legacy re-establishment procedure. However, the cell re-selection procedure still cause relative long time to perform cell selection and compare the cell quality based on the L3 measurement.

Observation 4: For CHO based recovery, the UE performs the cell re-selection during RRC re-establishment procedure, if the selected cell is a CHO candidate cell, the UE shall trigger the execution of CHO to the selected candidate cell. Otherwise, the UE shall trigger legacy re-establishment.

Another possible way is to allow the UE directly select a suitable candidate cell to perform the second LTM execution, according to the L1 measurement, e.g. when the L1 measurement on the candidate cell is above a threshold configured by the NW. In this way, a new recovery procedure may need to be defined.

In option 2, in order to ensure the source cell quality when the UE falls back to the source cell, the UE is required to keep radio link monitoring and beam failure detection on the source cell during performing LTM execution to the target cell, i.e. like DAPS HO. It requires some enhancements for the current RLM and BFD mechanism, e.g. to support RLM/BFD on source cell while performing random access to the target cell.

Considering that the LTM candidate cell configurations are maintained at the UE, we slightly prefer option 1 to make full use of the stored candidate cell configuration to realize the fast failure recovery. And the similar recovery operation can be also applicable upon detection of RLF on source cell. And RAN2 can further discuss whether to reuse the CHO based recovery procedure or define a new procedure for LTM recovery.

Proposal 6: Upon detection of RLF or LTM failure, the UE selects one of another LTM candidate cells to trigger LTM execution autonomously based on the NW-configured condition/threshold, i.e. when the condition for that candidate cell is met. FFS: whether to reuse CHO based recovery procedure during RRC re-establishment or define a new recovery procedure.

- Conclusion and proposals

In this contribution, we discussed remaining issues for LTM with the following observations and proposals:

- Observation 1: In the case of the RACH-based LTM: if CBRA is initiated for LTM, the notification of UE arrival is the C-RNTI MAC CE contained in the Msg.3; if CFRA is initiated for LTM, the notification of UE arrival is the preamble.

- Observation 2: In the case of the RACH-less LTM, to guarantee the notification of the UE arrival that can be received successfully by target cell, at least two information shall be indicated in either explicit or implicit way: 1)the arrived UE identity and 2) the beam information when accessing to the target cell.

Proposal 1: For RACH-based LTM:

- For CFRA based LTM, reception of the Preamble is taken as the notification of the UE arrival.

- For CBRA based LTM, reception of the C-RNTI MAC CE in Msg.3 is taken as the notification of the UE arrival.

- Proposal 2: For RACH-less LTM, RAN2 to discuss options for the notification of the UE arrival:

- Option 1: The UE sends the C-RNTI MAC CE to the target cell with the preallocated UL grant which is associated with an indicated RS/beam by the cell switch command;

- Option 2: The UE sends to the target cell with an SRS which is associated with a RS/beam indicated by the cell switch command.

Proposal 3: The UE considers the successful completion of LTM execution by:

- For RACH based LTM: The successful completion of the RACH procedure.

- For RACH-less LTM: The reception of the DCI addressed to the new C-RNTI for dynamic scheduling of the UL grant or DL assignment from the target cell.

Proposal 4: A T304-like timer is introduced for LTM procedure, with the following principle:

- The UE starts the timer upon triggering the LTM execution, i.e. reception of the cell switch command.

- The UE stops the timer upon successful completion of LTM execution, i.e. for RACH-based LTM, successful completion of RACH procedure; for RACH-less LTM, upon reception of the DCI addressed to the new C-RNTI for dynamic scheduling of the UL grant or DL assignment from the target cell.

- When the timer expires, the UE considers the failure of LTM.

Observation 3: The value range for the current T304 timer is from 50ms to 10000ms, which is used for RACH-based handover. Considering that RACH-less LTM is to be supported and LTM is targeted for a quick cell switch with less interruption time, a short timer could be useful for the fast failure detection.

Proposal 5: RAN2 to discuss whether smaller values are required for the LTM timer, e.g. 10ms, 20ms, 40ms.

Observation 4: For CHO based recovery, the UE performs the cell re-selection during RRC re-establishment procedure, if the selected cell is a CHO candidate cell, the UE shall trigger the execution of CHO to the selected candidate cell. Otherwise, the UE shall trigger legacy re-establishment.

Proposal 6: Upon detection of RLF or LTM failure, the UE selects one of another LTM candidate cells to trigger LTM execution autonomously based on the NW-configured condition/threshold, i.e. when the condition for that candidate cell is met. FFS: whether to reuse CHO based recovery procedure during RRC re-establishment or define a new recovery procedure.

- Reference

- RAN2#120 Chair Notes