3GPP TSG-RAN WG2 Meeting #121     			          	                                R2-2301249

Athens, February 27 - March 3, 2023

Agenda item:	6.1.3.1

Source:	CMCC

Title:	Correction on the access attempt associated NSAG info for RACH

WID/SID:	NR_Slice-Core

Document for:	Discussion and Decision

Introduction

In RAN2#120[1], RAN2 discuss how to select an NSAG when one or more S-NSSAI(s) triggering the access attempt associated with more than one NSAG, and the following agreement related to slice based RACH was achieved:

NSAG that are used for RACH are given by SIB1 and NSAG that are used for cell reselection are given by SIB16. Capture in the CR that UE checks SIB1 for NSAGs to use for slice-specific RACH (e.g. as in Ericsson CR).

And the following text is added for RRC connection establishment and RRC connection resume procedure in TS 38.331[2]:

- 5.3.3 RRC connection establishment - 5.3.3.2 Initiation <omitted> 1> if the upper layers provide NSAG information and one or more S-NSSAI(s) triggering the access attempt (TS 23.501 [32] and TS 24.501 [23]): 2> apply the NSAG with highest NSAG priority among the NSAGs that are included in SIB1 (i.e., in FeatureCombination and in RA-PrioritizationSliceInfo), and that are associated with the S-NSSAI(s) triggering the access attempt, in the Random Access procedure (TS 38.321 [3], clause 5.1); - 5.3.13 RRC connection resume - 5.3.13.2 Initiation <omitted> 2> if the upper layers provide NSAG information and one or more S-NSSAI(s) triggering the access attempt (TS 23.501 [32] and TS 24.501 [23]): 3> apply the NSAG with highest NSAG priority among the NSAGs that are included in SIB1 (i.e., in FeatureCombination and in RA-PrioritizationSliceInfo), and that are associated with the S-NSSAI(s) triggering the access attempt, in the Random Access procedure (TS 38.321 [3], clause 5.1);

This contribution will focus on the remaining issues related to slice based RACH triggered by more S-NSSAIs associated with more than one NSAG.

Discussion

As specified in TS 38.331[2], the IE FeatureCombination may include a NSAG ID list, such as NSAG#1+ NSAG#2, SDT+NSAG#2+NSAG#3, etc. And an access attempt may be also related to more than one NSAG.

FeatureCombination-r17 ::= SEQUENCE {

redCap-r17                 ENUMERATED {true}                        OPTIONAL,  -- Need R

smallData-r17              ENUMERATED {true}                        OPTIONAL,  -- Need R

nsag-r17                   NSAG-List-r17                            OPTIONAL,  -- Need R

msg3-Repetitions-r17       ENUMERATED {true}                        OPTIONAL,  -- Need R

spare4                     ENUMERATED {true}                        OPTIONAL,  -- Need R

spare3                     ENUMERATED {true}                        OPTIONAL,  -- Need R

spare2                     ENUMERATED {true}                        OPTIONAL,  -- Need R

spare1                     ENUMERATED {true}                        OPTIONAL   -- Need R

}

NSAG-List-r17 ::= SEQUENCE (SIZE (1.. maxSliceInfo-r17)) OF NSAG-ID-r17

If the access attempt is related to more than one network slice, the UE NAS provides to UE AS(i.e. RRC layer) the S-NSSAIs triggering the access, along with the NSAG information (e.g. the association between S-NSSAI and NSAG, the NSAG priority information), then RRC layer determines the NSAG for Random Access based on the information. When the S-NSSAIs triggering the access attempt are associated with more than one NSAG, the NSAG with highest priority among the NSAGs that are included in SIB1 is selected.

For example, the UE1 triggers an access attempt related to S-NSSAI#1 and S-NSSAI#2, and S-NSSAI#1 is associated with NSAG#1 (NSAG priority: 2), S-NSSAI#2 is associated with NSAG#2 (NSAG priority: 5), and it is assumed that a lower value means a higher priority. According to current spec, there are three cases as follows:

- Case 1: If both NSAG#1 and NSAG#2 are included in SIB1, NSAG#1 will be selected and applied in random access procedure because NSAG#1 has highest priority.

- Case 2: If only NSAG#1 is included in SIB1, NSAG#1 will be selected and applied in random access procedure.

- Case 3: If only NSAG#2 is included in SIB1, NSAG#2 will be selected and applied in random access procedure.

For case 1, if other feature (e.g. SDT) is triggered at the same time, the feature combination for this random access procedure is NSAG#1+SDT; if no other feature is triggered at the same time, the feature combination for this random access procedure is NSAG#1. Then the UE1 will select a set of random access resource for this random access procedure as specified in section 5.1.1b, 5.1.1c and 5.1.1d in TS 38.321[3].

For simplicity, we take case 1 as baseline and only consider NSAG feature. Assuming that RACH resource set#1 is related to NSAG#1, RACH resource set#2 is related to NSAG#1+NSAG#2. Then the UE1 will select RACH resource set#1.

In practice, however, RACH resource set#2 is more suitable for this access attempt from the UE perspective because it fulfills all triggered NSAGs (i.e. NSAG#1+NSAG#2). But as specified in current spec, RRC layer only selects the highest priority NSAG which is included in SIB1, and MAC cannot be aware of other NSAGs with lower priority triggered in this access attempt.

Figure 1. Inter-layer interaction for RACH partition

Observation 1: If only the NSAG with highest NSAG priority among the NSAGs that are included in SIB1 is applied, the RACH resource set which includes more NSAGs cannot be selected, since MAC cannot be aware of other NSAGs with lower priority triggered in this access attempt.

To avoid this issue, we think it’s better to consider all triggered NSAGs included in SIB1 during the selection of random access resource. For case 1, NSAG#1 and NSAG#2 will be selected and applied in random access procedure.

In addition, if more than one NSAG is selected in random access procedure, the NSAG priority or the order of NSAG should also be sent to MAC together. When there is no RACH resource set which includes all triggered NSAGs, the UE can select a set of RACH resource based on NSAG priority. For example, when the applicable RACH resource set is set#3 {NSAG#1}, set#4 {NSAG#2}, the UE1 should select RACH resource set#3 since the NSAG#1 has higher priority than NSAG#2.

Figure 2. Inter-layer interaction for RACH partition based on P1 and P2

Proposal 1: If the access attempt is associated with more than one NSAG, and multiple NSAGs are included in SIB1 among the associated NSAGs, apply the NSAGs with priority in random access procedure.

Proposal 2: If there are more than one NSAGs associated with current Random Access procedure and no RACH resource set is associated with all NSAGs, the UE selects a set of Random Access resource based on the NSAG priority.

If proposal 1 and proposal 2 are agreed, some corrections for RRC and MAC operation are needed. The corresponding CRs for TS 38.331 and TS 38.321 are provided in R2-2301250 [4] and R2-2301795 [5].

Proposal 3: RAN2 agree the CR for TS 38.331 in R2-2301250 and CR for TS 38.321 in R2-2301795.

Conclusion

Here are the proposals for slice-based RACH.

Observation 1: If only the NSAG with highest NSAG priority among the NSAGs that are included in SIB1 is applied, the RACH resource set which includes more NSAGs cannot be selected, since MAC cannot be aware of other NSAGs with lower priority triggered in this access attempt.

Proposal 1: If the access attempt is associated with more than one NSAG, and multiple NSAGs are included in SIB1 among the associated NSAGs, apply the NSAGs with priority in random access procedure.

Proposal 2: If there are more than one NSAGs associated with current Random Access procedure and no RACH resource set is associated with all NSAGs, select a set of Random Access resource based on the NSAG priority.

Proposal 3: RAN2 agree the CR for TS 38.331 in R2-2301250 and CR for TS 38.321 in R2-2301795.

References

- Report of 3GPP TSG RAN WG2 meeting #120, 14 - 18 November, 2022

- 3GPP TS 38.331, NR; Radio Resource Control (RRC) protocol specification

- 3GPP TS 38.321, NR; Medium Access Control (MAC) protocol specification

- R2-2301250, Correction on the access attempt associated NSAG info for RACH, CR for TS 38.331

- R2-2301795, Correction on the access attempt associated NSAG info for RACH, CR for TS 38.321