3GPP TSG-RAN2#121       	R2-2301250

Athens, Greece, 27th February– 3rd March 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3875 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the access attempt associated NSAG info for RACH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Slice-Core |  |  |  |  |  | Date: |  |  | 2023-2-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | If only the NSAG with highest NSAG priority among the NSAGs that are included in SIB1 is applied, the RACH resource set which includes more NSAGs cannot be selected, since MAC cannot be aware of other NSAGs with lower priority triggered in this access attempt. For example, the UE1 triggers an access attempt related to S-NSSAI#1 and S-NSSAI#2, and S-NSSAI#1 is associated with NSAG#1 (NSAG priority: 2), S-NSSAI#2 is associated with NSAG#2 (NSAG priority: 5), and it is assumed that a lower value means a higher priority. When both NSAG#1 and NSAG#2 are included in SIB1, NSAG#1 will be selected and applied in random access procedure because NSAG#1 has highest priority. Assuming there are two RACH resources: RACH resource set#1 is related to {NSAG#1}, RACH resource set#2 is related to {NSAG#1+NSAG#2}, the UE1 will select RACH resource set#1, because only NSAG#1 is applied for selection of RACH resource. In practice, however, RACH resource set#2 is more suitable for this access attempt from the UE perspective because it fulfills all triggered NSAGs (i.e. NSAG#1+NSAG#2). But as specified in current spec, RRC layer only selects the highest priority NSAG which is included in SIB1, and MAC cannot be aware of other NSAGs with lower priority triggered in this access attempt. It is beneficial to consider all triggered NSAGs included in SIB1 during the selection of RACH resource, i.e. NSAG#1 and NSAG#2 in above example. In addition, if more than one NSAG is selected in random access procedure, the NSAG priority should also be sent to MAC together. When there is no RACH resource set which includes all triggered NSAGs, the UE can select a set of RACH resource based on NSAG priority. For example, when the applicable RACH resource set is set#3 {NSAG#1}, set#4 {NSAG#2}, the UE1 should select RACH resource set#3 since the NSAG#1 has higher priority than NSAG#2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the RRC connection establishment and RRC connection resume procedure, clarify that the NSAG(s) among the NSAGs that are included in SIB1 with NSAG priority provided by upper layer is applied, if the upper layers provide NSAG information and the correponding S-NSSAI(s). Impact analysis Impacted functionality: RAN slicing Inter-operability: If the network is implemented according to the CR while the UE is not, the UE may not be able to determine the NSAG to perform the slice-specific RACH. If the UE is implemented according to the CR while the network is not, no interoperability problems are foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE requirements on slice based RACH cannot be well met. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3.2, 5.3.13.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.321 CR 1567 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

5.3.3	RRC connection establishment

5.3.3.2	Initiation

The UE initiates the procedure when upper layers request establishment of an RRC connection while the UE is in RRC_IDLE and it has acquired essential system information, or for sidelink communication as specified in clause 5.3.3.1a.

The UE shall ensure having valid and up to date essential system information as specified in clause 5.2.2.2 before initiating this procedure.

Upon initiation of the procedure, the UE shall:

1>	if the upper layers provide an Access Category and one or more Access Identities upon requesting establishment of an RRC connection:

2>	perform the unified access control procedure as specified in 5.3.14 using the Access Category and Access Identities provided by upper layers;

3>	if the access attempt is barred, the procedure ends;

1>	if the upper layers provide NSAG information and one or more S-NSSAI(s) triggering the access attempt (TS 23.501 [32] and TS 24.501 [23]):

2>	apply the NSAG(s) among the NSAGs that are included in SIB1 (i.e., in FeatureCombination and in RA-PrioritizationSliceInfo), and that are associated with the S-NSSAI(s) triggering the access attempt, with NSAG priorities provided by upper layer, in the Random Access procedure (TS 38.321 [3], clause 5.1);

NEXT CHANGE

5.3.13	RRC connection resume

5.3.13.2	Initiation

The UE initiates the procedure when upper layers or AS (when responding to RAN paging, upon triggering RNA updates while the UE is in RRC_INACTIVE, for NR sidelink communication/discovery/V2X sidelink communication as specified in clause 5.3.13.1a) requests the resume of a suspended RRC connection or requests the resume for initiating SDT as specified in clause 5.3.13.1b.

The UE shall ensure having valid and up to date essential system information as specified in clause 5.2.2.2 before initiating this procedure.

Upon initiation of the procedure, the UE shall:

1>	if the resumption of the RRC connection is triggered by response to NG-RAN paging:

2>	select '0' as the Access Category;

2>	perform the unified access control procedure as specified in 5.3.14 using the selected Access Category and one or more Access Identities provided by upper layers;

3>	if the access attempt is barred, the procedure ends;

1>	else if the resumption of the RRC connection is triggered by upper layers:

2>	if the upper layers provide an Access Category and one or more Access Identities:

3>	perform the unified access control procedure as specified in 5.3.14 using the Access Category and Access Identities provided by upper layers;

4>	if the access attempt is barred, the procedure ends;

2>	if the upper layers provide NSAG information and one or more S-NSSAI(s) triggering the access attempt (TS 23.501 [32] and TS 24.501 [23]):

3>	apply the NSAG(s) among the NSAGs that are included in SIB1 (i.e., in FeatureCombination and in RA-PrioritizationSliceInfo), and that are associated with the S-NSSAI(s) triggering the access attempt, with NSAG priorities provided by upper layer, in the Random Access procedure (TS 38.321 [3], clause 5.1);

END OF CHANGE