3GPP TSG-RAN WG2 Meeting #121        	                        R2-2301253

Athens, 27 February – 3 March, 2023

Agenda item:	8.6.3.1

Source:	CMCC

Title:	Discussion on enhancements for neighbour cell measurements

WID/SID:	IoT_NTN_enh

Document for:	Discussion

# 1	Introduction

In RAN2#120 meeting [1], the following agreements on mobility enhancements are achieved:

Agreements:

- For NB-IoT we support a trigger for neighbour cell measurements based on T-service (in the quasi-Earth fixed case) (this does not preclude anything for eMTC discussion)

Agreements:

- At least for NB-IoT NTN, for quasi-earth fixed cells, UE shall start intra/inter frequency measurement in connected mode before the t-Service if present. The exact time to start measurements in connected mode before t-Service can be left to UE implementation” (can revisit if we agree other proposal based on neighbour cell coverage)

- RAN2 will not specify the condition of stopping UE measurement before t-Service

- For earth-moving cell, the UE derives when loss of coverage of current cell happens (how to derive this information is FFS)

- For earth-moving cell, UE shall start intra/inter frequency measurements in RRC connected mode before losing coverage. The exact time to start measurements can be left to UE implementation

Agreements:

- CHO time trigger event is defined as time duration [t1, t2] associated for each CHO candidate cell. The UE shall execute CHO to that candidate cell during the time duration, if all other configured CHO execution conditions will apply and there is only one triggered candidate cell.: UE is allowed to perform HO only during T1 to T2.

- For eMTC NTN, introduce a location-based conditional reconfiguration trigger based on condEventD1 in NR, where the event will be satisfied if the distance between the UE and a first reference location (e.g. within the serving cell) is above a threshold, and the distance between the UE and a second reference location (e.g. within a neighbour cell) is below a threshold. (similar to condEventD1 in NR)

- For eMTC NTN, introduce event A4 based conditional trigger (similar to condEventA4 in NR).

- FFS whether time and location-based trigger conditions may be configured independently (i.e., without a jointly configured event A4 measurement condition) for eMTC NTN.

This contribution will discuss the potential enhancements for neighbour cell measurements.

- Discussion

## 2.1	Neighbour cell assistance information

During the R17 discussion in last meeting, RAN2 agreed not to introduce neighbour cell ephemeris for neighbour cell measurement for R17 IoT-NTN, but to support this in R18, with details FFS.

Agreements:

- RAN2 doesn’t introduce neighbour cell ephemeris in Rel-17 IoT-NTN, neither for eMTC not for NB-IoT. RAN2 agrees to support this in Rel-18, with details FFS.

In RAN#98-e, the WID of IoT-NTN [2] is revised as following:

4.1.2 Mobility enhancements The following mobility enhancements objectives are listed. - Support of neighbour cell measurements and corresponding measurement triggering before RLF, using Rel-17 (TN) NB-IoT, eMTC as a baseline. [RAN2] - Support signalling in system information of neighbour cell ephemeris, for eMTC and NB-IoT [RAN2] - Re-use the solutions introduced in Rel-17 NR NTN for mobility enhancements for eMTC, with minimum necessary changes to adapt them to eMTC [RAN2] - Define UE RRM core requirements for the above mobility enhancement features [RAN4].

Firstly, there is no limitation for RRC state and the neighbour cell ephemeris is broadcast in system information, we understand that the information can be used for neighbour cell measurement for RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED.

Proposal 1: The neighbour cell ephemeris can be used for neighbour cell measurement for RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED.

For quasi-earth fixed cell, the start time of neighbour cell is useful for neighbour cell measurement. If the start time of neighbour cell is earlier than the stop time of serving cell, the UE also can start measurement before the stop time of serving cell. If the start time of neighbor cell is later than the stop time of serving cell, the UE can start to perform measurement after entering the coverage of neighbour cell.

Proposal 2: For quasi-earth fixed cell, the start time of neighbour cell is introduced for neighbour cell measurement.

For earth moving cell, the serving time is different for UEs at different location. We understand that the UE can estimate the stop time of serving cell and start time of neighbour cell based on UE location, satellite ephemeris and cell coverage information (e.g. radius or elevation angles) of serving cell and neighbour cell, which is similar for estimation of discontinuous coverage in R17. Then the UE can start intra and inter-frequency measurements before losing coverage. To support the estimation, the neighbour cell coverage information (e.g. radius or elevation angles) can be provided to the UE. And how to estimate the start time of neighbour cell is up to UE implementation.

Proposal 3: For earth moving cell, the neighbour cell ephemeris and coverage information (e.g. radius or elevation angles) can be provided to the UE for estimating the start time of neighbour cell. How to estimate is up to UE implementation.

## 2.2 Measurement enhancements for NB-IoT NTN

### 2.1.1 Distance-based trigger

Whether to introduce the distance-based trigger for connected mode measurement is still FFS in RAN2.

RAN1 has agreed that the UE can re-acquire GNSS position fix with a new GNSS measurement gap. Therefore, the distance-based trigger can be also supported for IoT-NTN. This means that the UE can start the neighbour cell measurement when the UE is in the cell edge. The similar mechanism in NR NTN can be reused for IoT-NTN.

- For quasi-fixed cell scenario, the cell can broadcast a reference point and location-related parameter (a threshold). The UE can decide whether it is in the cell edge based on the reference point and the threshold.

- For moving cell scenario, the cell can broadcast a reference point and a threshold, but the cell center changes with the moving of the satellite. In our understanding, the UE can calculate the current reference point based on the reference point and the ephemeris.

Proposal 4: The distance-based trigger (e.g. the UE is in the cell edge) for intra and inter frequency measurements in connected mode is supported for both quasi-earth fixed cell and earth moving cell.

- In addition, RAN2 agreed to introduce neighbor cell ephemeris for neighbour cell measurement. Similar to serving cell, the reference point and location-related parameter of neighbour cell can also be provided to the UE. Both the distance thresh from reference point of serving cell and from reference point of neighbour cell can be considered for neighbour cell measurement, if broadcast.

- Proposal 5: The reference location and distance threshold of the serving cell and neighbour cell can be broadcast in SIB for neighbour cell measurement for both quasi-earth fixed cell and earth moving cell.

## 2.2	Measurement enhancements for eMTC NTN

For eMTC UE, it can always perform the neighbor cell measurement in connected mode. In addition, RAN2 has agreed to introduce the time-based and location-based CHO for eMTC NTN, the UE can perform neighbor cell measurements for CHO. Therefore, there is no need to introduce the new trigger for neighbour cell measurement. On the contrary, if we introduce the new trigger for measurement, for example, the UE performs neighbor cell measurements when the serving cell is to stop serving the current area or when the UE is in the cell edge, we think the measurement is too late for CHO.

Proposal 6: For eMTC over NTN, it is proposed not to introduce new trigger for connected mode neighbor cell measurement.

# 3	Conclusion

Here are the proposals for mobility enhancements for IoT-NTN.

Proposal 1: The neighbour cell ephemeris can be used for neighbour cell measurement for RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED.

Proposal 2: For quasi-earth fixed cell, the start time of neighbour cell is introduced for neighbour cell measurement.

Proposal 3: For earth moving cell, the neighbour cell ephemeris and coverage information (e.g. radius or elevation angles) can be provided to the UE for estimating the start time of neighbour cell. How to estimate is up to UE implementation.

Proposal 4: The distance-based trigger (e.g. the UE is in the cell edge) for intra and inter frequency measurements in connected mode is supported for both quasi-earth fixed cell and earth moving cell.

- Proposal 5: The reference location and distance threshold of the serving cell and neighbour cell can be broadcast in SIB for neighbour cell measurement for both quasi-earth fixed cell and earth moving cell.

Proposal 6: For eMTC over NTN, it is proposed not to introduce new trigger for connected mode neighbor cell measurement.

# 4	Reference

- R2-22XXXX, Report of 3GPP TSG RAN WG2 meeting #120, Toulouse, 14-18 November, 2022

- RP-223519, Revised WID on IoT NTN enhancements