# 3GPP TSG-RAN WG2 Meeting #121	   R2-2301265

# Athens, Greece, Feb 27th - Mar 3rd, 2023

# Agenda Item:	8.13.7

# Source:	CMCC

# Title:	SON enhancement for NPN

# Document for:	Discussion and Decision

1	Introduction

In RAN2#120, we discussed SON/MDT for NPN and the following agreements are made.

Agreements:

1	PNI-NPN (CAG) ID checking is NOT performed before sending the RLF/HOF report availability indication related to a PNI-NPN network.

2	PNI-NPN (CAG) ID checking is NOT performed before sending the logged MDT availability indication related to a PNI-NPN network.

3	Details of the checking of NPN IDs (e.g., Proposal 1 of R2-2211354) are FFS.

4	Introduce SNPN ID (e.g., NID) to RLF/HOF report. Details of how to introduce it are FFS.

FFS: Introduce SNPN ID to logged MDT report.

FFS: Introduce PNI-NPN ID to RLF/HOF report. Details of how to introduce it are FFS.

FFS: Introduce PNI-NPN ID (e.g., CAG ID) to logged MDT report. Details of how to introduce it are FFS.

FFS: Discuss whether to introduce of new NPN specific variables for PNI-NPNs.

FFS: Discuss whether to introduce of new NPN specific variables for SNPNs.

In this contribution we provide our opinions about the FFS in the above agreements and also some other SON/MDT enhancements for NPN.

2	Discussion

In RAN2#120, it has been agreed to introduce SNPN ID to RLF/HOF report. In our understanding, the NPN ID (e.g. NID, CAG ID) is beneficial for the network to understand whether the RLF/HOF/MDT report is for NPN or not. And it’s also a convenient way to identify the NPN report from PN report by NPN ID.

Proposal 1: Introduce NPN ID to RLF/HOF/MDT report.

MDT enhancement for NPN:

For logged MDT, the measurement logging is performed in IDLE and INACTIVE state, and it is configured with the logged measurement configuration parameters, which including the logging area (AreaConfiguration) to indicate area for which UE is requested to perform measurement logging. If AreaConfiguration is not configured, measurement logging is not restricted to specific cells or tracking areas but applies as long as the RPLMN is contained in plmn-IdentityList stored in VarLogMeasReport. In our understanding, if the NPN ID is not included in the logging area, UE may not perform measurement logging for NPN. And when the AreaConfiguration is not configured, if the npn-IdentityList is not included in LoggedMeasurementConfiguration, the UE may also not perform measurement logging for NPN.

In order to perform measurement logging for the NPN area, NPN ID is proposed to be added in logging area and LoggedMeasurementConfiguration to enhance the MDT configuration.

Proposal 2: Add NPN ID in the area configuration for logged MDT.

Proposal 3: Add plmn-IdentityList in LoggedMeasurementConfiguration for logged MDT to enable the measurement logging for NPN when the AreaConfiguration is not configured.

Common or separate NPN specific UE variable(s):

For SNPN, as in TS 23.501, an SNPN-enabled UE supports the SNPN access mode. When the UE is set to operate in SNPN access mode, the UE only selects and registers with SNPNs over Uu. If a UE is not set to operate in SNPN access mode, even if it is SNPN-enabled, the UE does not select and register with SNPNs. A UE not set to operate in SNPN access mode performs PLMN selection procedures. UEs operating in SNPN access mode only select cells and networks broadcasting both PLMN ID and NID of the selected SNPN.

In our understanding, a UE cannot moving between PN and SNPN. When a UE is operating in PN, it cannot move and register to SNPN. When a UE is operating in SNPN, it only operating between SNPNs. It seems that the UE may only contains the SON/MDT report for SNPN, but not the SON/MDT report for both SNPN and public network.

For SNPN, there is no co-existence issue between SNPN and PN, so it’s no need to introduce separate UE variable(s) for SNPN, but whether the existing UE variable(s) need to be enhanced for SNPN need further discussion.

For PNI-NPN, there may exist co-existence issue between PNI-NPN and PN. But if the NPN ID is introduced to RLF/HOF/MDT report, the report for PN and PNI-NPN can identify by the NPN ID. Whether there are any other issues that require separate UE variables needs further discussion.

Proposal 4: No need to introduce separate UE variable(s) at least for SNPN, but whether the existing UE variable(s) need to be enhanced for SNPN need further discussion.

SON enhancement for NPN:

As UE will only store the measurement report for no more than 48 hours and SNPNs may not want to share the logs with each other, if a UE connected to a SNPN and recorded the SON/MDT reports, then move to another SNPN, it’s preferred not to transmit the previous SNPN SON/MDT report to the registered SNPN, except the previous SNPN is allowed the UE to transmit the report to the current SNPN.

If the SNPN do not want to share the report with other SNPNs, the reports will be discard if the UE do not move back to the previous SNPN cell in 48 hours. The measurement resource is wasted for this kind of scenarios. Whether reporting the currently stored UE measurement reports before the UE moves to another SNPN to avoid the waste of measurement resources suggests to be discussed.

Proposal 5: RAN2 discuss whether/how to report the currently stored UE SON/MDT reports to the current SNPN before the UE moves to another SNPN.

3	Conclusion

Proposal 1: Introduce NPN ID to RLF/HOF/MDT report.

Proposal 2: Add NPN ID in the area configuration for logged MDT.

Proposal 3: Add plmn-IdentityList in LoggedMeasurementConfiguration for logged MDT to enable the measurement logging for NPN when the AreaConfiguration is not configured.

Proposal 4: No need to introduce separate UE variable(s) for SNPN, but whether the existing UE variable(s) need to be enhanced for SNPN need further discussion.

Proposal 5: RAN2 discuss whether/how to report the currently stored UE SON/MDT reports to the current SNPN before the UE moves to another SNPN.

4	Reference

[1] 	RP-221825, “New WID on Further Enhancement of Data Collection for SON_MDT in NR standalone and MR-DC”, CMCC