3GPP TSG-RAN WG2 #121Athens	R2-2301274

Athens, Feb 27th – March 03rd 2023

Agenda Item:	8.13.2

Source:	Ericsson

Title:	MRO for inter-system handover for voice fallback

Document for:	Discussion, Decision

# 1	Introduction

In RAN2#119bis-e meeting, the following agreements were taken on MRO for inter-system handover for voice fallback.

Agreements:

1	An explicit indication is included in RLF-report when mobility from NR fails and the corresponding MobilityFromNRCommand includes voiceFallbackIndication

2	The below content is included in RLF-report when reestablishment procedure is initiated due to mobility From NR failure.

a. reestablishmentCellID

In this contribution, we discuss an issue that needs a resolution in the RLF report in Rel-18.

# 2	MobilityFromNR with VoiceFallBack indication

#### 5.4.3.5	Mobility from NR failure

The UE shall:

1>	if the UE does not succeed in establishing the connection to the target radio access technology:

2>	if the targetRAT-Type in the received MobilityFromNRCommand is set to eutra and the UE supports Radio Link Failure Report for Inter-RAT MRO EUTRA:

3>	store handover failure information in VarRLF-Report according to 5.3.10.5;

2>	if voiceFallbackIndication is included in the MobilityFromNRCommand message; or

2>	if the mobility from NR procedure is for emergency services fallback as specified in TS 23.502 [43]:

3>	attempt to select an E-UTRA cell:

4>	if a suitable E-UTRA cell is selected; or

4>	if no suitable E-UTRA cell is available and an acceptable E-UTRA cell supporting emergency call is selected when the UE has an ongoing emergency call:

5>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure';

It has been agreed to enable the UE in the voice fallback handover procedure to select an acceptable cell if no suitable E-UTRA cell is found. This has been captured in TS 38.331 (version 17.3.0). An excerpt from the procedural text concerning selection of acceptable cell is shown in the following.

Hence, for emergency calls and for scenarios in which the UE does not find a suitable cell, the UE can reselect an acceptable cell. In such scenarios, according to the solution designed for the RLF in case of voice fallback failure the UE includes the selected EUTRA cell ID in the RLF report (reconnectCellID).

This will become misleading for the network analysis, since an acceptable EUTRA cell would not be a good target for the next handover for the UEs with regular MTSI service (not emergency calls).

Therefore, propose to enable the network to distinguish the suitable and acceptable cells based on the RLF report. This assists the network to differentiate the suitable and acceptable cells that can be treated differently as target cell for the for the next fallback handovers depending on the fallback type i.e., fallback due to voice call or fallback due to emergency service.

Failing to distinguish between acceptable and suitable cells that UE connected to, in the RLF report, can mislead the network in selecting target LTE cell for the voice fallback handovers.

RAN2 to enable distinguishing a suitable cell from an acceptable cell in the RLF report.

# 3	Conclusion

We have the following observation in section 2.

- Failing to distinguish between acceptable and suitable cells that UE connected to, in the RLF report, can mislead the network in selecting target LTE cell for the voice fallback handovers.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to enable distinguishing a suitable cell from an acceptable cell in the RLF report.