3GPP TSG-RAN WG2 #121	R2-2301279

Athens, Feb 27th – March 3rd 2023

Agenda Item:	8.13.8

Source:	Ericsson

Title:	MRO for SCG failure and fast MCG recovery optimization

Document for:	Discussion, Decision

# 1	Introduction

This paper will discuss SON/MDT enhancements for SCG failure procedure with Conditional PSCell Addition/Change (CPAC) information and fast MCG recovery procedures. In addition, we discuss additions to the RLF report based on the Rel-17 RLM/BFD relaxation feature.

# 2	MRO for SCG Failure and fast MCG recovery

- SCG failure information enhancement

At the time of SCG failure, there exist some CPAC information and measurements at the UE (not available at the network side) that are valuable for the network and CPAC configuration optimization. Some examples are:

- the time elapsed between reception of CPAC configuration and the CPAC execution​

- the time duration elapsed between fulfilling the two CPAC execution events e.g., A3 and A5 events

- the first fulfilled event e.g., A3 or A5 events

In addition, we believe status information of the MCG at the time of SCG Failure  is valuable for the network in taking a correct decision upon SCG Failure i.e., how close the MCG is to a failure? This can be deduced by reporting the following information.

- The supervision timers running at the MCG such as t310 and t312 values, if running at the time of SCG Failure.

- RLC retransmission counter value if running at the MCG

At the time of SCGFailureInformation transmission, knowledge of the MCG status i.e., whether MCG is close to failure or not (e.g., T310 or T312 timers’ values, if running at the MCG) is beneficial for the network decision.

Hence, in our opinion, the UE status and information that does not exist in UE context in the network, should be reported by the UE for SON enhancement in SCGFailureInformation message.

UE reports the following information in SCGFailureInformation message:

- The time elapsed between reception of CPAC configuration and the CPAC execution​

- The time elapsed between the two CPAC execution events e.g., A3 and A5

- The first fulfilled CPAC execution event e.g., A3 or A5

- the MCG timers such as T310 and T312 values if they are running

- RLC retransmission counter value at the MCG leg

In addition, in Rel-17 SON enhancements to the SCGFailureInformation it has been agreed to include perRAInfoList when the SCG failure is due to reconfiguration with sync failure or random access problem failure. However, while operating in shared spectrum and failure in RACH execution the SCG failure type can be set to LBT failure. In such cases, including the RA information in the SCGFailureInformation message is needed. Hence, we propose the following.

UE includes the perRAInfoList in the SCGFailureInformation when the UE fails at SCG due to LBT failure (i.e., failureType is set to scg-lbtFailure).

- MRO enhancements for fast MCG recovery

In RAN2#120 it has been agreed to use RLF report for the sake of fast MCG recovery optimization.

RLF report is enhanced to support fast MCG recovery MRO.

However, according  to TS 38.331 (version 17.2.0), the RLF report is deleted upon successful MCG recovery. This is shown in the following excerpt from the specification.

##### 5.3.5.5.2	Reconfiguration with sync

The UE shall perform the following actions to execute a reconfiguration with sync.

1>	if the AS security is not activated, perform the actions upon going to RRC_IDLE as specified in 5.3.11 with the release cause 'other' upon which the procedure ends;

1>	stop timer T430 if running;

1>	start timer T430 with the timer value set to ntn-UlSyncValidityDuration from the subframe indicated by epochTime, according to the target cell NTN-config;

1>	if no DAPS bearer is configured:

2>	stop timer T310 for the corresponding SpCell, if running;

1>	if this procedure is executed for the MCG:

2>	if timer T316 is running;

3>	stop timer T316;

3>	clear the information included in VarRLF-Report, if any;

In our opinion, the RLF report can be kept by the UE for a successful MCG recovery to report the MCG failure recovery status to the network. For example, keeping the RLF report after the successful MCG recovery enables the UE to log the time spent for a successful MCG recovery that is useful for the sake of T316 timer optimization. Moreover, the UE indicates the RLF availability in the following RRCReconfigurationComplete and other RRC complete messages. Hence, to enable the agreement made in the meeting RAN2#120, (i.e., using RLF report for fast MCG optimization) we propose the following:

UE does not delete the RLF report upon successful MCG recovery.

In addition, it has been agreed to consider optimization of fast MCG recovery for the following cases.

2	Consider at least below scenarios for fast MCG recovery MRO:

a.	T316 expiry

b.	SCG failure/deactivation during fast MCG recovery (i.e., running of T316). The “upon fast MCG recovery case” is FFS.

We think that for the sake of optimizing the fast MCG recovery procedure e.g., optimizing the T316 timer value, it is necessary to measure the time of successful operation of fast MCG recovery, hence network can, based on collected statistics on the successful and failed MCG recovery procedures, set the T316 timer in such a way to maximize the rate of successful MCG recovery while having minimum MCG failures stemming from T316 timer expiry. Hence, we propose the following:

UE logs the time of fast MCG recovery (T316 timer value) for both failed and successful MCG recovery scenarios in the RLF report.

In addition, when the UE compiles the MCGFailureInformation, it is beneficial to log and report the SCG supervision timers, such as T310 and T312 timers. Such information assists the network to consider optimization of the SCG leg (e.g., reconfiguring RLM resources) before a failure occurs at the SCG leg.

When UE compiles the MCGFailureInformation message, the UE logs the SCG RLF supervision timers’ status e.g., SCG associated T310 and T312 timers’ status (i.e., if the timers were running or not)

SCG Failure status in RLF report

In general, the fast MCG recovery fails due to the T316 expiry or inability to recover from the SCG leg due to either SCG deactivation/suspension. In our opinion, when a UE detects the MCG failure recovery cause in dual connectivity operation (MR-DC operation), UE shall store both SCG and MCG related information at the time of RLF and report such information to the network via RLF report. Later, when RLF report is fetched by the network, the network realizes that the UE was in dual connectivity (DC) operation, MCG recovery procedure was unsuccessful due to either T316 expiry or SCG deactivation/suspension. Such information can be used by the network node to optimize its DC configuration in such a way enabling the UE to use the MCG link recovery procedure via SCG.  In the light of the above, RAN3#118 has agreed the following:

–It is beneficial for the UE to report at least the cause of the fast MCG recovery failure (at least T316 expiry, SCG failure) and also, if the problem is SCG failure, the SCG failure type (at least t310-Expiry, randomAccessProblem, rlc-MaxNumRetx).

In addition to the highlighted SCG Failure types agreed by RAN3, there are other SCG Failure types that are required to be logged by the UE upon SCG Failure as part of RLF report. For example, if the NR RRC reconfiguration fails, the failure type is indicated as scg-ReconfigurationFailure, while upon SRB3 integrity check failure, srb3-IntegrityFailure type is indicated. All these failure types, both agreed by RAN3 and proposed here, are included in SCGFailureInformation message which cannot be sent out by a UE to the network in the case of MCG failure simultaneous with the SCG failure. However, this information is of use to the network for failure analysis and DC operation optimization. Therefore, we propose the following.

Upon MCG recovery failure due to SCG failure UE logs all possible existing SCG Failure causes in the RLF report.

We would like to highlight that the SCG might have been deactivated right upon MCG failure. In such scenarios, the UE may fail to perform MCG recovery even if T316 timer is configured. This is shown in the following excerpt of the RRC specification:

#### 5.3.10.3	Detection of radio link failure

The UE shall:

[…]

4>	else:

5>	store the radio link failure information in the VarRLF-Report as described in clause 5.3.10.5;

5>	if T316 is configured; and

5>	if SCG transmission is not suspended; and

5>	if the SCG is not deactivated; and

5>	if neither PSCell change nor PSCell addition is ongoing (i.e. timer T304 for the NR PSCell is not running in case of NR-DC or timer T307 of the E-UTRA PSCell is not running as specified in TS 36.331 [10], clause 5.3.10.10, in NE-DC):

6>	initiate the MCG failure information procedure as specified in 5.7.3b to report MCG radio link failure.

In the meeting RAN2#120 the above-mentioned SCG deactivation/suspension scenario is left for further study (The “upon fast MCG recovery case” is FFS). Considering the fast MCG recovery procedure may fail due to the suspension or deactivation of the SCG (i.e., when the SCG is already deactivated/suspended), we believe it is beneficial that RAN2 consider the SCG suspension/deactivation scenarios right upon MCG recovery. Therefore, we propose the following:

If the UE detects SCG suspension/deactivation right before initiation of MCG recovery, the UE logs SCG status (suspended/deactivated) in the RLF report.

In Rel-17 the RLM/BFD relaxation feature is specified that enables the UE to relax the RLM/BFD procedures for the sake of UE energy saving. Network can configure the RLM/BFD relaxation based on the following configuration either based on UE mobility or based on the radio link quality of the RLM/BFD resources.

SpCellConfig ::=                        SEQUENCE {

servCellIndex                       ServCellIndex

...,

[[

lowMobilityEvaluationConnected-r17  SEQUENCE {

s-SearchDeltaP-Connected-r17        ENUMERATED {dB3, dB6, dB9, dB12, dB15, spare3, spare2, spare1},

t-SearchDeltaP-Connected-r17        ENUMERATED {s5, s10, s20, s30, s60, s120, s180, s240, s300, spare7, spare6, spare5,

spare4, spare3, spare2, spare1}

}

goodServingCellEvaluationRLM-r17    GoodServingCellEvaluation-r17                               OPTIONAL,   -- Need R

goodServingCellEvaluationBFD-r17    GoodServingCellEvaluation-r17                               OPTIONAL,   -- Need R

Sub-optimal configuration of these parameters impacts the RLF declaration procedure. For example, if the RLM/BDF relaxation parameters are not properly optimized, the UE may experience a bad radio coverage but since it applied relaxed RLM/BFD procedure, the UE may not declare RLF early enough, which may lead to late RLF which can in turn negatively affect the UEs QoS as well as end users QoE. In another example, if the BFD relaxation is applied either due to the good cell evaluation or low mobility criterion evaluation by the UE, the UE may not perform BFR procedure at the right time that may lead to the RLF. In fact, triggering BFR could rescue the UE from the failures. However, a relaxed BFD procedure may impede triggering the BFR procedure at the right time to avoid failures.

The UE logs indication of RLM/BFD relaxation in the failure report if RLM/BFD relaxation was applied at the time of MCG failure or SCG failures.

# Conclusion

In the previous sections we made the following observations:

Observation 1	At the time of SCGFailureInformation transmission, knowledge of the MCG status i.e.,  whether MCG is close to failure or not (e.g., T310 or T312 timers’ values, if running at the MCG) is beneficial for the network decision .

Based on the discussion in the previous sections we propose the following:

Proposal 1	UE reports the following information in SCGFailureInformation message:

	The time elapsed between reception of CPAC configuration and the CPAC execution​

	The time elapsed between the two CPAC execution events e.g., A3 and A5

	The first fulfilled CPAC execution event e.g., A3 or A5

	the MCG timers such as T310 and T312 values if they are running

	RLC retransmission counter value at the MCG leg

Proposal 2	UE includes the perRAInfoList in the SCGFailureInformation when the UE fails at SCG due to LBT failure (i.e., failureType is set to scg-lbtFailure).

Proposal 3	UE does not delete the RLF report upon successful MCG recovery.

Proposal 4	UE logs the time of fast MCG recovery (T316 timer value) for both failed and successful MCG recovery scenarios in the RLF report.

Proposal 5	When UE compiles the MCGFailureInformation message, the UE logs the SCG RLF supervision timers’ status e.g., SCG associated T310 and T312 timers’ status (i.e., if the timers were running or not)

Proposal 6	Upon MCG recovery failure due to SCG failure UE logs all possible existing SCG Failure causes in the RLF report.

Proposal 7	If the UE detects SCG suspension/deactivation right before initiation of MCG recovery, the UE logs SCG status (suspended/deactivated) in the RLF report.

Proposal 8	The UE logs indication of RLM/BFD relaxation in the failure report if RLM/BFD relaxation was applied at the time of MCG failure or SCG failures.

# References

Tdoc Number, Title, Source, Meeting, Date

Spec number, Title, Source, Version, Date