3GPP TSG-RAN WG2 Meeting #121	R2-2301286

# Athens, Greece, 27 February – 03 March 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1543 | rev | - | Current version: | 16.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on implicit BFD-RS change |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-Core |  |  |  |  |  | Date: |  |  | 2023-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TS 38.213 specifies implicit BFD-RS allocation based on the PDCCH beam: # 6 Link recovery procedures A UE can be provided, for each BWP of a serving cell, a set of periodic CSI-RS resource configuration indexes by failureDetectionResourcesToAddModList and a set of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRSList or candidateBeamRSListExt or candidateBeamRSSCellList for radio link quality measurements on the BWP of the serving cell. If the UE is not provided by failureDetectionResourcesToAddModList for a BWP of the serving cell, the UE determines the set to include periodic CSI-RS resource configuration indexes with same values as the RS indexes in the RS sets indicated by TCI-State for respective CORESETs that the UE uses for monitoring PDCCH and, if there are two RS indexes in a TCI state, the set includes RS indexes configured with qcl-Type set to 'typeD' for the corresponding TCI states. The UE expects the set to include up to two RS indexes. The UE expects single port RS in the set . The UE expects single-port or two-port CSI-RS with frequency density equal to 1 or 3 REs per RB in the set .. In addition to RRC, the TCI state for the PDCCH can also be changed also with TCI State Indication for UE-specific PDCCH MAC CE as an example. Beam failure detection procedure is reset (by setting the BFI_COUNTER to 0) upon BFD-RS change, however, this is limited only by BFD-RS reconfiguration by upper layers: ## 5.17 Beam Failure Detection and Recovery procedure The MAC entity shall for each Serving Cell configured for beam failure detection: 1> if the beamFailureDetectionTimer expires; or 1> if beamFailureDetectionTimer, beamFailureInstanceMaxCount, or any of the reference signals used for beam failure detection is reconfigured by upper layers associated with this Serving Cell: 2> set BFI_COUNTER to 0.. Obviously, same principle should apply for the case of MAC CE based TCI state switch for PDCCH which becomes also the BFD-RS in case no failureDetectionResourcesToAddModList for the BWP of the serving cell is configured. NW may have changed the PDCCH beam exactly for the reason of bad quality in the previos one in which case the BFD procedure should be reset. |
| Summary of change: |  | Add a condition to reset the BFI_COUNTER based on a change of BFD-RS associated with the serving cell. Implementation of this CR by a Release 15 UE will not cause compatibility issues. Impact analysis Impacted functionality: functionality impacted. Inter-operability: - If the network is implemented according to the CR and the UE is not, no inter-operability issues, however, the UE may trigger BFR soon after BFD-RS change since the BFI_COUNTER is not reset. - If the UE is implemented according to the CR and the network is not, no inter-operability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE will not reset BFD procedure when BFD-RS is changed by implicit BFD-RS mapping to PDCCH beam which may lead to unnecessary beam failure declarations and may hence affect to quality of service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

## 5.17	Beam Failure Detection and Recovery procedure

The MAC entity may be configured by RRC per Serving Cell with a beam failure recovery procedure which is used for indicating to the serving gNB of a new SSB or CSI-RS when beam failure is detected on the serving SSB(s)/CSI-RS(s). Beam failure is detected by counting beam failure instance indication from the lower layers to the MAC entity. If beamFailureRecoveryConfig is reconfigured by upper layers during an ongoing Random Access procedure for beam failure recovery for SpCell, the MAC entity shall stop the ongoing Random Access procedure and initiate a Random Access procedure using the new configuration.

RRC configures the following parameters in the BeamFailureRecoveryConfig, BeamFailureRecoverySCellConfig, and the RadioLinkMonitoringConfig for the Beam Failure Detection and Recovery procedure:

-	beamFailureInstanceMaxCount for the beam failure detection;

-	beamFailureDetectionTimer for the beam failure detection;

-	beamFailureRecoveryTimer for the beam failure recovery procedure;

-	rsrp-ThresholdSSB: an RSRP threshold for the SpCell beam failure recovery;

-	rsrp-ThresholdBFR: an RSRP threshold for the SCell beam failure recovery;

-	powerRampingStep: powerRampingStep for the SpCell beam failure recovery;

-	powerRampingStepHighPriority: powerRampingStepHighPriority for the SpCell beam failure recovery;

-	preambleReceivedTargetPower: preambleReceivedTargetPower for the SpCell beam failure recovery;

-	preambleTransMax: preambleTransMax for the SpCell beam failure recovery;

-	scalingFactorBI: scalingFactorBI for the SpCell beam failure recovery;

-	ssb-perRACH-Occasion: ssb-perRACH-Occasion for the SpCell beam failure recovery using contention-free Random Access Resources;

-	ra-ResponseWindow: the time window to monitor response(s) for the SpCell beam failure recovery using contention-free Random Access Resources;

-	prach-ConfigurationIndex: prach-ConfigurationIndex for the SpCell beam failure recovery using contention-free Random Access Resources;

-	ra-ssb-OccasionMaskIndex: ra-ssb-OccasionMaskIndex for the SpCell beam failure recovery using contention-free Random Access Resources;

-	ra-OccasionList: ra-OccasionList for the SpCell beam failure recovery using contention-free Random Access Resources;

-	candidateBeamRSList: list of candidate beams for SpCell beam failure recovery;

-	candidateBeamRSSCellList: list of candidate beams for SCell beam failure recovery.

The following UE variables are used for the beam failure detection procedure:

-	BFI_COUNTER (per Serving Cell): counter for beam failure instance indication which is initially set to 0.

The MAC entity shall for each Serving Cell configured for beam failure detection:

1>	if beam failure instance indication has been received from lower layers:

2>	start or restart the beamFailureDetectionTimer;

2>	increment BFI_COUNTER by 1;

2>	if BFI_COUNTER >= beamFailureInstanceMaxCount:

3>	if the Serving Cell is SCell:

4>	trigger a BFR for this Serving Cell;

3>	else:

4>	initiate a Random Access procedure (see clause 5.1) on the SpCell.

1>	if the beamFailureDetectionTimer expires; or

1>	if beamFailureDetectionTimer, beamFailureInstanceMaxCount, or any of the reference signals used for beam failure detection is reconfigured by upper layers associated with this Serving Cell ; or

1>	if the reference signal(s) associated with this Serving Cell used for beam failure detection is changed:

2>	set BFI_COUNTER to 0.

1>	if the Serving Cell is SpCell and the Random Access procedure initiated for SpCell beam failure recovery is successfully completed (see clause 5.1):

2>	set BFI_COUNTER to 0;

2>	stop the beamFailureRecoveryTimer, if configured;

2>	consider the Beam Failure Recovery procedure successfully completed.

1>	else if the Serving Cell is SCell, and a PDCCH addressed to C-RNTI indicating uplink grant for a new transmission is received for the HARQ process used for the transmission of the BFR MAC CE or Truncated BFR MAC CE which contains beam failure recovery information of this Serving Cell; or

1>	if the SCell is deactivated as specified in clause 5.9:

2>	set BFI_COUNTER to 0;

2>	consider the Beam Failure Recovery procedure successfully completed and cancel all the triggered BFRs for this Serving Cell.

The MAC entity shall:

1>	if the Beam Failure Recovery procedure determines that at least one BFR has been triggered and not cancelled for an SCell for which evaluation of the candidate beams according to the requirements as specified in TS 38.133 [11] has been completed:

2>	if UL-SCH resources are available for a new transmission and if the UL-SCH resources can accommodate the BFR MAC CE plus its subheader as a result of LCP:

3>	instruct the Multiplexing and Assembly procedure to generate the BFR MAC CE.

2>	else if UL-SCH resources are available for a new transmission and if the UL-SCH resources can accommodate the Truncated BFR MAC CE plus its subheader as a result of LCP:

3>	instruct the Multiplexing and Assembly procedure to generate the Truncated BFR MAC CE.

2>	else:

3>	trigger the SR for SCell beam failure recovery for each SCell for which BFR has been triggered, not cancelled, and for which evaluation of the candidate beams according to the requirements as specified in TS 38.133 [11] has been completed.

All BFRs triggered for an SCell shall be cancelled when a MAC PDU is transmitted and this PDU includes a BFR MAC CE or Truncated BFR MAC CE which contains beam failure information of that SCell.

End of Changes