3GPP TSG-RAN2 Meeting #121	R2-2301292

Athens, Greece, 27th February - 3rd March 2022

Agenda item:	8.19.1

Source:	Qualcomm Incorporated

Title:	Discussion on enhanced eDRX in RRC inactive

WID/SID:	NR_redcap_enh-Core – Release 18

Document for:	Discussion and Decision

# Introduction

The WID [1] of Rel-18 eRedCap was updated in RAN #98e meeting.

- Power saving/energy efficiency enhancements - Enhanced eDRX in RRC_INACTIVE (>10.24s) [RAN2, RAN3, RAN4] - Note that this objective requires SA2, CT1 and CT4 involvement - Complexity/cost reduction - Further reduced UE complexity in FR1 [RAN1, RAN2, RAN4] - UE BB bandwidth reduction - 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH, with 20 MHz RF bandwidth for UL and DL - The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth. - Support additional separate early indication(s) [RAN1, RAN2] - UE peak data rate reduction - Relaxation of the constraint (vLayers·Qm·f ≥ 4) for peak data rate reduction - The relaxed constraint is, e.g., 1 (instead of 4). - The parameters (vLayers, Qm, f) can be as in Rel-17 RedCap. - Both 15 kHz SCS and 30 kHz SCS are supported. - Aim to define at most one Rel-18 RedCap UE type for further UE complexity reduction. - The existing UE capability framework is used, and changes to capability signalling are specified only if necessary. By default, all UE capabilities applicable to a Rel-17 RedCap UE are applicable unless otherwise specified. Notes: - The work defined as part of this WI is not to overlap with LPWA use cases. - Coexistence with non-RedCap UEs and Rel-17 RedCap UEs should be ensured. - This WI considers all applicable duplex modes unless otherwise specified. Check in RAN#99 regarding: - Whether UE peak data rate reduction for UE is limited only with UE BB bandwidth reduction or standalone.

According to the WID, Rel-18 eRedCap UE should study eDRX >10.24s in RRC inactive. In this paper, we would like to discuss eRedCap supporting eDRX > 10.24s in RRC inactive.

# Discussion

Rel-17 RedCap already supports eDRX configuration in RRC idle and inactive. One remaining issue left in Rel-17 is to configure eDRX >10.24s in RRC inactive due to pending SA2 confirmation. For now, SA2 has made many progresses on their Rel-18 study about enabling long eDRX >10.24s for UE in RRC inactive. SA2 has made conclusion to support CN based MT communication handling for UE in RRC inactive with eDRX > 10.24s [2][3]. Therefore, RAN2 should further study eDRX > 10.24 cycle in RRC inactive in Rel-18 eRedCap.

In the early study of Rel-17 RedCap, RAN2 has made several agreements related to eDRX > 10.24s configured in RRC idle and inactive.

In RAN2 #113bie-e meeting, the following agreements were made.

Agreements:

- RAN decides and configures eDRX via RRC for RRC_INACTIVE (FFS on the need and details of coordination with the CN)

- At least for eDRX cycle, the configurations of the eDRX for RRC_IDLE and RRC_INACTIVE can be different (FFS for PTW, e.g. length and starting point, when eDRX cycles are longer than 10.24s)

Agreements - via email (from offline [101]):

- RAN2 assumes that CN provides necessary assistance information on eDRX config. for RRC_IDLE to RAN (e.g. reusing eDRX config. defined in “CN Assistance Information for RRC INACTIVE IE” for E-UTRA/5GC).

- eDRX feature, including the related parameters (i.e. PH, PTW. H-SFN) and corresponding paging operation defined for E-UTRA/5GC is used as baseline to enable eDRX >10.24sec for both RRC_IDLE and RRC_INACTIVE in NR/5GC

- RAN2 confirms that CN paging and RAN paging use the same paging frame offset and first PDCCH monitoring occasion in PO, which are configured by RAN without involvement of CN.

- RAN2 confirms that SI modification mechanism from LTE is used as a baseline for SI change (other than ETWS and CMAS), i.e. by using an eDRX acquisition period and a flag to indicate SI modification for eDRX in Short Message (e.g. systemInfoModification-eDRX)

In RAN2 #114e meeting, the following agreements were made.

Agreements via email - from offline 110:

- Lower bound for eDRX configuration in RRC_IDLE and RRC_INACTIVE is 2.56 seconds. Inform SA2/CT1 and check if there is any concern.

- It is up to RAN to configure the length for PTW for RAN paging, the RAN PTW length can be different from the CN PTW length.

- When RAN and CN paging coincide in the same PH, the PTW starting locations are the same. FFS how to calculate the PTW starting location so that it is the same for RAN and CN PTW.

The eDRX mechanism including the related parameters such as PH, PTW and corresponding paging monitoring procedure defined for LTE and NR can be reused as baseline to enable the eDRX > 10.24s in RRC inactive. According to the pervious study, RAN should be responsible for eDRX configuration for RRC inactive and have coordination with CN. For the eDRX cycle and PTW length, the value and the configuration can be different between RAN and CN. Therefore, we think RAN2 should first confirm the previous relevant agreements can be applied for the eDRX > 10.24s in RRC inactive.

Proposal 1: For eDRX > 10.24s configured in RRC inactive, RAN2 confirms that it is up to RAN to configure the length of PTW for RAN paging, and RAN PTW length can be different from the CN PTW length.

When UE is in RRC idle, UE only needs to monitor CN paging in the CN configured PTW and eDRX cycle. When UE is in RRC inactive, UE should monitor both RAN and CN paging. Since there may be possible RRC state mismatch between UE and RAN/CN, UE is required to monitor both RAN and CN paging. In the case of eDRX > 10.24s in RRC inactive, the RAN and CN paging may be delivered in the non-overlapped PHs or could coincide in the same PH.

The RAN2 #115 meeting has agreed that ‘RAN2 considers the configuration as invalid case, where INACTIVE eDRX cycle is longer than IDLE eDRX cycle.’ Therefore, the eDRX cycle configured by RAN should be smaller than the eDRX cycle configured by CN. For non-overlapped PHs for RAN and CN paging, it is simple that UE just monitors the RAN paging in the eDRX cycle configured by RAN and in the PTW configured for RRC inactive in the case of eDRX > 10.24s in RRC inactive.

Proposal 2: For non-overlapped PHs for RAN and CN paging, UE follows the PTW configured for RRC inactive when UE is in RRC inactive with eDRX > 10.24s.

When RAN and CN paging coincides in the same PH, in which PTW (RAN or CN) UE monitors paging should be studied.

According to the agreements made in RAN2 #114, and proposal 1 above, RAN decides and configures the PTW and the length can be different from the CN PTW, and meanwhile, when RAN and CN paging coincides in the same PH, the PTW starting locations are the same. Thus, when RAN and CN paging coincide in the same PH, the PTWs could be overlapped with each other. Then there could be different options for UE  monitoring paging in RAN PTW or CN PTW.

Option 1 is that UE only monitors paging in the PTW configured by RAN. This option is aligned with the PTW design principle for different RRC states, i.e., UE in RRC inactive state should only follow the paging window configuration by RAN. It is also beneficial for UE power saving to not require UE monitoring outside the RAN PTW. However, this option requires network to send CN paging also in the PTW configured by RAN. The coordination on the PTW configuration between RAN and CN is needed. For example, RAN may be asked to provide the necessary assistance information to CN about RAN PTW configuration.

Option 2 is to require UE to monitor paging in the PTW configured by CN. This option may require UE to monitor PTW configured by CN in RRC inactive. Since the PTW length configured by RAN and CN may be different, if the length of RAN PTW is always shorter than PTW for CN, the RAN PTW seems to be useless. It implies that UE also needs to monitor the paging outside PTW configured by RAN which is not good for UE power saving especially for an eRedCap device.

Option 3 is to configure the same length of RAN PTW and CN PTW. The paging monitoring could be simplified in this option, since only one PTW length is for UE to monitor paging. However, this option actually requires restrictions on the network configurations, and it is also no benefit for UE power saving.

Taking above analysis into consideration, we think option 1 is more preferable. UE monitors both RAN and CN paging in the PTW configured for RRC inactive when RAN and CN paging coincides in the same PH. It is beneficial for UE power saving and has less network impacts as long as the RAN has coordination with CN on the RAN eDRX cycle configuration.

Proposal 3: When RAN and CN paging coincides in the same PH and PTWs configured by RAN/CN overlapping each other, UE monitors both RAN and CN paging in the PTW configured by RAN when UE is in RRC inactive with eDRX > 10.24s.

It is necessary that RAN and CN have coordination on the eDRX cycle and length of PTW configuration so that UE only needs to monitor both RAN and CN paging in the PTW configured by RAN. Thus, the eDRX cycle and PTW length configured by RAN is informed to CN for eDRX > 10.24s in RRC inactive.

Proposal 4: The eDRX cycle and PTW length configured by RAN is provided to CN for eDRX > 10.24s in RRC inactive.

# Conclusion

We have the following observations, and we’d recommend RAN2 to discuss and adopt the following proposals:

Proposal 1: For eDRX > 10.24s configured in RRC inactive, RAN2 confirms that it is up to RAN to configure the length of PTW for RAN paging, and RAN PTW length can be different from the CN PTW length.

Proposal 2: For non-overlapped PHs for RAN and CN paging, UE follows the PTW configured for RRC inactive when UE is in RRC inactive with eDRX > 10.24s.

Proposal 3: When RAN and CN paging coincides in the same PH and PTWs configured by RAN/CN overlapping each other, UE monitors both RAN and CN paging in the PTW configured by RAN when UE is in RRC inactive with eDRX > 10.24s.

Proposal 4: The eDRX cycle and PTW length configured by RAN is provided to CN for eDRX > 10.24s in RRC inactive.

# References

- RP-223544, Enhanced support of reduced capability NR devices, Ericsson.

- R2-2206967, LS On FS_REDCAP_Ph2 option feasibility, Ericsson.

- R2-2211136, LS On long eDRX support for RRC_INACTIVE, Ericsson.

# Annex