3GPP TSG-RAN WG2 #121	R2-2301307

Athens, Greece February 27th – March 3rd, 2023

Agenda Item:	8.2.3

Source:	Ericsson

Title:	Signaling for LMF-based RAT-dependent positioning Integrity

Document for:	Discussion, Decision

# 1	Introduction

The following objective was agreed for the work of integrity support for RAT dependent positioning[1]:

- Specify the error modelling parameters, signalling, and procedures to support UE-based and LMF-based integrity of RAT-dependent positioning methods [RAN2, RAN3].

Integrity for RAT independent positioning has been supported by 3GPP since release 17, specifically for GNSS / RTK.  In this paper, we give our view on the possible LPP signaling for LMF-mode RAT dependent positioning.

# 2	Discussion

## 2.1 Integrity in general

The discussions at RAN2#120 led to an agreement [4] to send an LS [3] to SA2 about integrity parameters that can be configured by the LCS client/UE/AF:

RAN2 would like to provide the following answer to SA2's question on the parameters that are needed: - LCS client/UE/AF sends TIR, AL, TTA to the LMF - LMF returns the system available/unavailable indication to the LCS client/UE/AF

This agreement now implies that LMF can be configured with the integrity parameters TIR, AL and TTA.

LMF can be configured with the integrity parameters TIR, AL and TTA.

The integrity concept is about to determine whether measurements, estimates or information can be trusted for reliable use. As confirmed at RAN2#119-e, the agreed KPI mode 1 concerns UE determination and reporting of PL to the network as part of location information. This can enable the network to determine reliability of a reported position estimate from the device by comparing to the Alert Limit (AL) in consideration of a time to alert timer (TTA).

Position estimate reliability is assessed by comparing PL to AL in consideration of TTA.

Clearly, network based position estimate reliability determination needs very frequent position estimate and PL reporting to the network in order to be relevant.

A more practical alternative in case the beneficiary of the reliability information is the device itself, is to determine the position estimate reliability in the device. In such a case, the following aspects typically apply:

- UE-based GNSS position estimation is configured (with Rel 18 completed, UE-based RAT-dependent position estimation is an option)

- The position information can be determined very frequently and integrity results can be determined almost continuously, allowing a continuous position estimate reliability determination, provided that integrity requirements in terms of TIR, AL and TTA are available

  - TIR, AL and TTA can be pre-configured or made available from higher device layers

  - TIR, AL and TTA could be provided via LPP

- Position estimate reliability can be directly reported to higher layers of the device to promptly notifying layers using the position estimate, for example in safety critical use cases

- The agreed mode 1 PL reporting can be combined to allow the network to gather some SLA information

UE-based integrity with position estimate reliability determination has many advantages suitable for navigating devices. However, TIR, AL and TTA are needed, either pre-configured, proprietary from higher layers, or provided by the network.

UE-based with position estimate reliability determination is incomplete without signalling of TIR, AL and TTA

It is important to note that the need for TIR, AL and TTA to make UE-based integrity complete involves adding AL and TTA to the ProvideAssistanceData. This is different from the mode 1 and mode 2 reporting discussion, where mode 2 would have required AL and TTA to be included in the RequestLocationInformation. Mode 2 was not agreed, and even if it would have been agreed, it would not have made UE-based integrity complete, since it requires TIR, AL and TTA to be included in the ProvideAssistanceData.

Based on the discussion, we have the following proposal

Add TIR, AL and TTA to the integrity assistance data to make UE-based integrity complete

The TIR, AL and TTA inclusion was discussed at RAN2#119-e [1], and one main concern was whether the UE was required to use the provided TIR, AL and TTA or not. Therefore, we would like to make this aspect very clear by proposing the following logic

- If the device indicates a capability to support integrity reliability determination OR if the device requests integrity requirements for integrity reliability determination, THEN the device shall use provided integrity requirements for integrity reliability determination

- If the device does not indicate capability AND does to request integrity requirements, THEN the device uses other available integrity requirements for integrity reliability determination

The logic would be represented in field descriptions of the integrity requirements attributes TIR, AL and TTA

Define in TIR, AL and TTA field descriptions that these are mandatory to use for integrity reliability determination if the device has indicated a corresponding capability or has requested for TIR, AL and TTA as part of the assistance data.

The required LPP updates to support integrity requirements to make UE-based integrity complete are provided in Appendix B.

Agree to the 37.355 text proposal in Appendix A to make UE-based integrity complete

## 2.2 RAT-dependent integrity

In TR 38.859[2], the identified error sources and candidates for distributions to model the errors was captured in Table 6.1.1-1 and Table 6.1.1-2. The required signalling for UE-based positioning integrity and LMF-based positioning integrity are also briefly summarized.

Table 6.1.1-1: Error sources for LMF-based and UE-based positioning integrity modes

| Positioning Integrity Mode | DL TDOA | UL TDOA | Multi-RTT | UL AoA | DL AoD |
| --- | --- | --- | --- | --- | --- |
| LMF-based (as defined in Table 9.4.1.1.1 in TR 38.857 [2]) | - RSTD measurement - TRP location - Inter-TRP synchronization (can be caused in part by errors in SFN initialization time.) | - RTOA measurement - TRP location - Inter-TRP synchronization (can be caused in part by errors in SFN initialization time.) | - UE Rx-Tx time difference measurement - gNB Rx-Tx time difference measurement - TRP location | - Angle of arrival measurement - TRP location - ARP location (e.g., ARPLocationInformation in TS 38.455 [17]) | - TRP location - DL-PRS RSRPP of the first path or RSRP |
| UE-based (as defined in Table 9.4.1.1.1 in TR 38.857 [2]) | - TRP location (e.g., NR-TRP-LocationInfo in TS 37.355 [16]) - Inter-TRP synchronization (e.g., NR-RTD-Info in TS 37.355 [16]) |  |  |  | - TRP location (e.g., NR-TRP-LocationInfo in TS 37.355 [16]) |

Table 6.1.1-2: Identified candidates for distributions to model the errors due to different error sources

| Error source | Candidate(s) for distribution for error source |
| --- | --- |
| Timing measurement errors (NOTE 1, 2, 3) | Gaussian distribution |
| Inter-TRP synchronization errors | - Uniform distribution (NOTE 4) - Gaussian distribution |
| TRP location error (e.g., NR-TRP-LocationInfo in [16]) | - Uniform distribution (NOTE 5) - Gaussian distribution |
| TRP location error (e.g., Geographical coordinates in [17]) | - Uniform distribution - Gaussian distribution |
| ARP location error (e.g., ARPLocationInformation in [17]) | - Uniform distribution - Gaussian distribution |
| NOTE 1: Timing measurement errors are applicable to RSTD, RTOA and UE/gNB Rx-Tx time difference measurements. NOTE 2: It is assumed that the timing measurement error is associated with the first path. NOTE 3: It is assumed that the timing measurement error contains TEG related TX/RX timing error if the TEG related information is provided NOTE 4: This may already be consistent with the uncertainty related to NR-RTD-Info in [16]. NOTE 5: This may already be consistent with the uncertainty related to NR-TRP-LocationInfo in [16]. |  |

6.1.2.2 Signaling for UE-based positioning integrity

Rel-17 UE-based integrity mode signaling can be used as baseline at least with the following aspects:

UE sends capability info to LMF on integrity for UE-based mode using LPP capability transfer procedure.

-	LMF sends the assistance data for integrity calculation to UE. LMF provides, in assistance data, the information of error sources (e.g., originated from RAN node) to UE for integrity in UE-based mode.

-	LMF sends integrity requirement e.g., TIR to UE in LPP request location information message for integrity of UE-based mode.

-	UE sends integrity result to LMF using LPP location information Transfer message.

6.1.2.3	 Signaling for LMF-based positioning integrity

RAN2 studied LMF-based positioning integrity mode [2], and have identified the following impacts to signaling:

-	UE sends capability info to LMF for LMF-based positioning integrity mode using LPP capability transfer procedure

-	LMF sends the request of results related to integrity for integrity error sources to UE for integrity of LMF-based mode

-	LMF sends the request of results related to integrity for integrity error sources to RAN for integrity of LMF-based mode

-	RAN sends results related to integrity to LMF using NRPPa message.

NOTE 1: The signaling to transmit integrity KPI and integrity results can be discussed during normative work.

NOTE 2: Whether UE sends results related to integrity to LMF using LPP message or not can be discussed during normative work.

### 2.2.1 LMF-based positioning integrity signalling

For LMF-based positioning integrity mode, the identified error sources from UE are all associated with UE’s measurements, thus it would be natural to support LMF requesting UE to send error source statistics of error source using LPP RequestLocationInformation message and UE reporting the error source result to LMF in LPP in ProvideInformation message for each positioning method.

For LMF-based integrity for RAT-dependent positioning, the R17 UE-assisted integrity mode signaling can be used as baseline with the following aspects and agree to the text proposal as in Annex:

- UE sends capability info to LMF on integrity for UE-Assisted mode using LPP capability transfer procedure

- LMF provides the Assistance Data for Positioning (same as legacy) and request for Integrity error sources

- UE performs positioning measurements and computes the error (same as legacy)

- UE generates error sources for the requested measurements using mean and standard deviation and provides to the LMF using LPP

- LMF computes the Integrity.

### 2.2.2 LMF requests UE to send error source model statistics

For LMF-based positioning integrity mode, LMF requests UE to send error source statistics of error source and such request can be sent in the RequestLocationInformation for each RAT positioning method.

For LMF-based positioning integrity mode, LMF requests UE to send error source statistics of error source in the RequestLocationInformation for each RAT positioning method.

Agree to the LPP text proposal in Annex B.1

### 2.2.3 UE send error source model statistics

For LMF-based positioning integrity mode, the timing measurement error sources include RSTD measurement in DL-TDOA and UE Rx-Tx time difference measurement Multi-RTT. In TR 38.859, it is concluded that the timing measurement error can be modeled as Gaussian/Normal distribution. UE may send the error source statistics in the SignalMeasurementInformation message for the corresponding positioning method.

For LMF-based positioning integrity mode, UE sends the error source statistics in the SignalMeasurementInformation message for the corresponding positioning method.

Agree to the LPP text proposal in Annex B.2

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Add TIR, AL and TTA to the integrity assistance data to make UE-based integrity complete

Proposal 2	Define in TIR, AL and TTA field descriptions that these are mandatory to use for integrity reliability determination if the device has indicated a corresponding capability or has requested for TIR, AL and TTA as part of the assistance data.

Proposal 3	Agree to the 37.355 text proposal in Appendix A to make UE-based integrity complete

Proposal 4	For LMF-based integrity for RAT-dependent positioning, the R17 UE-assisted integrity mode signaling can be used as baseline with the following aspects and agree to the text proposal as in Annex:

	UE sends capability info to LMF on integrity for UE-Assisted mode using LPP capability transfer procedure

	LMF provides the Assistance Data for Positioning (same as legacy) and request for Integrity error sources

	UE performs positioning measurements and computes the error (same as legacy)

	UE generates error sources for the requested measurements using mean and standard deviation and provides to the LMF using LPP

	LMF computes the Integrity.

Proposal 5	For LMF-based positioning integrity mode, LMF requests UE to send error source statistics of error source in the RequestLocationInformation for each RAT positioning method.

Proposal 6	Agree to the LPP text proposal in Annex B.1

Proposal 7	For LMF-based positioning integrity mode, UE sends the error source statistics in the SignalMeasurementInformation message for the corresponding positioning method.

Proposal 8	Agree to the LPP text proposal in Annex B.2

# References

RP-223549, “New WID on Expanded and Improved NR Positioning”, December 12 - 16, 2022

TR 38.859, “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on expanded and improved NR positioning;(Release 18)”, v18.0.0, December 2022

R2-2213320, “Reply LS on GNSS integrity requirement provisioning”, Huawei, Nov 2022

RAN2#120 Chairman’s notes

# Annex A – Text Proposal to 37.355

### 6.4.1	Common Lower-Level IEs

[…]

#### –	HorizontalWithVerticalVelocityAndUncertainty

The IE HorizontalWithVerticalVelocityAndUncertainty is used to describe a velocity shape as defined in TS 23.032 [15].

-- ASN1START

HorizontalWithVerticalVelocityAndUncertainty ::= SEQUENCE {

bearing						INTEGER(0..359),

horizontalSpeed				INTEGER(0..2047),

verticalDirection			ENUMERATED{upward, downward},

verticalSpeed				INTEGER(0..255),

horizontalUncertaintySpeed	INTEGER(0..255),

verticalUncertaintySpeed	INTEGER(0..255)

}

-- ASN1STOP

#### –	IntegrityRequirementsSup

The IE IntegrityRequirementsSup is used to indicate support of position estimate reliability determination based on integrity requirements attributes provided in the assistance data.

-- ASN1START

IntegrityRequirementsSup-r17::= SEQUENCE {

...

}

-- ASN1STOP

### 6.4.2	Common Positioning

[…]

#### –	CommonIEsProvideCapabilities

The CommonIEsProvideCapabilities carries common IEs for a Provide Capabilities LPP message Type.

-- ASN1START

CommonIEsProvideCapabilities ::= SEQUENCE {

...,

[[

segmentationInfo-r14			SegmentationInfo-r14			OPTIONAL,	-- Cond Segmentation

lpp-message-segmentation-r14	BIT STRING { serverToTarget	(0),

targetToServer	(1) }	OPTIONAL

]] ,

[[

integrityRequirementsSup-r17	IntegrityRequirementsSup-r17	OPTIONAL	-- Cond IntReq

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| Segmentation | This field is optionally present, need OP, if lpp-message-segmentation-req has been received from the location server with bit 1 (targetToServer) set to value 1. The field shall be omitted if lpp-message-segmentation-req has not been received in this location session, or has been received with bit 1 (targetToServer) set to value 0. |
| IntReq | This field is mandatory present if the target device supports IntegrityRequirements; otherwise it is not present. |

CommonIEsProvideCapabilities field descriptions

segmentationInfo This field indicates whether this ProvideCapabilities message is one of many segments, as specified in clause 4.3.5.

lpp-message-segmentation This field, if present, indicates the target device's LPP message segmentation capabilities. If bit 0 is set to value 1, it indicates that the target device supports receiving segmented LPP messages; if bit 0 is set to value 0 it indicates that the target device does not support receiving segmented LPP messages. If bit 1 is set to value 1, it indicates that the target device supports sending segmented LPP messages; if bit 1 is set to value 0 it indicates that the target device does not support sending segmented LPP messages.

integrityRequirementsSup This field specifies that position estimate reliability determination based on provided integrity requirements is supported by the target device.

#### –	CommonIEsRequestAssistanceData

The CommonIEsRequestAssistanceData carries common IEs for a Request Assistance Data LPP message Type.

-- ASN1START

CommonIEsRequestAssistanceData ::= SEQUENCE {

primaryCellID		ECGI		OPTIONAL,	-- Cond EUTRA

...,

[[

segmentationInfo-r14		SegmentationInfo-r14		OPTIONAL	-- Cond Segmentation

]],

[[

periodicAssistanceDataReq-r15

PeriodicAssistanceDataControlParameters-r15

OPTIONAL,	-- Cond PerADreq

primaryCellID-r15			NCGI-r15					OPTIONAL	-- Cond NR

]] ,

[[

integrityRequirementsReq-r17	NULL					OPTIONAL

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| EUTRA | The field is mandatory present for E-UTRA or NB-IoT access. The field shall be omitted for non-EUTRA and non-NB-IoT user plane support. |
| Segmentation | This field is optionally present, need OP, if lpp-message-segmentation-req has been received from the location server with bit 1 (targetToServer) set to value 1. The field shall be omitted if lpp-message-segmentation-req has not been received in this location session, or has been received with bit 1 (targetToServer) set to value 0. |
| PerADreq | The field is mandatory present if the target device requests periodic assistance data delivery. Otherwise it is not present. |
| NR | The field is mandatory present for NR access. The field shall be omitted for non-NR user plane support. |
| IntReqReq | This field is mandatory present if the target device requests integrity requirements for position estimate reliability determination; otherwise it is not present. |

CommonIEsRequestAssistanceData field descriptions

primaryCellID This parameter identifies the current primary cell for the target device.

segmentationInfo This field indicates whether this RequestAssistanceData message is one of many segments, as specified in clause 4.3.5.

periodicAssistanceDataReq This field indicates a request for periodic assistance data delivery, as specified in clause 5.2.1a.

integrityRequirementsReq This field, if present, indicates a request for integrity requirements..

#### –	CommonIEsProvideAssistanceData

The CommonIEsProvideAssistanceData carries common IEs for a Provide Assistance Data LPP message Type.

-- ASN1START

CommonIEsProvideAssistanceData ::= SEQUENCE {

...,

[[

segmentationInfo-r14		SegmentationInfo-r14		OPTIONAL	-- Need ON

]],

[[

periodicAssistanceData-r15	PeriodicAssistanceDataControlParameters-r15

OPTIONAL	-- Cond PerAD

]] ,

[[

integrityRequirements-r17	IntegrityRequirements-r17	OPTIONAL

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| PerAD | The field is mandatory present in a periodic assistance data delivery session. Otherwise it is not present. |

CommonIEsRequestAssistanceData field descriptions

segmentationInfo This field indicates whether this ProvideAssistanceData message is one of many segments, as specified in clause 4.3.5.

periodicAssistanceData This field indicates a periodic assistance data delivery, as specified in clauses 5.2.1a and 5.2.2a.

integrityRequirements This field provides integrity requirements which the device shall use for UE-based position estimate reliability deterimation if the device has indicated support via IntegrityRequirementsSup or requested the integrity requirements via integrityRequirementsReq.

[…]

### 6.4.3	Common NR Positioning Information Elements

[…]

#### –	DL-PRS-ID-Info

The IE DL-PRS-ID-Info provides the IDs of the reference TRPs' DL-PRS Resources.

-- ASN1START

DL-PRS-ID-Info-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255),

nr-DL-PRS-ResourceID-List-r16	SEQUENCE (SIZE (1..nrMaxResourceIDs-r16)) OF

NR-DL-PRS-ResourceID-r16

OPTIONAL, -- Need ON

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16

OPTIONAL  -- Need ON

}

-- ASN1STOP

DL-PRS-ID-Info field descriptions

nr-DL-PRS-ResourceID-List This field provides a list of DL-PRS Resource IDs under the same DL-PRS Resource Set.

#### –	IntegrityRequirements

The IE IntegrityRequirements provides the requirement parameters the device needs to assess positioning integrity

-- ASN1START

IntegrityRequirements-r17 ::= SEQUENCE {

targetIntegrityRisk-r17     		INTEGER (10..90),

horizontalAlertLimit-r17			INTEGER (0..50000),

verticalAlertLimit-r17				INTEGER (0..50000)				OPTIONAL, -- Need ON

timeToAlert-r17			        	INTEGER (1,2000)				OPTIONAL, -- Need ON

...

}

-- ASN1STOP

IntegrityRequirements field descriptions

targetIntegrityRisk This field indicates the Target Integrity Risk (TIR) for the integrity principle of operation by the device. The TIR is calculated by P=10-0.1n [hour-1] where n is the value of targetIntegrityRisk and the range is 10-1 to 10-9 per hour.

horizontalAlertLimit This field indicates the horizontal alert limit for the integrity principle of operation by the device along the semi-major axis of the error ellipse. Scale factor 0.01 metre; range 0 – 500 metres.

verticalAlertLimit This field indicates the vertical alert limit for the integrity principle of operation by the device. Scale factor 0.01 metre; range 0 – 500 metres. To be compared to the horizontal protection level determined by the device.

timeToAlert The maximum allowable elapsed time from when the protection level (PL) exceeds the Alert Limit (AL) until the function providing positioning integrity annunciates a corresponding alert. Scale factor 0.1 second.

# Annex B – Text proposals to 37.355

## B.1 For RequestLocationInformation for each RAT positioning method.

-- ASN1START

NR-DL-TDOA-RequestLocationInformation-r16 ::= SEQUENCE {

nr-DL-PRS-RstdMeasurementInfoRequest-r16	ENUMERATED { true }				OPTIONAL,-- Need ON

nr-RequestedMeasurements-r16				BIT STRING { prsrsrpReq (0),

firstPathRsrpReq-r17 (1)

} (SIZE(1..8)),

nr-AssistanceAvailability-r16				BOOLEAN,

nr-DL-TDOA-ReportConfig-r16					NR-DL-TDOA-ReportConfig-r16		OPTIONAL, -- Need ON

additionalPaths-r16							ENUMERATED { requested }		OPTIONAL, -- Need ON

...,

[[

nr-UE-RxTEG-Request-r17						ENUMERATED { requested }		OPTIONAL, -- Need ON

nr-los-nlos-IndicatorRequest-r17	SEQUENCE {

type-r17			LOS-NLOS-IndicatorType1-r17,

granularity-r17	LOS-NLOS-IndicatorGranularity1-r17,

...

}										OPTIONAL, -- Need ON

additionalPathsExt-r17						ENUMERATED { requested }		OPTIONAL, -- Need ON

additionalPathsDL-PRS-RSRP-Request-r17		ENUMERATED { requested }		OPTIONAL, -- Need ON

multiMeasInSameReport-r17					ENUMERATED { requested }		OPTIONAL  -- Need ON

]]

[[

nr-RSTD-Error-Request-r18					ENUMERATED { requested }		OPTIONAL  -- Need ON

]]

}

NR-DL-TDOA-ReportConfig-r16 ::= SEQUENCE {

maxDL-PRS-RSTD-MeasurementsPerTRPPair-r16	INTEGER (1..4)					OPTIONAL, -- Need ON

timingReportingGranularityFactor-r16 		INTEGER (0..5)					OPTIONAL, -- Need ON

...,

[[

measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17

ENUMERATED { n0, n2, n3, n4, n6, n8, ... }

OPTIONAL, -- Need ON

reducedDL-PRS-ProcessingSamples-r17			ENUMERATED { requested, ... }	OPTIONAL, -- Need ON

lowerRxBeamSweepingFactor-FR2-r17			ENUMERATED { requested } 		OPTIONAL  -- Need ON

]]

}

-- ASN1STOP

NR-DL-TDOA-RequestLocationInformation field descriptions

nr-RSTD-Error-Request This field indicates whether the target device is requested to report error source statistics of the RSTD measurements.

-- ASN1START

NR-Multi-RTT-RequestLocationInformation-r16 ::= SEQUENCE {

nr-UE-RxTxTimeDiffMeasurementInfoRequest-r16

ENUMERATED { true }					OPTIONAL, -- Need ON

nr-RequestedMeasurements-r16		BIT STRING { prsrsrpReq (0),

firstPathRsrpReq-r17 (1) } (SIZE(1..8)),

nr-AssistanceAvailability-r16		BOOLEAN,

nr-Multi-RTT-ReportConfig-r16		NR-Multi-RTT-ReportConfig-r16,

additionalPaths-r16					ENUMERATED { requested }			OPTIONAL, -- Need ON

...,

[[

nr-UE-RxTxTEG-Request-r17			ENUMERATED { case1, case2, case3, ... }

OPTIONAL, -- Need ON

measureSameDL-PRS-ResourceWithDifferentRxTxTEGs-r17

ENUMERATED { n0, n2, n3, n4, n6, n8, ... }

OPTIONAL, -- Need ON

measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17

ENUMERATED { n0, n2, n3, n4, n6, n8, ... }

OPTIONAL, -- Need ON

reducedDL-PRS-ProcessingSamples-r17

ENUMERATED { requested, ... }		OPTIONAL, -- Need ON

nr-los-nlos-IndicatorRequest-r17	SEQUENCE {

type-r17		LOS-NLOS-IndicatorType1-r17,

granularity-r17	LOS-NLOS-IndicatorGranularity1-r17,

...

}									OPTIONAL, -- Need ON

additionalPathsExt-r17				ENUMERATED { requested }			OPTIONAL, -- Need ON

additionalPathsDL-PRS-RSRP-Request-r17

ENUMERATED { requested }			OPTIONAL, -- Need ON

multiMeasInSameReport-r17			ENUMERATED { requested }			OPTIONAL, -- Need ON

lowerRxBeamSweepingFactor-FR2-r17	ENUMERATED { requested }			OPTIONAL  -- Need ON

]]

[[

nr-UE-RxTxTimeDiff-Error-Request-r18 ENUMERATED { requested }			OPTIONAL  -- Need ON

]]

}

NR-Multi-RTT-ReportConfig-r16 ::= SEQUENCE {

maxDL-PRS-RxTxTimeDiffMeasPerTRP-r16 	INTEGER (1..4)					OPTIONAL, -- Need ON

timingReportingGranularityFactor-r16 	INTEGER (0..5)					OPTIONAL  -- Need ON

}

-- ASN1STOP

NR-Multi-RTT-RequestLocationInformation field descriptions

nr-UE-RxTxTimeDiff-Error-Request This field indicates whether the target device is requested to report error source statistics of the UE Rx-Tx time difference measurements.

-- ASN1START

NR-DL-AoD-RequestLocationInformation-r16 ::= SEQUENCE {

nr-AssistanceAvailability-r16				BOOLEAN,

nr-DL-AoD-ReportConfig-r16					NR-DL-AoD-ReportConfig-r16,

...,

[[

multiMeasInSameReport-r17					ENUMERATED { requested }	OPTIONAL  -- Need ON

]]

[[

nr-DL-PRS-RSRPP-Error-Request-r18 			ENUMERATED { requested }	OPTIONAL  -- Need ON

nr-DL-PRS-RSRP-Error-Request-r18 			ENUMERATED { requested }	OPTIONAL  -- Need ON

]]

}

NR-DL-AoD-ReportConfig-r16 ::= SEQUENCE {

maxDL-PRS-RSRP-MeasurementsPerTRP-r16		INTEGER (1..8)				OPTIONAL, -- Need ON

...,

[[

maxDL-PRS-RSRP-MeasurementsPerTRP-r17		INTEGER (9..24)				OPTIONAL, -- Need ON

maxDL-PRS-RSRPP-MeasurementsPerTRP-r17		INTEGER (1..24)				OPTIONAL, -- Need ON

nr-los-nlos-IndicatorRequest-r17			SEQUENCE {

type-r17	LOS-NLOS-IndicatorType1-r17,

granularity-r17

LOS-NLOS-IndicatorGranularity1-r17,

...

}							OPTIONAL, -- Need ON

reducedDL-PRS-ProcessingSamples-r17			ENUMERATED { requested, ... }

OPTIONAL, -- Need ON

lowerRxBeamSweepingFactor-FR2-r17			ENUMERATED { requested }	OPTIONAL  -- Need ON

]]

}

-- ASN1STOP

NR-DL-AoD-RequestLocationInformation field descriptions

nr-DL-PRS-RSRPP-Error-Request This field indicates whether the target device is requested to report error source statistics of the DL-PRS RSRPP of the first path.

nr-DL-PRS-RSRP-Error-Request This field indicates whether the target device is requested to report error source statistics of the DL-PRS RSRP.

## B.2 Error source statistics in the SignalMeasurementInformation message for the corresponding positioning method

#### –	nr-Timing-Error-Source

The IE nr-Timing-Error-Source characterizes the mean and standard deviation of a normal distribution.

-- ASN1START

nr-Timing-Error-Source-r18 ::= SEQUENCE {

mean 							INTEGER(FFS)

Standard-deviation      		INTEGER(FFS)

}}

-- ASN1STOP

-- ASN1START

NR-DL-TDOA-SignalMeasurementInformation-r16 ::= SEQUENCE {

dl-PRS-ReferenceInfo-r16		DL-PRS-ID-Info-r16,

nr-DL-TDOA-MeasList-r16			NR-DL-TDOA-MeasList-r16,

...,

[[

nr-UE-RxTEG-TimingErrorMargin-r17	TEG-TimingErrorMargin-r17		OPTIONAL	-- Cond UERxTEG

]]

}

NR-DL-TDOA-MeasList-r16 ::= SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-DL-TDOA-MeasElement-r16

NR-DL-TDOA-MeasElement-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255),

nr-PhysCellID-r16				NR-PhysCellID-r16								OPTIONAL,

nr-CellGlobalID-r16				NCGI-r15										OPTIONAL,

nr-ARFCN-r16					ARFCN-ValueNR-r15								OPTIONAL,

nr-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16	 					OPTIONAL,

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16						OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-RSTD-r16						CHOICE {

k0-r16						INTEGER (0..1970049),

k1-r16						INTEGER (0..985025),

k2-r16						INTEGER (0..492513),

k3-r16						INTEGER (0..246257),

k4-r16						INTEGER (0..123129),

k5-r16						INTEGER (0..61565),

...

},

nr-AdditionalPathList-r16		NR-AdditionalPathList-r16						OPTIONAL,

nr-TimingQuality-r16			NR-TimingQuality-r16,

nr-DL-PRS-RSRP-Result-r16		INTEGER (0..126)								OPTIONAL,

nr-DL-TDOA-AdditionalMeasurements-r16

NR-DL-TDOA-AdditionalMeasurements-r16			OPTIONAL,

...,

[[

nr-UE-Rx-TEG-ID-r17					INTEGER (0..maxNumOfRxTEGs-1-r17)			OPTIONAL,

nr-DL-PRS-FirstPathRSRP-Result-r17	INTEGER (0..126)							OPTIONAL,

nr-los-nlos-Indicator-r17			CHOICE {

perTRP-r17						LOS-NLOS-Indicator-r17,

perResource-r17					LOS-NLOS-Indicator-r17

}																				OPTIONAL,

nr-AdditionalPathListExt-r17		NR-AdditionalPathListExt-r17				OPTIONAL,

nr-DL-TDOA-AdditionalMeasurementsExt-r17

NR-DL-TDOA-AdditionalMeasurementsExt-r17	OPTIONAL

]]

[[

nr-RSTD-Error-Source-r18			nr-Timing-Error-Source-r18					OPTIONAL,

]]

}

NR-DL-TDOA-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..3)) OF

NR-DL-TDOA-AdditionalMeasurementElement-r16

NR-DL-TDOA-AdditionalMeasurementsExt-r17 ::= SEQUENCE (SIZE (1..maxAddMeasTDOA-r17)) OF

NR-DL-TDOA-AdditionalMeasurementElement-r16

NR-DL-TDOA-AdditionalMeasurementElement-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16	 					OPTIONAL,

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16 					OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-RSTD-ResultDiff-r16			CHOICE {

k0-r16						INTEGER (0..8191),

k1-r16						INTEGER (0..4095),

k2-r16						INTEGER (0..2047),

k3-r16						INTEGER (0..1023),

k4-r16						INTEGER (0..511),

k5-r16						INTEGER (0..255),

...

},

nr-TimingQuality-r16			NR-TimingQuality-r16,

nr-DL-PRS-RSRP-ResultDiff-r16	INTEGER (0..61)									OPTIONAL,

nr-AdditionalPathList-r16		NR-AdditionalPathList-r16						OPTIONAL,

...,

[[

nr-UE-Rx-TEG-ID-r17				INTEGER (0..maxNumOfRxTEGs-1-r17)				OPTIONAL,

nr-DL-PRS-FirstPathRSRP-ResultDiff-r17

INTEGER (0..61)									OPTIONAL,

nr-los-nlos-IndicatorPerResource-r17

LOS-NLOS-Indicator-r17							OPTIONAL,

nr-AdditionalPathListExt-r17	NR-AdditionalPathListExt-r17					OPTIONAL

]]

[[

nr-RSTD-ResultDiff-Error-Source-r18 nr-Timing-Error-Source-r18					OPTIONAL,

]]

}

-- ASN1STOP

NR-DL-TDOA-SignalMeasurementInformation field descriptions

nr-RSTD-Error-Source This field specifies the mean and standard deviation of the nr-RSTD for RAT-dependent positioning integrity.

nr-RSTD-ResultDiff-Error-Source This field specifies the mean and standard deviation of the nr-RSTD-ResultDiff for RAT-dependent positioning integrity.

-- ASN1START

NR-Multi-RTT-SignalMeasurementInformation-r16 ::= SEQUENCE {

nr-Multi-RTT-MeasList-r16		NR-Multi-RTT-MeasList-r16,

nr-NTA-Offset-r16				ENUMERATED { nTA1, nTA2, nTA3, nTA4, ... }		OPTIONAL,

...,

[[

nr-SRS-TxTEG-Set-r17			SEQUENCE (SIZE(1..maxTxTEG-Sets-r17)) OF

NR-SRS-TxTEG-Element-r17					OPTIONAL

-- Cond Case2-3

]],

[[

nr-UE-RxTEG-TimingErrorMargin-r17	TEG-TimingErrorMargin-r17		OPTIONAL,-- Cond TEGCase3

nr-UE-TxTEG-TimingErrorMargin-r17	TEG-TimingErrorMargin-r17		OPTIONAL,-- Cond TEGCase2-3

nr-UE-RxTxTEG-TimingErrorMargin-r17	RxTxTEG-TimingErrorMargin-r17	OPTIONAL -- Cond TEGCase1-2

]]

}

NR-Multi-RTT-MeasList-r16 ::= SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-Multi-RTT-MeasElement-r16

NR-Multi-RTT-MeasElement-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255),

nr-PhysCellID-r16				NR-PhysCellID-r16								OPTIONAL,

nr-CellGlobalID-r16				NCGI-r15										OPTIONAL,

nr-ARFCN-r16					ARFCN-ValueNR-r15								OPTIONAL,

nr-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16						OPTIONAL,

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16 					OPTIONAL,

nr-UE-RxTxTimeDiff-r16			CHOICE {

k0-r16						INTEGER (0..1970049),

k1-r16						INTEGER (0..985025),

k2-r16						INTEGER (0..492513),

k3-r16						INTEGER (0..246257),

k4-r16						INTEGER (0..123129),

k5-r16						INTEGER (0..61565),

...

},

nr-AdditionalPathList-r16		NR-AdditionalPathList-r16						OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-TimingQuality-r16			NR-TimingQuality-r16,

nr-DL-PRS-RSRP-Result-r16		INTEGER (0..126)								OPTIONAL,

nr-Multi-RTT-AdditionalMeasurements-r16

NR-Multi-RTT-AdditionalMeasurements-r16			OPTIONAL,

...,

[[

nr-UE-RxTx-TEG-Info-r17				NR-UE-RxTx-TEG-Info-r17						OPTIONAL,

nr-DL-PRS-FirstPathRSRP-Result-r17	INTEGER (0..126)							OPTIONAL,

nr-los-nlos-Indicator-r17			CHOICE {

perTRP-r17					LOS-NLOS-Indicator-r17,

perResource-r17				LOS-NLOS-Indicator-r17

}																				OPTIONAL,

nr-AdditionalPathListExt-r17		NR-AdditionalPathListExt-r17				OPTIONAL,

nr-Multi-RTT-AdditionalMeasurementsExt-r17

NR-Multi-RTT-AdditionalMeasurementsExt-r17	OPTIONAL

]]

[[

nr-UE-RxTxTimeDiff-Error-Source-r18	nr-Timing-Error-Source-r18					OPTIONAL,

]]

}

NR-Multi-RTT-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..3)) OF

NR-Multi-RTT-AdditionalMeasurementElement-r16

NR-Multi-RTT-AdditionalMeasurementsExt-r17 ::= SEQUENCE (SIZE (1..maxAddMeasRTT-r17)) OF

NR-Multi-RTT-AdditionalMeasurementElement-r16

NR-Multi-RTT-AdditionalMeasurementElement-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceID-r16			NR-DL-PRS-ResourceID-r16					OPTIONAL,

nr-DL-PRS-ResourceSetID-r16			NR-DL-PRS-ResourceSetID-r16 				OPTIONAL,

nr-DL-PRS-RSRP-ResultDiff-r16		INTEGER (0..61)								OPTIONAL,

nr-UE-RxTxTimeDiffAdditional-r16	CHOICE {

k0-r16							INTEGER (0..8191),

k1-r16							INTEGER (0..4095),

k2-r16							INTEGER (0..2047),

k3-r16							INTEGER (0..1023),

k4-r16							INTEGER (0..511),

k5-r16							INTEGER (0..255),

...

},

nr-TimingQuality-r16				NR-TimingQuality-r16,

nr-AdditionalPathList-r16			NR-AdditionalPathList-r16					OPTIONAL,

nr-TimeStamp-r16					NR-TimeStamp-r16,

...,

[[

nr-UE-RxTx-TEG-Info-r17					NR-UE-RxTx-TEG-Info-r17			OPTIONAL,

nr-DL-PRS-FirstPathRSRP-ResultDiff-r17	INTEGER (0..61)					OPTIONAL,

nr-los-nlos-IndicatorPerResource-r17	LOS-NLOS-Indicator-r17			OPTIONAL,

nr-AdditionalPathListExt-r17			NR-AdditionalPathListExt-r17	OPTIONAL

]]

[[

nr-UE-RxTxTimeDiffAdditional-Error-Source-r18	nr-Timing-Error-Source-r18	OPTIONAL,

]]

}

NR-SRS-TxTEG-Element-r17 ::= SEQUENCE {

nr-TimeStamp-r17			NR-TimeStamp-r16							OPTIONAL,	-- Need OP

nr-UE-Tx-TEG-ID-r17			INTEGER (0..maxNumOfTxTEGs-1-r17),

carrierFreq-r17				SEQUENCE {

absoluteFrequencyPointA-r17		ARFCN-ValueNR-r15,

offsetToPointA-r17				INTEGER (0..2199)

}											OPTIONAL,

srs-PosResourceList-r17		SEQUENCE (SIZE (1..maxNumOfSRS-PosResources-r17)) OF

INTEGER (0..maxNumOfSRS-PosResources-1-r17),

...

}

NR-UE-RxTx-TEG-Info-r17 ::= CHOICE {

case1-r17				SEQUENCE {

nr-UE-RxTx-TEG-ID-r17	INTEGER (0..maxNumOfRxTxTEGs-1-r17)

},

case2-r17				SEQUENCE {

nr-UE-RxTx-TEG-ID-r17	INTEGER (0..maxNumOfRxTxTEGs-1-r17),

nr-UE-Tx-TEG-Index-r17	INTEGER (1..maxTxTEG-Sets-r17)

},

case3-r17				SEQUENCE {

nr-UE-Rx-TEG-ID-r17		INTEGER (0..maxNumOfRxTEGs-1-r17),

nr-UE-Tx-TEG-Index-r17	INTEGER (1..maxTxTEG-Sets-r17)

},

...

}

-- ASN1STOP

NR-Multi-RTT-SignalMeasurementInformation field descriptions

nr-UE-RxTxTimeDiff-Error-Source This field specifies the mean and standard deviation of the nr-UE-RxTxTimeDiff for RAT-dependent positioning integrity.

nr-UE-RxTxTimeDiffAdditional-Error-Source This field specifies the mean and standard deviation of the nr-UE-RxTxTimeDiffAdditional for RAT-dependent positioning integrity.