3GPP TSG-RAN WG2 Meeting #121	R2-2301309

Athens, Greece February 27th – March 3rd, 2023

Agenda Item:	8.2.5

Source:	Ericsson

Title:	RedCap positioning, carrier phase positioning, and bandwidth aggregation for positioning

Document for:	Discussion, Decision

# 1   Introduction

One Rel-18 work item on expanded and improved NR positioning has been approved in RAN#98 [1], and its objective on reduced capability (RedCap) positioning, carrier phase positioning, and bandwidth aggregation for positioning are as follows:

- Specify support of positioning for UEs with Reduced Capabilities (RedCap UEs) - Specify support of Frequency Hopping (FH) beyond maximum RedCap UE bandwidth for reception of DL PRS and transmission of UL SRS for positioning [RAN1, RAN2]. - NOTE: The complexity of the corresponding capabilities for RedCap UEs should be addressed for the introduction of appropriate capabilities for RedCap UEs. - Specify RRM requirements for positioning including RRM measurements and procedures for RedCap UEs for both with and without frequency hopping [RAN4]. - Specify physical layer measurements and signalling to support NR DL and UL carrier phase positioning for UE-based, UE-assisted, and NG-RAN node assisted positioning [RAN1, RAN2, RAN3, RAN4]. - Existing DL PRS and UL SRS for positioning are used for NR carrier phase measurements. - Specify measurements that are limited to a single carrier/PFL. - Specify bandwidth aggregation for positioning measurements across up to three intra-band contiguous carriers [RAN1, RAN2, RAN4]. - Specify signalling and procedures to support aggregation of PRS/SRS (respectively) resources across PFLs/carriers (respectively) for positioning measurements under the assumption that the signals over aggregated resources are transmitted and received (respectively) using a single RF chain (same antenna) [RAN1, RAN2]. - NOTE: The support of bandwidth aggregation for positioning measurements applies only to timing related measurements (e.g., RSTD, RTOA, and UE/gNB Rx-Tx time difference).

In this contribution, we give our view on the possible LPP signaling on RedCap positioning, carrier phase positioning, and bandwidth aggregation for positioning from RAN2 part of view.

# 2   Discussion

## 2.1 RedCap positioning

RedCap is designed for NR IoT communication to support wearables (e.g. smart watches, wearable medical devices, AR/VR goggles, etc.), industrial wireless sensors and video surveillance, with the requirements of lower data rate than normal NR UEs but higher data rate than LTE-M and NB-IoT, and relatively relaxed latency requirements compared to URLLC as shown in Table 1.

Table 1 Use cases and requirements of RedCap

|  | Data rate | Latency | Availability/reliability | Battery lifetime | Device size |
| --- | --- | --- | --- | --- | --- |
| Wearables | Reference data rate: 5-50 Mbps in downlink and 2-5 Mbps in uplink | Relaxed | - | At least several days and up to 1-2 weeks | Compact form factor |
| Industrial wireless sensors | < 2 Mbps | < 100 ms | 99.99% | At least a few years | - |
| Video surveillance | 2-4 Mbps for economic video and 7.5-25 Mbps for high-end video | < 500 ms | 99%-99.9% | - | - |

The agreed target positioning accuracy requirements for RedCap in Rel-18 to support these use cases with positioning functions are [2]

Agreement - For the purpose of the Rel-18 study - The target accuracy requirements for RedCap UEs for commercial use cases are defined as follows: - Indoor and outdoor - Horizontal position accuracy (< 3 m) for 90% of UEs - Vertical position accuracy (< 3 m) for 90% of UEs - The target accuracy requirements for RedCap UEs for IIoT use cases are defined as follows: - Horizontal position accuracy (<1 m) for 90% of UEs - Vertical position accuracy (< 3 m) for 90% of UEs - Note: the requirements may not be met in all scenarios and use cases

To achieve these target positioning requirements for RedCap UEs, the recommended solutions from the corresponding SI in Rel-18 are partially overlapped DL PRS frequency hopping and partially overlapped UL SRS frequency hopping for DL-TDOA, UL-TDOA and multi-RTT. The partially overlapped DL PRS frequency hopping can be implemented via wideband PRS transmission with narrowband PRS frequency hopping reception. While regarding the partially overlapped UL SRS frequency hopping, narrowband SRS frequency hopping transmission at the UE side as well as corresponding reception at the network side will be expected.

The current LPP Capability IE does not support UE to characterize the capabilities of RX frequency hopping on DL-PRS or TX frequency hopping on UL SRS.

For UL SRS frequency hopping transmission, new or extended signaling to configure the UL SRS configuration are needed between LMF and UE, LMF and gNB, and between UE and gNB.

RAN2 discuss the potential signaling to support UE RX frequency hopping on DL-PRS and TX frequency hopping on UL SRS. FFS wait for RAN1 and RAN4 progress.

## 2.2 Carrier phase positioning

The Rel. 18 SI concluded that multipath/NLOS deteriorates the performance of carrier phase positioning, and it is necessary to consider multipath mitigation for NR carrier phase positioning [3]. The following multipath mitigation methods is suggested during the SI:

-	Identification and separation of the first path and other paths.

-	Reporting of the carrier phase of the first path, and optionally, the additional paths.

-	The use of LOS/NLOS indication for the carrier phase measurements.

-	NOTE: Rel-17 LOS/NLOS indicator can be considered as a starting point.

-	The report of other channel information, such as existing RSRP/RSRPP.

A new carrier phase IE needs to be defined for the measurement report in LPP, and the carrier phase of the first path shall be reported and optionally additional paths.

Joint positioning using carrier phase measurements and some other positioning method can simplify the integer ambiguity problem significantly if e.g., DL-TDOA can be employed to obtain a first coarse position estimate, for instance to reduce the search space of a multi-hypothesis integer resolution algorithm.

The existing positioning measurement reports often has a structure where the measurement is defined for a specific path. It will be natural to extend this structure to include the carrier phase of specific path.

RAN2 discuss the measurement report for carrier phase positioning, FFS wait for RAN1 and RAN4 progress.

## 2.3 Bandwidth aggregation for positioning

Based on Rel. 18 SI, bandwidth aggregation for intra-band contiguous carriers is concluded as feasible for single chain Tx/Rx architectures at both UE and gNB.

From RAN2’s perspective, UE’s capability in terms of bandwidth aggregation for positioning should be added. Besides to efficiently indicate DL-PRS that can be aggregated together in AD message should also be discussed.

RAN2 discuss the signalling of UE capability and mechanism to indicate DL-PRS that can be aggregated together in AD message in bandwidth aggregation for positioning, FFS wait for RAN1 and RAN4 progress.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	The current LPP Capability IE does not support UE to characterize the capabilities of RX frequency hopping on DL-PRS or TX frequency hopping on UL SRS.

Observation 2	For UL SRS frequency hopping transmission, new or extended signaling to configure the UL SRS configuration are needed between LMF and UE, LMF and gNB, and between UE and gNB.

Observation 3	A new carrier phase IE needs to be defined for the measurement report in LPP, and the carrier phase of the first path shall be reported and optionally additional paths.

Observation 4	The existing positioning measurement reports often has a structure where the measurement is defined for a specific path. It will be natural to extend this structure to include the carrier phase of specific path.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 discuss the potential signaling to support UE RX frequency hopping on DL-PRS and TX frequency hopping on UL SRS. FFS wait for RAN1 and RAN4 progress.

Proposal 2	RAN2 discuss the measurement report for carrier phase positioning, FFS wait for RAN1 and RAN4 progress.

Proposal 3	RAN2 discuss the signalling of UE capability and mechanism to indicate DL-PRS that can be aggregated together in AD message in bandwidth aggregation for positioning, FFS wait for RAN1 and RAN4 progress.

# References

RP-223549, “New WID on expanded and improved NR positioning”, RAN#98-e, December 12-16, 2022.

3GPP TR 37.875, “Study on support of reduced capability NR devices”, March 2021.

3GPP TR 38.859 V18.0.0, Study on expanded and improved NR positioning (Release 18)