3GPP TSG-RAN2 Meeting #121	R2-2301310

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.304 | CR | 0860 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Small corrections on coverage-based paging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh4_LTE_eMTC6-Core |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the NB-IoT Rel-17 feature of coverage-based paging, parameter cbp-Index is introduced in both SIB22 (e.g. in pcch-Config-r17) and UE dedicated signalling (e.g. in RRCEarlyDataComplete or RRCConnectionRelease). UE selects the paging carrier by matching the cbp-Index in pcch-Config-r17 with the cbp-Index received via UE dedicated signalling. However, in TS 36.304, there is description that “use the list of carriers in dl-ConfigList configured with pcch-Config-r17 and with cbp-Index equal to the value of cbp-Config”. Based on the above summary for coverage-based paging scheme and also with reference to TS 36.331(there is only IE of cbp-ConfigList but no IE of cbp-Config), it is clear that cbp-Config here is a typo and the correct one should be “cbp-Index received from RRCEarlyDataComplete or RRCConnectionRelease”. Furthermore, since cbp-Index are configured in both SIB22 and UE dedicated signalling, it is not clear which cbp-Index is used in another description of “if Srxlev > nrsrpMin in cbp-ConfigList indexed by value of cbp-Index”. Last, the coverage-based paging in section 7.7 is a NB-IoT specific function, so all the related messages should have a suffix “-NB”, that is also aligned with the ASN.1 signaling in TS 36.331. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To correct “cbp-Config” in section 7.7 to “cbp-Index received from RRCEarlyDataComplete or RRCConnectionRelease”. - To clarify the cbp-Index used in the description of “if Srxlev > nrsrpMin in cbp-ConfigList indexed by value of cbp-Index” is the cbp-Index received from RRCEarlyDataComplete or RRCConnectionRelease. - To add suffix “-NB” to the messages RRCEarlyDataComplete, RRCConnectionRelease in section 7.7. Impact Analysis Impacted functionality: Coverage-based paging. Inter-operability: As the changes are either editorial changes (#1 and #3 changes) or just clarification (#2 change), if the network is implemented the changes and the UE is not or vice versa, no inter-operability issue can be seen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE behaviour is unclear when the NB-IoT Rel-17 feature of coverage-based paging is used. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of the change>

7.7	Coverage based paging

Coverage-based paging carrier selection is only used in the cell in which the UE most recently entered RRC-IDLE triggered by:

-	reception of RRCEarlyDataComplete-NB or RRCConnectionRelease-NB;

-	and the message includes cbp-Index.

Coverage-based paging is enabled when at least one DL carrier in dl-ConfigList is configured with cbp-Index.

When coverage-based paging is used, the UE shall:

-	if cbp-HystTimer is not running:

-	if Srxlev > nrsrpMin in cbp-ConfigList indexed by value of cbp-Index received from RRCEarlyDataComplete-NB or RRCConnectionRelease-NB:

-	use the list of carriers in dl-ConfigList configured with pcch-Config-r17 and with cbp-Index equal to the value of cbp-Index received from RRCEarlyDataComplete-NB or RRCConnectionRelease-NB for carrier selection as described in clause 7.1.

-	use the nB and ue-SpecificDRX-CycleMin configured in the entry of cbp-ConfigList corresponding to the value of cbp-Index.

-	else:

-	use the list of carriers in dl-ConfigList configured with pcch-Config-r14 for carrier selection as described in clause 7.1.

-	else:

-	continue using list of DL carriers previously selected for carrier selection as described in clause 7.1.

-	when UE switches between paging carriers configured with pcch-Config-r14 and paging carriers configured with pcch-Config-r17 for carrier selection:

-	start cbp-HystTimer;

<End of the change>