3GPP TSG-RAN WG2 Meeting #121	R2-2301357

Athens, Greece, 27 February – 03 March 2023

Agenda item:	8.15.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On Sidelink DRX and remaining CAPC issues

WID/SID:	NR_SL_enh2 - Release 18

Document for:	Discussion and Decision

# 1	Introduction

In RAN Meeting #96, the WID on NR sidelink evolution was agreed. It includes the following objective for operation of sidelink in unlicensed spectrum:

2. Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation o Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation  No specific enhancements for Rel-17 resource allocation mechanisms  If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum o The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98. - Note: In sidelink unlicensed operation, the gNB does not perform Type 1 channel access to initiate and share a channel occupancy, neither Type 2 channel access to share an initiated channel occupancy, nor semi-static channel access procedures to access an unlicensed channel.

In this contribution, we discuss aspects related to Sidelink DRX and remaining CAPC issues.

# 2	On Sidelink DRX

In RAN2#120, the following agreements were made in relation to the impact of operation in unlicensed spectrum into the SL-DRX procedure:

Agreements on SL DRX impact 1: If there is one PSFCH resource for a PSSCH, start sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH resource when the SL HARQ feedback is not transmitted due to the LBT failure. 2: RAN2 waits for RAN1 decision/progress for multiple PSFCH resources case Agreements on SL COT sharing: 1: RAN2 will study whether/how LCP is impacted from COT sharing. 2: RAN2 will consider interaction between DRX operation and shared COT.

The Sidelink DRX feature generally consists of a configured DRX period wherein a single DRX active time, and a single DRX off period is present. Hereby stating, that there is no short DRX, or other opportunities for small sleep durations within the DRX active time definition. It was even agreed that the periodic transmissions associated to an SCI are all included in the active time, even though this was less power efficient. Since the current Rel-18 work item does not have DRX enhancements in scope, we think it only make sense to discuss DRX impact on procedures which may enhance the DRX active time.

It has already been agreed to discuss the impact on shared COT or PSFCH and sidelink DRX, however there was no progress in RAN1 regarding both multiple PSFCH and shared COT conditions. As to not diverge from the design of Rel-17 SL DRX RAN2 should at least postpone the discussion of any DRX enhancements until these two aspects are clear.

Proposal 1: RAN2 to deprioritize work on unlicensed operations specific DRX enhancements until at least the multiple PSFCH resources and shared COT conditions design in RAN1 has progressed, or other issues are found which may prolong the UE active time.

# 3	On Remaining CAPC issues

In RAN2#120, the following working assumptions were made in relation to SL CAPC mapping:

Working assumptions: - Mapping PQI 90/91/92/93/21/22/23/55/56/57/58 to CAPC priority class 1. FFS on other SL CAPC mapping criterion. - Mapping PQI 59/61 to CAPC priority class 3. - Mapping PQI 25 to CAPC priority class 2. - Mapping PQI 24/26/60 to CAPC priority class 1 - If PQI-based CAPC mapping is agreed, as in NR-U, the lowest priority CAPC of the logical channel(s) with MAC SDU multiplexed in the TB is used regardless of whether the TB also contains SL MAC CEs in addition to MAC SDUs. - Use the CAPC of the standardized PQI or the CAPC of non-standardized PQI configured in SIB/pre-configuration which best matches the QoS characteristics of the current non-standardized PQI based on one or more QoS characteristics

RAN 2 should confirm the working assumptions, even the mapping of PQI 24/26/60 to CAPC priority class 1 since although the associated PDB is quite relaxed these PQIs still correspond to mission critical services.

Proposal 2: RAN2 to confirm working assumption for the PQI to CAPC mapping.

# 4	Conclusion

This documents has made the following proposals:

Proposal 1: RAN2 to deprioritize work on unlicensed operations specific DRX enhancements until at least the multiple PSFCH resources and shared COT conditions design in RAN1 has progressed, or other issues are found which may prolong the UE active time.

Proposal 2: RAN2 to confirm working assumption for the PQI to CAPC mapping.