3GPP TSG-RAN WG2 Meeting #121	R2-2301369

Athens, Greece, 27th February – 3rd March, 2023

Agenda Item:	8.5.2.1

Source:	MediaTek Inc.

Title:	PDU set characteristics and their usage in RAN

Document for:	Discussion and decision

# 1 Introduction

SA2 have responded [1] to our questions on PDU set characteristics and the granularity at which they can be provided. In this paper, we go over each of those characteristics and how they could be used in RAN. We also touch upon the topic of differential handling of PDU sets based on SA2’s response.

# 2 Discussion

## 2.1 On PDU set characteristics and their potential usage in RAN

Excerpt 1: On PDU set QoS parameters, granularity and usage

Based on the conclusion from the FS_XRM study (See TR 23.700-60), SA2 agreed to define new 5G QoS parameters for PDU Set concept. The PDU Set comprises of one or more PDUs for which the following PDU Set QoS parameters are applicable:

-	PDU Set Delay Budget (PSDB)

-	PDU Set Error Rate (PSER)

-	PDU Set Integrated handling Indication (PSIHI)

SA2 also agrees to define PDU Set importance that is conveyed on per-PDU Set basis.  All the PDU Sets within one QoS flow should apply the same PSER, PSDB and PSIHI.  The PDU Set importance of the different PDU Sets within one QoS flow can be different.

SA2 has agreed that 1) Different types of PDU set can be mapped into the same QoS flow if their PDU set QoS parameters (and other QoS characteristics, e.g. 5QI, ARP) are the same. One QoS flow is associated with one PSER and one PSDB at any time. 2) Different PDU sets within one QoS flow can be associated with different ‘PDU Set importance’ information.

As concluded by SA2 in the FS_XRM study, the PDU Set information ‘PDU Set importance’ may be provided by the UPF to NG-RAN via GTP-U header of user plane packet. It may be used by NG-RAN for PDU Set level packet discarding in presence of congestion.

SA2 defined a new QoS parameter PDU Set Error Rate (PSER) and kindly asks RAN2 to provide feedback on this new QoS parameter in relation to its intended purpose i.e. appropriate link layer protocol configurations.

The PDU Set Error Rate (PSER) defines an upper bound for the rate of PDU Sets that have been processed by the sender of a link layer protocol (e.g. RLC in RAN of a 3GPP access) but that are not successfully delivered by the corresponding receiver to the upper layer (e.g. PDCP in RAN of a 3GPP access). Thus, the PSER defines an upper bound for a rate of non-congestion related packet losses. The purpose of the PSER is to allow for appropriate link layer protocol configurations (e.g. RLC and HARQ in RAN of a 3GPP access).

In Excerpt 1, we have the following feedback on PDU set QoS parameters, their granularity and usage from SA2.

We take a closer at each QoS parameter and how it could be used by RAN:

1. PSDB – QoS flow specific value

This Qos parameter sets the maximum delay a PDU set can experience over the 3GPP system. As agreed in R2#121, this can be used as a trigger for PDU set discard when the PSDB is exceeded for a PDU set.

2. PDU set importance – PDU set specific value

As indicated in SA2’s response highlighted above, this QoS parameter can be used to trigger packet discard in the presence of congestion. Our companion paper [2] outlines how this QoS parameter can be used in those cases where PSDB based discard cannot be used (e.g. when video traffic is differentially encoded).

Observation 1: PSDB and PDU set importance can be used by RAN to trigger PDU discard.

3. PSER – QoS flow specific value

This QoS parameter defines an error rate at the granularity of PDU sets. SA2’s highlighted response indicates that PSER is meant to be used for an appropriate link layer protocol configuration for RLC or HARQ. However, we have already agreed in R2#119bis that the reliability mechanisms that we have in RAN, i.e. HARQ and RLC, do not operate based on PDU sets. Therefore, this QoS parameter is unenforceable from RAN’s perspective. Given that SA2 have asked for our feedback on this parameter, we should respond to them that PSER cannot be enforced by RAN as RLC and HARQ are not based on PDU sets.

Observation 2: PSER is unenforceable in RAN as reliability mechanisms in RAN are not based on PDU sets.

Proposal 1: Respond to SA2 that PSER is unenforceable in RAN.

4. PSIHI – QoS flow specific value

This QoS parameter indicates whether all PDUs of the PDU set are needed by the application layer. Regardless of whether this parameter indicates that all PDUs of the PDU set is needed or not, RAN will always attempt to transmit all packets to the receiver. In case of failure or congestion (e.g. PSDB is exceeded), PDUs from the PDU set may be discarded. However, such a discard mechanism would operate independent of this parameter, i.e.:

- Parameter indicates all packets are needed – In case discard is triggered, transmitter can discard PDUs that haven’t yet been transmitted as its transmission is pointless

- Parameter indicates not all packets are needed – In case discard is triggered, transmitter can discard PDUs that haven’t yet been transmitted as perhaps those PDUs already transmitted are sufficient

This QoS parameter appears to have no impact on RAN operation, and we should respond to SA2 accordingly.

Observation 3: PSIHI has no impact on RAN operation.

Proposal 2: Respond to SA2 that PSIHI has no impact on RAN operation.

## 2.2 On Differential handling of PDU sets in RAN

In R2#121 we had a discussion on the need for differential handling of PDU sets within a DRB, including whether a DRX can be split into multiple LCHs to deal with different QoS parameters. The arguments for splitting a DRB were:

- PSER can be different for different PDU sets within a QoS flow

- PDU set importance can be different for different PDU sets within a QoS flow

SA2 have now clarified that one QoS flow is always associated with one PSER only. This is regardless of whether different types of PDU sets are mapped to a QoS flow or not. SA2 have also clarified that regardless of whether a PDU set is marked as important or not, all PDU sets within the QoS flow have the same error target. PDU set importance is only meant to be used as a trigger to discard PDUs in case of congestion. Given that there is only a single error target for all PDU sets in a QoS flow, the above reasons for differential handling of PDU sets no longer hold true.

Given the response from SA2, we can only conclude that that differential handling of PDU sets in RAN, e.g. by splitting a DRB into multiple LCHs, does not serve any purpose.

Proposal 3: There is no need to split DRBs into multiple LCHs for differential handling of PDU sets in RAN.

# 3 Conclusion

In this contribution we observe that:

Observation 1: PSDB and PDU set importance can be used by RAN to trigger PDU discard.

Observation 2: PSER is unenforceable in RAN as reliability mechanisms in RAN are not based on PDU sets.

Observation 3: PSIHI has no impact on RAN operation.

Based on the observations above, we propose:

Proposal 1: Respond to SA2 that PSER is unenforceable in RAN.

Proposal 2: Respond to SA2 that PSIHI has no impact on RAN operation.

Proposal 3: There is no need to split DRBs into multiple LCHs for differential handling of PDU sets in RAN.

# 4 References

- R2-2300071 – Reply LS on PDU Set Handling (SA2)

- R2-2301371 – PDU discard based on PSDB and PDU set importance (MediaTek)

# 5 Draft LS to SA2

Title:	Reply LS on PDU Set Handling

Response to:	(S2-2301378) Reply LS on PDU Set Handling

Release:	Rel-18

Work Item:	FS_NR_XR_enh

Source:	MediaTek Inc.

To:	SA2

Cc:	SA4, RAN3

Contact Person:

Name:	Pradeep Jose

E-mail Address:	pradeep dot jose at mediatek dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

RAN2 thanks SA2 for their response LS on PDU set handling.

SA2 defined a new QoS parameter PDU Set Error Rate (PSER) and kindly asks RAN2 to provide feedback on this new QoS parameter in relation to its intended purpose i.e. appropriate link layer protocol configurations.

The PDU Set Error Rate (PSER) defines an upper bound for the rate of PDU Sets that have been processed by the sender of a link layer protocol (e.g. RLC in RAN of a 3GPP access) but that are not successfully delivered by the corresponding receiver to the upper layer (e.g. PDCP in RAN of a 3GPP access). Thus, the PSER defines an upper bound for a rate of non-congestion related packet losses. The purpose of the PSER is to allow for appropriate link layer protocol configurations (e.g. RLC and HARQ in RAN of a 3GPP access).

On the request for feedback on PSER, RAN2 would like to provide the following response:

RAN2 feedback: RAN2 have already agreed that RLC and HARQ will not operate based on PDU sets. As a result, PSER cannot be enforced by RAN.

RAN2 also have the following feedback on the PSIHI:

RAN2 feedback: RAN2 does not foresee any impact on RAN operation based on the PSIHI. In general, RAN will attempt to transmit all PDUs to the receiver. In the presence of congestion, PDU discard may be triggered (e.g. based on PSDB expiry or PDU set importance). However, such a discard operation would occur regardless of the PSIHI indication.

2. Actions:

To SA2

ACTION: 	RAN2 kindly asks SA2 to take the above into account and provide feedback, if any.

3. Date of Next TSG RAN WG2 Meetings:

TSG-RAN WG2#121-bis-e	Apr 17 – 26, 2023		Online

TSG-RAN WG2#122	May 22 – 26, 2023		Incheon, KR