3GPP TSG-RAN WG2 Meeting #121	R2-2301371

Athens, Greece, 27th February – 3rd March, 2023	Revision of R2-2211859

Agenda Item:	8.5.2.3

Source:	MediaTek Inc.

Title:	PDU discard based on PSDB and PDU set importance

Document for:	Discussion and decision

# 1 Introduction

Based on the conclusion from the FS_XRM study (See TR 23.700-60), SA2 agreed to define new 5G QoS parameters for PDU Set concept. The PDU Set comprises of one or more PDUs for which the following PDU Set QoS parameters are applicable:

-	PDU Set Delay Budget (PSDB)

-	PDU Set Error Rate (PSER)

-	PDU Set Integrated handling Indication (PSIHI)

SA2 also agrees to define PDU Set importance that is conveyed on per-PDU Set basis.  All the PDU Sets within one QoS flow should apply the same PSER, PSDB and PSIHI.  The PDU Set importance of the different PDU Sets within one QoS flow can be different.

In response to our question on differential handling of PDU sets, SA2 have responsed with the following [1]:

In this document, we discuss how PSDB and PDU Set importance can be used in the RAN, and its impact on user plane operation.

# 2 Discussion

Video frames in an XR stream have stringent associated delay budgets. Once the delay budget is exceeded, it is not useful to present the video frame to the user. In some cases, its presentation may be detrimental to the user’s experience as it could trigger nausea [2]. In some delay critical real time streaming cases, video frames that exceed their delay budget can be considered as lost and there may be limited  value in its transmission. However, in other cases such as when differential encoding is employed, the transmission of the video frame may still be important to process future video frames. We address these cases separately below.

## Case 1: Transmission of the PDU set is not important after PSDB expiry

The PDU-set delay budget (PSDB) defines the maximum delay that a PDU set can experience over the 3GPP link. In cases where the PDU set can be considered as lost after its delay budget is exceeded, discarding PDUs that have not been transmitted by the time the PSDB has been exceeded, can be beneficial to i) reduce congestion and ii) prioritise the transmission of new information.

Observation 1: A partial/complete PDU-set discard operation when the PSDB is exceeded can be useful to reduce congestion and priortise new data in some cases.

Impact on the Downlink

The RAN will be aware of the PSDB as well as the set of PDUs that belong to a PDU set/burst as indicated in SA2’s response. This awareness enables the NW to implement PDU discard mechanisms, which can be completely left to NW implementation.

Impact on the Uplink

Given that the XR device and the modem are typically expected to be the same device or in close proximity to each other, jitter associated with frame arrival is nearly non-existent. When there is no jitter between PDUs that belong to a PDU set, the PSDB metric reduces to existing PDB metric, i.e. all PDUs arrive at the same time and their respective PDB start and end at the same time. In this case, if discard is needed, existing PDCP discard mechanism can be reused to model PDU discard behaviour. RAN awareness of PSDB will enable proper configuration of the PDCP discard timer.

Proposal 1: PDU discard based on PSDB expiry can be left to NW implementation on the downlink and can be enabled using PDCP discard on the uplink.

## Case 2: Transmission of the PDU set is still important after PSDB expiry

In this section, we discuss the case of differential encoded video, where the encoder looks at what has changed from a previous scene (e.g. motion on a screen) and provides only that information to the decoder. The decoder applies this differential information to the current scene to generate the next scene. We use the Group of Pictures (GOP) example in Figure 1 below as a simple illustration of how this works.

Figure 1: Illustrative example of a GOP approach to video

A GOP consists of a set of interdependent frames, consisting of a set of I and P frames, that form part of a video stream. The I frame contains information that is independent of a previous frame (e.g. the entire image to be displayed on the screen). The set of P frames that follow are differentially encoded and consist of that information in the current scene which is different from the preceding frame. In this context, the I frame can be described as an independent frame, while the P frame can be described as a dependent frame. As can be seen above, even if the delay budget associated with presenting the video frame to the end user has been exceeded, transmission of frames are still needed due to their dependencies on earlier frames.

Observation 2: In case of differential encoded video, PDB/PSDB expiry cannot be used as a trigger for PDU discard as the information is still needed to decode future frames.

In case of real time streams, it is also equally important to present the latest information to the end user where possible. This is possible when independently decodable frames such as I-frames arrive. If the RAN is aware of an independent frame (using the PDU Set importance indicator of the PDU set), the transmission of this information to the receiver can be expedited. Given that all existing data in the transmission buffer correspond to older video frames that are now irrelevant to the real time video stream, there is no point in transmitting them any longer and can be dropped as illustrated in Figure 2 below. This will help ease congestion in the network while improving the end-user experience of the real time video stream.

Figure 2: Prioritisation of an independent frame along with data discard to ease congestion

Proposal 2: In cases where PSDB expiry based PDU discard cannot be used (e.g. differentially encoded information), it is useful to drop all earlier PDU sets in the transmission buffer of the transmitter when a PDU set of high importance arrives, to ease congestion and to ensure that newly arriving independent video frames can be provided to the end-user in a timely manner.

When data is discarded, it may result in ‘holes’ in the sequence number space of RLC or PDCP headers. If the RLC or PDCP entity at the receiver side is unaware of the existence of such ‘holes’, it can result in a window stall at the receiver (reassembly/reordering timers) as it waits for this missing data to arrive. Such a window stall at the receiver is undesirable as it results in unnecessary delays to delay-sensitive XR traffic, and should be avoided. Therefore, when PDU discard takes place at the transmitter, it is important that the receiver is made aware of this operation to avoid unnecessary costly delays to the processing of XR traffic.

Observation 3: Window stalls at the receiver entity as a consequence of discarded PDUs by the transmitter entity is undesirable for delay-sensitive XR traffic.

Proposal 3: The receiver entity is notified of any PDUs that are discarded by the transmitter entity to avoid window stalls.

# 3 Conclusion

In this contribution we have the following observations and proposals:

Observation 1: A partial/complete PDU-set discard operation when the PSDB is exceeded can be useful to reduce congestion and priortise new data in some cases.

Proposal 1: PDU discard based on PSDB expiry can be left to NW implementation on the downlink and can be enabled using PDCP discard on the uplink.

Observation 2: In case of differential encoded video, PDB/PSDB expiry cannot be used as a trigger for PDU discard as the information is still needed to decode future frames.

Proposal 2: In cases where PSDB expiry based PDU discard cannot be used (e.g. differentially encoded information), it is useful to drop all earlier PDU sets in the transmission buffer of the transmitter when a PDU set of high importance arrives, to ease congestion and to ensure that newly arriving independent video frames can be provided to the end-user in a timely manner.

Observation 3: Window stalls at the receiver entity as a consequence of discarded PDUs by the transmitter entity is undesirable for delay-sensitive XR traffic.

Proposal 3: The receiver entity is notified of any PDUs that are discarded by the transmitter entity to avoid window stalls.

# 4 References

- R2-2300071 - Reply LS on PDU Set Handling (SA2)

- https://en.wikipedia.org/wiki/Virtual_reality_sickness