3GPP TSG-RAN WG2 Meeting #121                                    R2-2301385

Athens, Greece, 27th February – 3rd March, 2023

Agenda item:		8.2.5

Source:				Samsung

Title:					Discussion on bandwidth aggregation

Document for:		Discussion & Decision

1	Introduction

In the latest version of WID for R18 POS, the objectives for bandwidth aggregation are described as below [1].

- Specify bandwidth aggregation for positioning measurements across up to three intra-band contiguous carriers [RAN1, RAN2, RAN4]. - Specify signalling and procedures to support aggregation of PRS/SRS (respectively) resources across PFLs/carriers (respectively) for positioning measurements under the assumption that the signals over aggregated resources are transmitted and received (respectively) using a single RF chain (same antenna) [RAN1, RAN2]. - NOTE: The support of bandwidth aggregation for positioning measurements applies only to timing related measurements (e.g., RSTD, RTOA, and UE/gNB Rx-Tx time difference). - Specify RRM requirements with measurement gaps in connected mode, and in inactive mode, including PRS measurement period/reporting [RAN4].

In this contribution, we discuss some potential spec. impact in RAN2 regarding bandwidth aggregation.

2	Discussion

2.1	DL PRS configuration

According to the obejectives in WID, the maximum number of intra-band contiguous PFL is three. In release 17, the UE can be configured with one or more DL PRS positioning frequency layers (PFLs) and the maximum number of PFL (i.e., nrMaxFreqLayers) is defined as 4 in LPP [2]. The number of configurable PFLs is already enough for the bandwidth aggregation. Thus, if the separated PRS sequences in each of PFLs are used for the PRS bandwidth aggregation, the legacy PRS configuration per PFL can be reused. On the other hand, if a single PRS sequence across the aggregated PFLs is used, there seems the need of some enhancement in LPP to configure DL PRS transmitted across multiple PFLs.

Observation 1: If the separated PRS sequences in each of PFLs are used for bandwidth aggregation, the existing PRS configuration per PFL can be reused.

Observation 2: If a single PRS sequence across the aggregated PFLs is used for bandwidth aggregation, there seems the need of some enhancement on DL PRS configuration in LPP.

Since there is no agreement on the issue (i.e., use a single PRS sequence across multiple PFLs or separated PRS in each PFL) even in RAN1, RAN2 can discuss any enhancement on DL PRS configuration in LPP later.

Proposal 1: RAN2 to discuss any potential enhancement on DL PRS configuration in LPP after having the conclusion on PRS design for bandwith aggregation from RAN1.

2.2	UL SRS configuration

Regarding the SRS configuration for positioning, the UE is allowed to transmit SRS within the active UL BWP of a CC in RRC_CONNECTED mode in release 16. However, in release 17, the SRS transmissionin RRC_INACTIVE mode is introduced to reduce the power consumption at the UE side. The UE can be configured with SRS resource for positioning inside the initial UL BWP or outside the initial UL BWP with additionally configured frequency location. The SRS resource for positioning outside the initial BWP in RRC_INACTIVE mode is configured in the same band and CC as the initial UL BWP. Meanwhile, for release 18, the WID specify that the SRS for positioning can be transmitted across up to three intra-band contiguous carriers.

With the current RRC specification, SRS resources for positioning can be configured per BWP per serving cell and they can be activated/deactivated in unit of SRS resource set by the MAC CE received at each cell. For the SRS bandwidth aggregation, if the separated SRS sequences in each of carriers are used for bandwidth aggregation, the existing SRS configuration per BWP per serving cell can be reused. The network can just configure the separate SRS configurations for each serving cell in parallel. On the other hand, if a single PRS sequence across the aggregated carriers is used, there seems the need of some enhancement on SRS configuration in RRC to configure SRS transmitted across multiple carriers.

Observation 3: If separated SRS sequences in each of carriers are used for bandwidth aggregation, the existing SRS configuration per BWP per serving cell can be reused.

Observation 4: If a single SRS sequence across the aggregated carriers is used for bandwidth aggregation, there seems the need of some enhancement on SRS configuration in RRC.

As in the case of DL PRS, RAN1 has not made any agreement on the issue (i.e., use a single SRS sequence across multiple carriers or separated SRS in each carrier). Thus, RAN2 can discuss any enhancement on SRS configuration in RRC later.

Proposal 2: RAN2 to discuss any potential enhancement on SRS configuration in RRC after having the conclusion on SRS design for bandwith aggregation from RAN1.

3	Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

Observation 1: If separated PRS sequences in each of PFLs are used for bandwidth aggregation, the existing PRS configuration per PFL can be reused.

Observation 2: If a single PRS sequence across the aggregated PFLs is used for bandwidth aggregation, there seems the need of some enhancement on DL PRS configuration in LPP.

Proposal 1: RAN2 to discuss any potential enhancement on DL PRS configuration in LPP after having the conclusion on PRS design for bandwith aggregation from RAN1.

Observation 3: If separated SRS sequences in each of carriers are used for bandwidth aggregation, the existing SRS configuration per BWP per serving cell can be reused.

Observation 4: If a single SRS sequence across the aggregated carriers is used for bandwidth aggregation, there seems the need of some enhancement on SRS configuration in RRC.

Proposal 2: RAN2 to discuss any potential enhancement on SRS configuration in RRC after having the conclusion on SRS design for bandwith aggregation from RAN1.

4	Reference

- RP-223549, ‘New WID on Expanded and Improved NR Positioning’

- 3GPP TS 37.355 v17.3.0, ‘LTE Positioning Protocol (LPP)’