3GPP TSG RAN2 Meeting #121	R2-2301393

Athens, 27 February – 3 March, 2023

Agenda item:		7.2.1

Source:	Deutsche Telekom, Huawei, HiSilicon, Ericsson, Nokia, Nokia Shanghai Bell, Thales, Vodafone, Telit, BT, Telstra, Telecom Italia, Turkcell

Title:	Considerations on AS deactivation due to discontinuous coverage

Document for:		Discussion and Decision

# Introduction

In the previous RAN Plenary meeting, RAN#98-e, Deutsche Telekom flagged the approval of the agreed CR 0859rev1 to 36.304 (R2-2213080), since the proposed change to paragraph 4.1 referring to the clarification added for idle mode operation during discontinuous coverage, “(e.g., performing intra-frequency, inter-frequency or inter-RAT measurements)”, introduced further unclarity in terms of the UE behaviour when SIB32 is received and the UE is out of coverage. RAN Plenary decided not to include this clarification in the revised CR (RP-223533) and to continue having further discussions/analyses in the next RAN2 meeting on how to better clarify in 36.304 the UE behaviour when SIB32 is received, and the UE is out of NTN coverage.

In this contribution, our intention is to highlight the issue raised with regard to idle mode tasks towards TN and propose an update of the CR in question for discussion and agreement in RAN2.

# Discussion

The agreements made regarding the deactivation of access stratum due to discontinuous coverage have been captured in TS 36.304 in section 4 on “General description of Idle mode”:

“If SystemInformationBlockType32 has been received and if the UE has determined that it is out of coverage using available satellite assistance information (e.g. ephemeris parameters and coverage parameters in current or previously received SystemInformationBlockType32, SystemInformationBlockType31, t-Service in SystemInformationBlockType3 or other parameters), the AS configuration (e.g. priorities provided by dedicated signalling and logged measurements) is kept, but the UE need not perform any idle mode tasks. It is up to UE implementation to handle running timers. The detection of out of coverage using satellite assistance information is up to UE implementation and once in coverage the UE shall perform all idle mode tasks.”

In RAN2#119bis-e, RAN2 received an LS from SA2 (R2-2209191/S2-2207420) on the deactivation of access stratum due to discontinuous coverage and was asked to provide feedback on the applicable cases of AS functions deactivation due to discontinuous coverage. RAN2 discussed the question and informed SA2 that “it is up to UE implementation to deactivate Access Stratum functions due to discontinuous coverage. This applies to any RAT supported by the UE including satellite and terrestrial access.”. Based on this reply, RAN2 approved a CR of TS36.304 (R2-2213080), in which a clarification in paragraph 4.1 was added referring to the idle mode tasks not required to be performed by a UE during discontinuous coverage, “(e.g., performing intra-frequency, inter-frequency or inter-RAT measurements)”; however, RAN Plenary decided not to include this clarification in the revised CR (RP-223533) and to continue having further discussions/analyses in the next RAN2 meeting on how to better clarify in 36.304 the UE behaviour when SIB32 is received, and the UE is out of NTN coverage.

According to the agreed text captured in TS36.304, the UE does not need to perform any idle mode tasks (“… but the UE need not perform any idle mode tasks.”), NTN-related idle mode tasks but also TN-related ones, once in a discontinuous NTN coverage. Thus, the introduction of this sentence changes UE legacy behaviour with regard to being out of coverage that has been in the specification since UMTS times (see TS 25.304, chapters 4.1, 4.2), has been applied to LTE some 10 years ago (see TS 38.304, chapters 4.1, 4.2), and also is consistently required for NR since Rel-15 (see TS 38.304, chapters 4.1, 4.2).

It is also far from clear what “any idle mode tasks” refers to! In our understanding, these are at least the AS functions a UE performs, as defined in 36.304 (and 25.304/38.304), which are at least:

- Measurements for and process of Cell Selection

- Measurements for and process of Cell Reselection

- Measurements for and process of Any Cell Selection

- Measurements for and process of Support of task for NAS (i.e., to support PLMN selection process).

This approach might be acceptable for NTN only cases, as in case of NTN discontinuous coverage, the UE might end up searching for too long while there is no alternative NTN coverage. However, there seems to be an issue with this approach in case of TN and NTN interworking. To be more specific, let’s consider a scenario in which a UE, supporting both TN and NTN, is initially under TN coverage, but loses TN coverage due to coverage hole and ends up on NTN to be served. Suddenly, the UE experiences discontinuous coverage in NTN, but there is TN coverage available again. In such a case, based on the currently agreed text of TS 36.304, it is not ensured that the UE will perform a cell selection or at least an any cell selection on the TN as required in chapter 4.2 of said TS 36.304, even when the UE is back under perfect TN coverage conditions. This is a clear change of legacy TN behaviour, where the UE that drops out of (TN) coverage is required to perform the idle mode tasks as defined in the TS 36.304 introduced by NTN optimizations.

Observation 1: According to the agreed text captured in TS 36.304, it is up to UE implementation (quote “UE need not to perform any idle mode tasks”) and consequently it is not ensured that an NTN/TN capable UE that is out of NTN coverage but under usable TN coverage will perform a cell selection or at least an any cell selection on the TN even when under perfect TN coverage conditions. This functionality clearly changes any legacy behaviour related to TN.

The aim of this contribution is not to overturn what has been agreed in RAN2 with regards to NTN discontinuous coverage and reasonable UE behaviour. However, it cannot be accepted by the co-signing companies, at least from a TN operator’s perspective, to introduce this new functionality of “not performing any idle mode tasks” towards TN as it changes any legacy behaviour related to TN that has been up and running for 20 years.

Proposal 1: Having identified that this new functionality of “not performing any idle mode tasks” when the UE is out of NTN coverage, clearly changes any legacy behaviour related to TN, it is proposed to limit the “UE is not required to perform any AS idle mode tasks” behaviour only to NTN and specifically update the relevant specifications to capture this unambiguously.

# Summary and Conclusions

In this document, we have analyzed how the currently agreed text captured in TS36.304 in terms of the UE behaviour when SIB32 is received and the UE is out of coverage has an impact on legacy behaviour related to TN and we propose the following:

Observation 1: According to the agreed text captured in TS36.304, it is up to UE implementation (quote “UE need not to perform any idle mode tasks”) and consequently it is not ensured that an NTN/TN capable UE that is out of NTN coverage but under usable TN coverage will perform a cell selection or at least an any cell selection on the TN even when under perfect TN coverage conditions. This functionality clearly changes any legacy behaviour related to TN.

Proposal 1: Having identified that this new functionality of “not performing any idle mode tasks” when the UE is out of NTN coverage, clearly changes any legacy behaviour related to TN, it is proposed to limit the “UE is not required to perform any AS idle mode tasks” behaviour only to NTN and specifically update the relevant specifications to capture this unambiguously.

Draft CRs are provided by Deutsche Telekom et al for this meeting [5, 6].

# References

[1]		R2-2213080 | R2-223533 (revision of R2-2213080), “Miscellaneous idle mode corrections”

[2] 		R2-2209191/ S2-2207420, “Reply LS on the deactivation of access stratum due to discontinuous coverage”

[3]		R2-2210865, “Reply LS on the deactivation of access stratum due to discontinuous coverage”

[4]		TS 36.304 (v17.3.0), “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode”

[5]		R2-2301380, Draft CR TS 36.300 “Correction related to AS deactivation due to discontinuous coverage”

[6]		R2-2301388, Draft CR TS 36.304 “Correction related to AS deactivation due to discontinuous coverage”