3GPP TSG-RAN WG2 Meeting #121 						                       R2-2301398

Athens, Greece, February 27 –March 3, 2023

Source: 	Xiaomi

Title:  	Discussion on flight path reporting for NR UAV

Agenda Item:	8.8.3 Flight path reporting

Document for:	Discussion and Decision

Introduction

A WID on NR support for UAV (Uncrewed Aerial Vehicles) [1] has been approved. In the WID, one objective is to enhance measurement reports, shown as below.

1. Specify the following enhancements on measurement reports [RAN2]: - UE-triggered measurement report based on configured height thresholds - Reporting of height, location and speed in measurement report - Flight path reporting - Measurement reporting based on a configured number of cells (i.e. larger than one) fulfilling the triggering criteria simultaneously Note: Work done in LTE is a starting point for this objective. NR-specific enhancements can be considered, if needed, while overall the LTE and NR solutions should be harmonized as much as possible.

In this contribution, we will provide our understandings on the flight path reporting for NR UAV.

Discussion

  - The content of flight path information

In last meeting, RAN2 has discussed the content of flight path information and reached the following agreements [2]:

RAN2#120: 1. A waypoint is a planned location for the UE along the flight path and is described via the existing parameter type LocationCoordinates defined in TS 37.355. 2. A timestamp provides the UTC time associated with estimated time of arrival to a waypoint as baseline. FFS on granularity 3. No requirements are placed on spatial distribution of waypoints 7. FFS The maximum number of waypoints within flight path plan is left FFS.

The meanings of waypoint and timestamp has reached consensus in RAN2#120. However, which waypoints should be included in the flight path information is still FFS. Some options have been provided in previous meetings, e.g. the flight path information contains the granularity of the waypoints (i.e. the distance or interval between consecutive waypoints) and/or provides the information on the destination of the flight. However, it is uncertain whether UE has full flight path information or not e.g. whether the flight path information stored in UE includes the destination of flight and can satisfy the granularity configured by network. And the size and speed of UAV UE may influence over the granularity of waypoint. Hence, whether the requirement of flight path information configured by network can be fulfilled or not depend on the flight path information in UE. It may be difficult to configured appropriate requirements to UE to limit the content of flight path information. And, in LTE UAV, which waypoints are included in flight path information is up to UE implementation. Therefore, no optimization for the content of flight path information needs to be introduced in NR.

Proposal 1: It is up to UE implementation which waypoints are included in flight path information.

RAN2 also need to clarify the maximum number of waypoints within flight path plan. In LTE, the maximum number of waypoints is configured by network via FlightPathInfoReportConfig.

–	FlightPathInfoReportConfig [3]

The IE FlightPathInfoReportConfig specifies flight path information report configuration.

FlightPathInfoReportConfig information element

-- ASN1START

FlightPathInfoReportConfig-r15 ::= SEQUENCE {

maxWayPointNumber-r15				INTEGER (1..maxWayPoint-r15),

includeTimeStamp-r15				ENUMERATED {true}					OPTIONAL

}

-- ASN1STOP

FlightPathInfoReportConfig field descriptions

maxWayPointNumber Indicates the maximum number of way points UE can include in the flight path information report if this information is available at the UE.

includeTimeStamp Indicates whether time stamp of each way point can be reported in the flight path information report if time stamp information is available at the UE.

And the maxWayPoint-r15 is 20 [3].

maxWayPoint-r15				INTEGER ::= 20	-- Maximum number of flight path information waypoints

As in LTE, the maximum number of waypoints within flight path is configured by network and the maximum value is 20 in NR UAV.

Proposal 2: The maximum number of waypoints within flight path is configured by network and the maximum value is 20 in NR UAV.

  - Flight path reporting

In last meeting, RAN2 reached the following agreements about flight path reporting [2]:

RAN2#120: 4. A UE indicates whether flight plan information is available within the RRCReconfigurationComplete, RRCReestablishmentComplete, RRCResumeComplete, or RRCSetupComplete message. Flight path reporting uses at the UE Information request/response procedure as baseline. 5. UE indicates to the network a new flight path is available in the UE (whether it is initial or update). Then, reuse the normal request/response procedure of flight path report. 6. UAI message can also be used to indicate the UE has flight path availability. 7. FFS whether and what triggering conditions are specified for flight update.

RAN2 has agreed that flight path availability can be reported within the RRCReconfigurationComplete, RRCReestablishmentComplete, RRCResumeComplete, or RRCSetupComplete message and UE can indicate the UE has a new flight path availability (whether it is initial or update).

In legacy LTE procedure, the initial flight path availability can be reported within above-mentioned RRC complete messages. However, if UE doesn’t have flight path information available upon reception of the corresponding RRC message, UE will not report the indication of flight path availability, i.e. flightPathInfoAvailable, and then if UE has flight path information available after the transmission of RRC complete message, UE will not report flightPathInfoAvailable unless UE receives a new RRC message e.g. RRCReconfiguration, RRCReestablishment, RRCResume, or RRCSetup, which will cause that network cannot obtain flight path information timely.

In last meeting, RAN2 has agreed that UAI message can be used to indicate the UE has flight path availability. UAI message can be used to report the indication of initial flight path availability. If UE has flight path information available and the UE has not reported the indication in the latest complete message, the UE should report flightPathInfoAvailable via UAI if configured.

Proposal 3：UAI message can be used to report the indication of initial flight path availability. If UE has flight path information available and the UE has not reported the indication in the latest complete message, the UE should report flightPathInfoAvailable via UAI if configured.

UAI message also can be used for the update of flight path and whether and what triggering conditions are specified for flight update is still FFS. For flight update, when UE has a new flight path available, the UE should inform network and network can required the UE to report the new flight path information. If UE has flight path information available and the flight path information in the UE is changed compared to last time the UE reported this information, the UE should report flightPathInfoAvailable via UAI if configured.

Proposal 4: UAI information can be used for flight update. If UE has flight path information available and the flight path information in the UE is changed compared to last time the UE reported this information, the UE should report flightPathInfoAvailable via UAI if configured.

For flight update, RAN2 should discuss how to confirm the flight path information has been changed. Base on this, UE can determine whether to initiate the UE assistance information procedure to indicate the UE has updated flight path information. There are two options can be considered:

- Option 1: It is up to UE implementation.

- Option 2: Any waypoint (location and associated timestamps (if included)) reported in last time is not included in the currently available flight path information.

Proposal 5: RAN2 should discuss how to confirm the flight path information is changed:

- Option 1: It is up to UE implementation.

- Option 2: Any waypoint (location and associated timestamps (if included)) reported in last time is not included in the currently available flight path information.

After aerial UE reports the fight path information, if the aerial UE is controlled by the operator or user, the reported flight path information will become unavailable and the UE won’t have the available flight path. Hence, the UE needs to inform network that the flight path information reported in last time has become unavailable. Without the indication, network may configure handover command based on the unavailable flight path which may result in handover failure. The indication of flight path unavailable can be introduced and UAI message can be used to indicate the UE doesn’t have flight path availability.

Proposal 6: The indication of flight path unavailable can be introduced and UAI message can be used to indicate the UE doesn’t have flight path availability.

To reduce signaling overhead, delta flight path information can be supported for flight path update and the new flight path information is the delta of the flight path information reported in last time. The timestamp of waypoints can be used as the index. If timestamps is not included in the previous report. Full flight path information can be reported.

Proposal 7: Delta flight path information can be supported for flight update. The timestamp of waypoints can be used as the index for delta configuration.

Conclusions

According to the analysis given above, we have the following observations and proposals:

The content of flight path information

Proposal 1: It is up to UE implementation which waypoints are included in flight path information.

Proposal 2: The maximum number of waypoints within flight path is configured by network and the maximum value is 20 in NR UAV.

Flight path reporting

Proposal 3：UAI message can be used to report the indication of initial flight path availability. If UE has flight path information available and the UE has not reported the indication in the latest complete message, the UE should report flightPathInfoAvailable via UAI if configured.

Proposal 4: UAI information can be used for flight update. If UE has flight path information available and the flight path information in the UE is changed compared to last time the UE reported this information, the UE should report flightPathInfoAvailable via UAI if configured.

Proposal 5: RAN2 should discuss how to confirm the flight path information is changed:

- Option 1: It is up to UE implementation.

- Option 2: Any waypoint (location and associated timestamps (if included)) reported in last time is not included in the currently available flight path information.

Proposal 6: The indication of flight path unavailable can be introduced and UAI message can be used to indicate the UE doesn’t have flight path availability.

Proposal 7: Delta flight path information can be supported for flight update. The timestamp of waypoints can be used as the index for delta configuration.

Reference

[1] RP-223545, Nokia, “Revised WID: NR Support for UAV (Uncrewed Aerial Vehicles)”

[2] R2-xxxxxx, “Report of 3GPP TSG RAN WG2 meeting #120”

[3] TS36.331, “Radio Resource Control (RRC) (V17.3.0)”;