3GPP TSG-RAN WG2 #120	Tdoc R2-2301402

Toulouse, November 14 –18, 2022

Agenda Item:	8.21.2

Source:	Ericsson

Title:	On RAN2 aspects for UL TX switching Rel-18

Document for:	Discussion, Decision

# 1	Introduction

In RAN2#120 the following was agreed:

R2 assumes For UE capability to report applicability of DL interruption for Rel-18 UL Tx switching, RAN2 reuses uplinkTxSwitching-DL-Interruption-r16 (no spec impact).

R2 assumes to reuse the per band per BC capability, uplinkTxSwitching2T2T-PUSCH-TransCoherence-r17, on UL-MIMO coherence for the 2Tx-capable UL band(s) for Rel-18 UL Tx switching (fallback description FFS).

This contribution discusses a general aspect for UL TX switching and the open points based on RAN1/4 input.

# 2	Discussion

## 2.1	General aspect

For Rel-17 UL Tx switching, it was concluded that the UE must support at least one common switching option (Option 1:Switched UL or Option 2:Dual UL) between 1Tx-2Tx switching and 2Tx-2Tx switching:

uplinkTxSwitching-OptionSupport2T2T-r17

Indicates which option is supported for dynamic UL 2Tx-2Tx switching for inter-band UL CA. switchedUL represents option 1 as specified in TS 38.214 [12], dualUL represents option 2 as specified in TS 38.214 [12], both represents both option 1 and option2 as specified in TS 38.214 [12]. The field is mandatory for inter-band UL CA cases where UE supports dynamic UL 2Tx-2Tx switching. The UE indicating support of this feature shall indicate support of at least one common switching option between uplinkTxSwitching-OptionSupport2T2T-r17 and uplinkTxSwitching-OptionSupport-r16.

As discussed during RAN2#119-e in R2-2207333, a possible way forward would be to not specify any dependency between Rel-17 and Rel-16 UL Tx switching options. But given the above conclusion, it would be good to follow the same principle for Rel-18 UL Tx Switching for the sake of consistency and avoid repeating the discussion already held for the conclusion in Rel-17. This may be further discussed depending on RAN1 outcome, but it would be good to consider it as a baseline. At this stage, it could not be captured in the same level of detail as for Rel-17, i.e. it is not clear how the case of 3 and 4 bands would be supported. But we should expect at least some dependency between the Rel-18 support of the feature and the Rel-17 support.

As a baseline, RAN2 understands that a UE supporting Rel-18 UL Tx switching would also support at least one switching option (Option 1:Switched UL or Option 2:Dual UL) as defined for the Rel-17 capability of UL Tx switching.

## 2.2	RAN1 progress

On the RAN1 progress, several points were discussed in R1-2210724. This contribution highlights the ones that should have an immediate RAN2 impact.

The following was concluded in RAN1#110bis-e:

- Ask RAN2 to consider following alternatives for UE capability reporting about the supported UL Tx switching options

  - Alt.1: report {switchedUL, dualUL, both} for each band pair in the band combination

  - Alt.2: report {switchedUL, dualUL, both} for the band combination and report supported band pair for concurrent transmission for the band combination

    - Note：If there is no report on the supported band pair(s) for concurrent transmission while the UE reports “dualUL” or “both” for the band combination, gNB may assume that the UE supports concurrent transmission on all the band pairs within the band combination

  - Alt.3: report {dualUL} for each band pair in the band combination

    - Note: Within the band combination, the UE shall be capable of being operated in switched UL mode for all band pairs

- Ask RAN2 to consider following alternatives and specify gNB configuration

  - Alt.1: configure {switchedUL, dualUL} for all serving cells (i.e., for the band combination)

  - Alt.2: configure {switchedUL, dualUL} for combination(s) of serving cells (i.e., for each band pair in the band combination)

  - Alt.3: configure {switchedUL, dualUL} for all serving cells (i.e., for the band combination), and configure combination(s) of serving cells (i.e., as supported serving cell pair(s) for each band pair in the band combination) for concurrent transmission

For the UE capability reporting, since the UE mechanism to report UL Tx switching in Rel-16 and Rel-17 is based on the report of UL Tx switching option per band combination, it would be simpler to adopt the same for Rel-18, i.e. Alt.2. The other options would not only imply in a finer granularity signaling but also one would have to understand the interaction between the newly defined per-band framework with the existing per-BC framework.

The UE capability report of Tx switching option (switchedUL, dualUL, both) is done per BC.

For the UE configuration, since the UL Tx switching configuration in Rel-16 and Rel-17 is performed for the applicable serving cells, it would be simpler to adopt the same for Rel-18, i.e. Alt.2. While Alt.1 may seem simpler, it deviates from the current way of configuring UL Tx switching and thus would end up being more complex to configure the feature in multiple levels. Similarly, Alt.3 would introduce a new logic of combination of serving cells, which may not be compatible with the current Rel-16 and Rel-17 configuration.

The configuration of Tx switching option is done for combination(s) of serving cells (i.e., for each band pair in the band combination).

While there are further discussions ongoing in RAN1, we should expect an updated LS from RAN1 with the full picture of agreements.

## 2.3	RAN4 progress

On the RAN4 progress, two issues were discussed in R4-2220548:

- Impact from switching of one Tx chain on the other Tx chain

- Ambiguity issue when two Tx chains are switched between two different band pairs

The first issue is not concluded yet. While the second issue, as per RAN4 conclusion so far, would not have an impact in RAN2. Hence, RAN2 can wait for further discussion in RAN4.

In RAN2#120, the following agreement was made based on RAN4 progress:

R2 assumes to reuse the per band per BC capability, uplinkTxSwitching2T2T-PUSCH-TransCoherence-r17, on UL-MIMO coherence for the 2Tx-capable UL band(s) for Rel-18 UL Tx switching (fallback description FFS).

We understand the logic considering the Rel-18 UL TX switching would be as:

- If configuring Rel-18 UL TX switching, the agreed behaviour applies (i.e. apply uplinkTxSwitching2T2T-PUSCH-TransCoherence-r17. If uplinkTxSwitching2T2T-PUSCH-TransCoherence-r17 is absent, the Rel-15 per band UE capability pusch-TransCoherence is applicable).

- If configuring Rel-17 UL Tx switching, the current behaviour already captured in 38.306 applies (i.e. apply uplinkTxSwitching2T2T-PUSCH-TransCoherence-r17. If uplinkTxSwitching2T2T-PUSCH-TransCoherence-r17 is absent, the per BC UE capability reported in uplinkTxSwitching-PUSCH-TransCoherence-r16 is applied, and if uplinkTxSwitching2T2T-PUSCH-TransCoherence-r17 and uplinkTxSwitching-PUSCH-TransCoherence-r16 are both absent, the UE capability reported in pusch-TransCoherence is applied).

While generally we should capture the UE capabilities based on what the UE supports and not network configuration, there are exceptional cases. Particularly in this case, it would be clearer to capture the field description based on the configuration of Rel-18 UL TX switching. An example is provided below. Note the example assumes that a new field would be needed for the configuration of the Rel-18 UL Tx switching options, but if that is not the case, some other field could be used as reference.

| UplinkTxSwitchingBandParameters-v1700 Contains the UL Tx switching specific band parameters for a given band combination. The capability signalling comprises of the following parameters: - bandIndex-r17 indicates a band on which UE supports dynamic UL Tx switching with another band in the band combination. bandIndex xx refers to the xxth band entry in the band combination. - uplinkTxSwitching2T2T-PUSCH-TransCoherence-r17 indicates support of the uplink codebook subset for the carrier(s) on a band capable of two antenna connectors on which UE supports dynamic UL 2Tx-2Tx switching with another band in the band combination. UE indicating support of full coherent codebook subset shall also support non-coherent codebook subset. If the network configures uplinkTxSwitching-r16: if this field is absent, the per BC UE capability reported in uplinkTxSwitching-PUSCH-TransCoherence-r16 is applied, and if this field and uplinkTxSwitching-PUSCH-TransCoherence-r16 are both absent, the UE capability reported in pusch-TransCoherence is applied when uplink Tx switching is triggered between last transmitted SRS and scheduled PUSCH transmission, as specified in TS 38.101-1 [2]. If the network configures uplinkTxSwitching-r18: if this field is absent, the UE capability reported in pusch-TransCoherence is applied when uplink Tx switching is triggered between last transmitted SRS and scheduled PUSCH transmission, as specified in TS 38.101-1 [2]. | BC | No | N/A | FR1 only |
| --- | --- | --- | --- | --- |

Capture the fallback description in 38.306 of Rel-18 UL Tx switching for UL MIMO coherence based on the configuration of Rel-18 UL Tx switching i.e. if configuring Rel-18 UL TX switching, apply uplinkTxSwitching2T2T-PUSCH-TransCoherence-r17. If configuring Rel-18 UL TX switching and uplinkTxSwitching2T2T-PUSCH-TransCoherence-r17 is absent, the Rel-15 per band UE capability pusch-TransCoherence is applicable.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	As a baseline, RAN2 understands that a UE supporting Rel-18 UL Tx switching would also support at least one switching option (Option 1:Switched UL or Option 2:Dual UL) as defined for the Rel-17 capability of UL Tx switching.

Proposal 2	The UE capability report of Tx switching option (switchedUL, dualUL, both) is done per BC.

Proposal 3	The configuration of Tx switching option is done for combination(s) of serving cells (i.e., for each band pair in the band combination).

Proposal 4	Capture the fallback description in 38.306 of Rel-18 UL Tx switching for UL MIMO coherence based on the configuration of Rel-18 UL Tx switching i.e. if configuring Rel-18 UL TX switching, apply uplinkTxSwitching2T2T-PUSCH-TransCoherence-r17. If configuring Rel-18 UL TX switching and uplinkTxSwitching2T2T-PUSCH-TransCoherence-r17 is absent, the Rel-15 per band UE capability pusch-TransCoherence is applicable.