3GPP TSG RAN WG2 Meeting #121               	R2-2301411

# Athens, Greece, February 27th –M arch 3rd 2023

Agenda Item: 	8.2.4

Source:		Sony

Title:			Considerations on Low Power High Accuracy Positioning

Document for:	Discussion & Decision

# Introduction

A new release-18 work item on Expanded and Improved NR positioning was approved at RANP#98e meeting in December 2022 [1]. The result from the study item was captured in TR 38.859 ]2]. t

For Low Power High Accuracy Positioning (LPHAP) the following objectives were agreed

- Specify enhancements for enabling LPHAP use-case 6 as defined in TS 22.104 including: - Extending eDRX cycle beyond 10.24s in RRC_INACTIVE state towards meeting the battery life requirement for LPHAP [RAN2, RAN3, RAN4] - Positioning-specific enhancement for eDRX cycle beyond 10.24s to be defined as part of Rel-18 WI on expanded and improved NR positioning. - NOTE: Work on this objective should be coordinated with that in Rel-18 WI on eRedCap. Towards this, the feature of extending eDRX cycle beyond 10.24s should be defined as part of Rel-18 WI on eRedCap. - NOTE: Inputs from RAN1 as necessary may be facilitated via LSs - For UL and DL+UL positioning for UEs in RRC_INACTIVE state, specify SRS configuration enhancements based on SRS positioning validity area to avoid frequent RRC connection for SRS (re)configuration [RAN2, RAN1, RAN3]. - SRS for positioning configurations in multiple cells [RAN2, RAN1]. - Note: Details including issues such as interference, timing advance, spatial relation information, pathloss reference and common SRS parameters across multiple cells can be further discussed during normative work. - Pre-configuration of one or multiple SRS for positioning configurations [RAN2, RAN3]. - SRS for positioning activation/request procedure(s) [RAN2, RAN1]. - Specify solutions for DL PRS measurements for a UE in RRC_IDLE state and reporting of the measurements in RRC_CONNECTED state [RAN2]. - Specify solutions for alignment between eDRX and PRS configurations [RAN2]. - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

In this contribution, we discuss aspects related to SRS configuration enhancement and techniques to align eDRX and PRS configuration.

# Discussion

## 2.1 SRS configuration enhancements

A UE supporting LPHAP is expected to reduce its power consumption significantly compared to the legacy UE in NR. Hence, the LPHAP target battery life as discussed during study item phase can be met. This, however, might leads to some limitations in the operation of LPHAP UE. In many of the LPHAP use-cases, for example, on the operation of LPHAP UE in a factory / warehouse, there exists a certain number of base-stations. The position of the LPHAP UE needs to be tracked by the system for certain purposes, such as factory automation, tracking, logistic, etc. The LPHAP UE is expected to only have mobility function within the factory / warehouse. Hence, it will interact with those base-stations (gNB) as illustrated in Figure 1. Here, the LPHAP UE supports mobility with a limited number of gNBs. We assume once the LPHAP UE is outside the factory then the LPHAP UE can no longer utilize the low power capability or may not be operated at all.

In this illustrated indoor scenario, the LPHAP UE transmits uplink sounding reference signal (SRS) for positioning purposes to the gNBs. Each gNB performs positioning measurements, based on for instance UL-TDOA, or UL-AoA. The positioning measurements results are transmitted to the LMF. Furthermore, the LMF performs the legacy positioning estimation based on the obtained positioning measurements and the geographical position of the gNBs.

Figure 1: Illustration of LPHAP UE in an indoor factory

During Rel-18 study phase, the target battery life required by LPHAP use case 6 cannot be satisfied when the existing Rel-17 positioning techniques are used for UEs in RRC_INACTIVE state. The enhancements on SRS for positioning has been recommended as one of the solutions to meet the LPHAP target battery life for UL and DL+UL positioning.

As described earlier, the LPHAP is expected to be operated in a certain location (e.g, warehouse, indoor factory) in which there are a few numbers of gNBs. The location or the operation area of a few numbers of gNBs can also be called as validity area. Here, we can consider the operation of LPHAP UE is limited and only interact with those gNBs. Since the LPHAP UE operation is limited and can also be predicted, especially if we consider LPHAP UE operates with certain dedicated function, then the LPHAP UE can be preconfigured with UL SRS configuration. There are limited number of gNBs, so that there could be multiple UL SRS configuration that can be provided to the UE. In the legacy positioning, the UE retrieves the UL SRS configuration when the UE is in CONNECTED mode. When the UE moves to other cell then the UE would require to be in CONNECTED mode to receive the UL SRS configuration of a new cell. It would be beneficial if the UE receives the multiple UL SRS configuration for positioning and the UE can use that when the UE operates in that validity area.

Proposal 1: Support multiple UL SRS configuration supporting several gNBs for positioning of the UE to operate UL and DL+UL positioning in RRC_INACTIVE state.

The configuration can be provided when the UE is in the connected mode via dedicated RRC signalling or it can also be provided using system information.

Proposal 2: Multiple UL SRS configuration can be provided to the UE via dedicated RRC (e.g., when the UE is in connected mode) or system information (SI).

When the LPHAP UE operates in RRC_INACTIVE state, it would be beneficial if the LPHAP UE can be triggered to perform positioning procedure, such as UL positioning by transmitting UL SRS. In the case of the trigger comes from the network (e.g., LMF), the trigger can be included in downlink signal / channel in RRC_INACTIVE state, some wake-up mechanism would be suitable, e.g., using PEI, or paging DCI. For this likely legacy functionality can be used, but some positioning trigger indication would be needed.

Proposal 3: Further discuss wake-up mechanism to trigger UE in idle/inactive mode to support network activation of UL SRS transmission via downlink signal.

Furthermore, the indication can be related to that the MT-SDT procedure is to be used to trigger the MT-LR procedure.

Proposal 4: Introduce new parameter “positioning trigger indication”, in relation to the MT-SDT procedure to initiate/trigger the positioning procedure/measurements.

The SRS should be transmitted in relatively short time after the reception of paging message, such as relative to the start of DRX active time. In this case, after the paging reception, the UE performs RACH procedure in which the timing advance acquiring information can be used to validate the SRS configuration. If the SRS configuration is still valid, the UE continues to perform SRS for positioning.

Proposal 5: Support aligning the DRX on duration and UL SRS transmission for positioning

## 2.2 Alignment between eDRX and PRS configurations

In the RRC_IDLE/RRC_INACTIVE the operation of DRX and positioning procedure are performed independently from each other. This can result in an unnecessary UE power consumption as the UE needs to power on and off its receiver several times to perform both paging related and positioning related operations. To avoid such unnecessary wake-ups, there is therefore a need for positioning or DRX operation enhancements, where the operations of positioning and paging procedure can be aligned.

In the agreed workitem one objective is related to alignment between eDRX and PRS. We consider this valid for any Idle/Inactive DRX scheme.

There can be two types of PRS transmissions, these are periodic and aperiodic (on-demand) PRS transmissions. Below, we describe an enhancement technique for each type.

- In case of aperiodic/event triggered PRS transmission, the PRS transmission (by gNB) is initiated by LMF, but triggered by a defined event such as movement, change of temperature etc. In this case, LMF is not aware of DRX occasion configuration, e.g., occasion time, periodicity, etc. We propose LMF to assign and optionally configure a PRS pattern to relevant gNB(s), and UE, that is aligned with the DRX pattern. In order to make the LMF aware of UE’s DRX pattern, UE reports to LMF, where the report contains the UE-ID, and/or its calculated DRX cycle/Paging Frame, i.e., in which SFN it will wake up to monitor Paging Occasion (PO). This report can be included in a new information Element IE, in the On-demand PRS request message. The LMF will then assign and configure a PRS pattern to relevant gNB and UE that is aligned with the configured DRX pattern

- In case of periodic PRS transmission, we propose the gNB to adjust UE’s DRX occasion based on UE request so that it is aligned with PRS transmission. To facilitate this, we propose the UE calculates the distance in time between PRS occasion and DRX occasion. Then, the UE proposes a DRX offset to be used by the gNB (or AMF). Once, it is granted (by gNB of AMF), the UE is provided with the new (adjusted) DRX,

Proposal 6: In case of aperiodic/event triggered PRS transmission let the LMF align the PRS transmission to the UE DRX cycle/PO,

Proposal 7: In case of periodic PRS transmission let the AMF (or gNB) align the UE DRX/PO to the Periodic PRS transmissions by adding an Offset to the UE-ID for PO calculation, similar to the solution in LTE for MuSIM as specified in 36.304 and 23.502 used for avoiding paging collision.

# Summary

In this contribution, we have discussed our view on potential positioning enhancements in NR. Our observations and proposals are listed below:

Proposal 1: Support multiple UL SRS configuration for positioning for the UE to operate UL and DL+UL positioning in RRC_INACTIVE state.

Proposal 2: Multiple UL SRS configuration can be provided to the UE via dedicated RRC (e.g., when the UE is in connected mode) or system information (SI).

Proposal 3: Further discuss wake-up mechanism to trigger UE in idle/inactive mode to support network activation of UL SRS transmission via downlink signal.

Proposal 4: Introduce new parameter “positioning trigger indication”, in relation to the MT-SDT procedure to initiate/trigger the positioning procedure/measurements.

Proposal 5: Support aligning the DRX on duration and UL SRS transmission for positioning

Proposal 6: In case of aperiodic/event triggered PRS transmission let the LMF align the PRS transmission to the UE DRX cycle/PO,

Proposal 7: In case of periodic PRS transmission let the AMF (or gNB) align the UE DRX/PO to the Periodic PRS transmissions by adding an Offset to the UE-ID for PO calculation, similar to the solution in LTE for MuSIM as specified in 36.304 and 23.502 used for avoiding paging collision.

# References

- RP-223549, New WID on Expanded and Improved NR Positioning, RANP #98e, December 2022

- TR 38.859, “Study on expanded and improved NR position (Release 18)”, December 2022