3GPP TSG-RAN WG2 Meeting #121		R2-2301421

27th February – 3rd March 2023

Source:	Qualcomm Incorporated

Title:	Discussion on SHR for inter-RAT handover and successful PSCell change reporting

Agenda Item:	8.13.4 SHR and SPCR

Document for:	Discussion and Decision

# Introduction & Background

Rel-18, SON/MDT WID [1] has the following as one of the objectives,

Support of SON/MDT enhancements for [RAN3, RAN2]:

- MR-DC CPAC

- Successful PSCell change report

- Successful Handover Report (e.g., inter-RAT)

- NPN

- RACH report

- fast MCG recovery

- NR-U (MRO and UL MLB)

This paper will discuss SON enhancements for the SHR for inter-RAT handover and successful PSCell change reporting.

# Discussion

- SON/MDT enhancements for successful handover report

In the RAN2#120 meeting [2], RAN2 confirmed to only support T310 and T312-related triggered conditions for inter-RAT SHR for mobility from NR to LTE. This avoids any specification impact on the LTE specification. Some details about the configuration and trigger conditions of these timers are still FFS. We believe RAN2 agreements on intra-RAT SHR can be extended to the SHR for inter-RAT mobility from NR to LTE.

Observation 1: In the RAN2#120-emeeting, RAN2 confirmed to only support T310 and T312 trigger conditions for inter-RAT SHR from NR to LTE.

Proposal 1: Similar to the rel-17 intra-NR SHR report, the T310 and T312 threshold is provided to the UE by source-MN in the otherConfig.

Proposal 2: T310 and T312 thresholds are configured as the percentage of the timer value, similar to the threshold configured for intra-NR SHR.

Proposal 3: There is no need for cross-RAT reporting of inter-RAT SHR, i.e., UE reports the SHR report to the network when it comes back to NR.

In the RAN3#110 meeting, RAN3 further agreed that the inter-RAT SHR from LTE to NR can be supported (at least for T304-related triggers) if there are no impacts to LTE.

Proposal 4: To support inter-RAT SHR from LTE to NR without having an LTE specification impact, RAN2 only supports the T304 threshold.

Proposal 5: Similar to intra-NR SHR, the T304 threshold is provided by the target cell.

Proposal 6: NR SHR is generated when the T304 trigger condition meets during inter-RAT mobility from LTE to NR.

- SON/MDT enhancements for SPR

In RAN2#119bis-emeeting [3], RAN2 made the following agreements,

Agreements

1	RAN2 confirms the scenarios for SPR for NR-DC, including:

•	SN- and MN-initiated classic PSCell change / CPC

•	Intra-SN classic PSCell change / CPC

•	Classic Addition / CPA

1a	RAN2 will discuss HO with SN change later, after the basic solution for SPR is known

2	Given that PSCell addition is proposed by all companies, SPR is used as the abbreviations to use for the feature.

3	RAN2 confirm to prioritize NR-DC scenario for SPR.

4	SHR solution is taken as baseline for the SPR in terms of configuration and reporting at high level. Details of the configuration and report need to be tailored/customized/new message per use case.

5	Network configures SPR configuration IE for the UE, with at least the following triggering conditions:

•	T310 triggering condition

•	T312 triggering condition

•	T304 triggering condition

5a: Other triggering conditions are FFS

5b: Values of the triggering conditions are FFS

5c: Which node configures the triggering condition is FFS.

6	RAN2 agree to the following:

A.	SPR configuration is configured by network through otherConfig

B.	SPR is fetched via UE Information Request/Response procedure

7	UE logs at least the following information and measurements in the SPR IE (other information and measurements are FFS).

a)	Source PSCell info (cell ID, measurement result)

b)	Target PScell info (cell ID, measurement result)

c)	Neighbour Cells info (cell ID, measurement result, CPAC Candidate cells flag)

d)	Success PSCell change/addition cause value (e.g., t304, t310, t312 cause, etc.)

f)	The time elapsed between the CPAC execution towards the target cell and the corresponding latest CPAC configuration received for the selected target cell

7a: FFS on whether to reuse CHO candidate cell flag for the CPAC candidate cells or define a new flag to indicate CPAC candidate cell.

7b: FFS on whether to include or on conditional inclusion of random access related information.

7c:	FFS on Location Information

Furthermore, in RAN2#120 meeting [2], RAN2 made the following agreements for SPR,

Agreements:

3	Only MN can retrieve the SPR from the UE.

4	For Q8, RAN2 agree following options: depends on which of nodes initiates SPR, i.e.:

For the MN-initiated PSCell Change/Addition, MN sends the SPR config to the UE

For the SN-initiated PSCell Change, the source-SN sends the Successful PSCell Change configuration within the container through MN.

T304 trigger needs to be configured by the target SN node.

Agreements:

1	UE stores both SPR and SHR configuration (one for each type at most) if received from NW.

2	UE can send the (stored) SPR to gNB. FFS how long UE keeping SPR is FFS.

3	Only the latest successful PSCell change/addition is reported by the UE.

4	Random access related information is included in SPR at least when the SPR is triggered due to T304 exceeds the configured threshold. Other conditions are FFS.

5	UE records/reports PCell SHR and PSCell SPR separately

- On the configuration of SPR for MN-initiated CPC and CPA

In the RAN3#118 meeting [4], RAN3 agreed that source SN determines the T310/T312 triggers (e.g timer threshold) and target SN node determines the T304 triggers (e.g timer threshold), which is in line with RAN2 agreement in RAN2#120 meeting. RAN3 further agreed that for the classic PSCell addition, the SN decides the T304 triggers(e.g timer threshold) and performs root cause analysis. RAN2 further agreed that for MN-initiated PSCell and SN-initiated PSCell change/addition, the T304 trigger condition is determined by the target SN. Note that for CPA, there is no source SN, therefore, T310 and T312 thresholds for CPA cannot be provisioned.

Observation 2: In the RAN3#118 meeting [4], RAN3 agreed that for SN-initiated classic PScell change the source SN node decides the T310/T312 triggers (e.g timer threshold) and the target SN node decides the T304 triggers (e.g timer threshold).

Observation 3: For CPA, there is no source SN, therefore, T310 and T312 thresholds for CPA cannot be provisioned.

Proposal 7: For CPA, the T304 trigger (e.g timer threshold) is configured by the target SN. T310/T312 triggers are not configured for CPA.

In the RAN2#120 meeting [2], RAN2 agreed that for the MN-initiated PSCell Change/Addition, MN sends the SPR config to the UE. However, note that to configure T310/T312 thresholds, MN may need to know the actual values of T310/T312 values. Therefore, there are the following options for deciding T310/T312 thresholds:

- Option 1: MN decides T310/T312 threshold for SPR, and SN provides assistance (i.e., configured T310/T312 values) to MN via Xn signaling for deciding T310/T312 thresholds

- Option 2: Source SN decides T310/T312 threshold for SPR and indicates MN via Xn signaling, then send the threshold value to MN for configuration

- Option 3: Source SN decides T310/T312 threshold for SPR and configures UE via SN RRCReconfiguration

Proposal 8: RAN2 should discuss the following two methods for determining/configuring T310/T312 threshold at the UE,

- MN decides T310/T312 threshold for SPR after SN provides assistance (i.e., configured T310/T312 values) to MN via Xn signaling for deciding T310/T312 thresholds. MN configures the T310/T312 SPR thresholds to UE via MN RRCReconfiguration.

- Source SN decides T310/T312 threshold for SPR and indicates MN via Xn signaling. MN configures the T310/T312 SPR threshold to UE via MN RRCReconfiguration.

- Source SN decides T310/T312 threshold for SPR and configures UE via SN RRCReconfiguration.

- How long UE keeps SPR?

Note that for SHR in Rel-17, a UE variable was introduced to store relevant measurements and information for the successful handover report. UE variable is released if the report is not retrieved by the network within 48 hours. Note that as agreed by RAN3, there can be different scenarios for generating SCPR as mentioned above. For MN-initiated CPC and CPA procedures, the execution conditions, measurement gaps, and others are determined by the MN. In another scenario, e.g., SN-initiated CPC or CPA, the execution conditions, measurement gaps, and others are determined by the source SN. Therefore, for the SPC report, we can have the following two options for clearing the successful PSCell change report,

- Clear after 48 hours if not retrieved by network: UE generates the PSCell change report when configured trigger condition meets. Send the availability indicator in RRCReconfigurationComplete (containing RRCReconfigurationWithSYNC for SCG), and other RRC complete messages. Clear after 48 hours if not retrieved by the network.

- Clear upon PCell change: UE generates the PSCell change report when configured trigger condition meets. Send the availability indicator in RRCReconfigurationComplete (containing RRCReconfigurationWithSYNC for SCG) messages. Clear after UE changes PCell.

Observation 4: There can following two options for clearing the SPCR,

- Clear after 48 hours if not retrieved by network: UE generates the PSCell change report when configured trigger condition meets. Send the availability indicator in RRCReconfigurationComplete (containing RRCReconfigurationWithSYNC for SCG), and other RRC complete messages. Clear after 48 hours if not retrieved by the network.

- Clear upon PCell change: UE generates the PSCell change report when configured trigger condition meets. Send the availability indicator in RRCReconfigurationComplete (containing RRCReconfigurationWithSYNC for SCG) messages. Clear after UE changes PCell.

Note that there can be multiple PSCell change/addition procedures when UE is connected to the same PCell. During these PSCell changes, UE may frequently overwrite the SPCR. Furthermore, note that the SPCR may have relevance for both MN and SN, as MN and SN may want to optimize relevant configurations, therefore storing the SPCR becomes useless. Therefore, UE should clear SPCR upon PCell change.

Observation 5: There can be multiple PSCell change/addition procedures when UE is connected to the same PCell.

Observation 6: As SPR can be frequently overwritten, therefore, there is no need for storing the PSCell report for a longer duration (i.e., for 48 hours) at the UE.

Proposal 9: UE can clear SPCR upon PCell change, i.e., UE generates the PSCell change report when configured trigger condition meets. Send the availability indicator in RRCReconfigurationComplete (containing RRCReconfigurationWithSYNC for SCG) messages. Clear after UE changes PCell.

# Conclusion

Observation 1: In the RAN2#120-emeeting, RAN2 confirmed to only support T310 and T312 trigger conditions for inter-RAT SHR from NR to LTE.

Proposal 1: Similar to the rel-17 intra-NR SHR report, the T310 and T312 threshold is provided to the UE by source-MN in the otherConfig.

Proposal 2: T310 and T312 thresholds are configured as the percentage of the timer value, similar to the threshold configured for intra-NR SHR.

Proposal 3: There is no need for cross-RAT reporting of inter-RAT SHR, i.e., UE reports the SHR report to the network when it comes back to NR.

In the RAN3#110 meeting, RAN3 further agreed that the inter-RAT SHR from LTE to NR can be supported (at least for T304-related triggers) if there are no impacts to LTE.

Proposal 4: To support inter-RAT SHR from LTE to NR without having an LTE specification impact, RAN2 only supports the T304 threshold.

Proposal 5: Similar to intra-NR SHR, the T304 threshold is provided by the target cell.

Proposal 6: NR SHR is generated when the T304 trigger condition meets during inter-RAT mobility from LTE to NR.

Observation 2: In the RAN3#118 meeting [4], RAN3 agreed that for SN-initiated classic PScell change the source SN node decides the T310/T312 triggers (e.g timer threshold) and the target SN node decides the T304 triggers (e.g timer threshold).

Observation 3: For CPA, there is no source SN, therefore, T310 and T312 thresholds for CPA cannot be provisioned.

Proposal 7: For CPA, the T304 trigger (e.g timer threshold) is configured by the target SN. T310/T312 triggers are not configured for CPA.

Proposal 8: RAN2 should discuss the following two methods for determining/configuring T310/T312 threshold at the UE,

- MN decides T310/T312 threshold for SPR after SN provides assistance (i.e., configured T310/T312 values) to MN via Xn signaling for deciding T310/T312 thresholds. MN configures the T310/T312 SPR thresholds to UE via MN RRCReconfiguration.

- Source SN decides T310/T312 threshold for SPR and indicates MN via Xn signaling. MN configures the T310/T312 SPR threshold to UE via MN RRCReconfiguration.

- Source SN decides T310/T312 threshold for SPR and configures UE via SN RRCReconfiguration.

Observation 4: There can following two options for clearing the SPCR,

- Clear after 48 hours if not retrieved by network: UE generates the PSCell change report when configured trigger condition meets. Send the availability indicator in RRCReconfigurationComplete (containing RRCReconfigurationWithSYNC for SCG), and other RRC complete messages. Clear after 48 hours if not retrieved by the network.

- Clear upon PCell change: UE generates the PSCell change report when configured trigger condition meets. Send the availability indicator in RRCReconfigurationComplete (containing RRCReconfigurationWithSYNC for SCG) messages. Clear after UE changes PCell.

Observation 5: There can be multiple PSCell change/addition procedures when UE is connected to the same PCell.

Observation 6: As SPR can be frequently overwritten, therefore, there is no need for storing the PSCell report for a longer duration (i.e., for 48 hours) at the UE.

Proposal 9: UE can clear SPCR upon PCell change, i.e., UE generates the PSCell change report when configured trigger condition meets. Send the availability indicator in RRCReconfigurationComplete (containing RRCReconfigurationWithSYNC for SCG) messages. Clear after UE changes PCell.

# Reference

[1] RP-221825, “Revised WID: Further enhancement of data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity)”

[3] RAN2#120 meeting report

[2] RAN2#119bis-emeeting report

[4] RAN3#118 meeting report